3GPP TSG-RAN4 Meeting #107	R4-2309777

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | - | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for Physical Layer TP requirements - FDD 2RX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_enh3-Perf |  |  |  |  |  | Date: |  |  | 2023-05-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 has agreed to define requirements for Phy layer TP with LA |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added requirements for Phy layer TP with LA for FDD with 2RX |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no requirements for FDD with 2RX for Phy layer TP |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------------------------Start of Change 1 --------------------------------------------------------

## 5.6	PDSCH absolute physical layer throughput requirements

The common parameters for PDSCH absolute physical layer throughput requirements specified in Table 6.1.2-1 are valid for all absolute physical layer throughput requirements unless otherwise stated.

### 5.6.1	1RX requirements

(Void)

### 5.6.2	2RX requirements

#### 5.6.2.1	FDD

##### 5.6.2.1.1	Requirements for physical layer throughput with link adaptation

The performance requirements are specified in Table 5.6.2.1.1-2, with additional test parameters in Table 5.6.2.1.1-1 and the downlink physical channel setup according to Annex C.3.1.

The purpose of the test is to verify PDSCH absolute phycial layer performance with link adaptation.

Table 5.6.2.1.1-1: Test parameters

| Parameter |  |  |  |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz |  |  | 10 |  |  |
| Subcarrier spacing |  |  |  | kHz |  |  | 15 |  |  |
| Duplex Mode |  |  |  |  |  |  | FDD |  |  |
| Propagation channel |  |  |  |  |  |  | TDLA30-5 |  |  |
| Antenna configuration |  |  |  |  |  |  | 2×2 |  |  |
| Correlation configuration |  |  |  |  |  |  | ULA Low |  |  |
| Beamforming Model |  |  |  |  |  | As defined in Annex B.4.1 |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  |  | Periodic |  |  |
|  | Number of CSI-RS ports (X) |  |  |  |  | 4 |  |  |
|  | CDM Type |  |  |  |  | FD-CDM2 |  |  |
|  | Density (ρ) |  |  |  |  | 1 |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  |  | Row 5,(4) |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  |  | (9) |  |  |
|  | CSI-RS periodicity and offset |  |  |  | slot | 5/1 |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  |  | Periodic |  |  |
|  | Number of CSI-RS ports (X) |  |  |  |  | 2 |  |  |
|  | CDM Type |  |  |  |  | FD-CDM2 |  |  |
|  | Density (ρ) |  |  |  |  | 1 |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  |  | Row 3 (6) |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  |  | (13) |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  |  | slot | 5/1 |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  |  |  | Periodic |  |  |
|  | CSI-IM RE pattern |  |  |  |  | Pattern 0 |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  |  | (4,9) |  |  |
|  | CSI-IM timeConfig periodicity and offset |  |  |  | slot | 5/1 |  |  |
| ReportConfigType |  |  |  |  |  | Periodic |  |  |
| CQI-table |  |  |  |  |  | Table 2 |  |  |
| reportQuantity |  |  |  |  |  | cri-RI-PMI-CQI |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  |  | not configured |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  |  | not configured |  |  |
| cqi-FormatIndicator |  |  |  |  |  | Wideband |  |  |
| pmi-FormatIndicator |  |  |  |  |  | Wideband |  |  |
| Sub-band Size |  |  |  |  | RB | 8 |  |  |
| csi-ReportingBand |  |  |  |  |  | 1111111 |  |  |
| CSI-Report periodicity and offset |  |  |  |  | slot | 5/0 |  |  |
| Codebook configuration |  | Codebook Type |  |  |  | typeI-SinglePanel |  |  |
|  |  | Codebook Mode |  |  |  | 1 |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  |  | N/A |  |  |
|  |  | CodebookSubsetRestriction |  |  |  | 111111 |  |  |
|  |  | RI Restriction |  |  |  | N/A |  |  |
| Physical channel for CSI report |  |  |  |  |  | PUCCH |  |  |
| CQI/RI/PMI delay |  |  |  |  | ms | 8 |  |  |
| Maximum number of HARQ transmission |  |  |  |  |  | 4 |  |  |
| Measurement channel |  |  |  | As specified in Table A.4-2 TBS.2-2 |
| Note1: For retransmission number 4 including initial transmission, RV {0,2,3,1} with same MCS and rank as initial transmission; follow the latest UE reported PMI whose rank is same as the initial transmission |  |  |  |  |  |  |  |  |

Table 5.2.2.1.1-2: Minimum performance requirement

| Test num. | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- |
| 1-1 | 35% | [19.3] |
| Note 1: Measurements channels are specified in Table A.4-2. Note 2: The maximum throughout is defined as with TBS corresponding to CQI index 15 with rank 2. |  |  |  |

--------------------------------------End of Change 1 --------------------------------------------------------