3GPP TSG-RAN WG4 Meeting # 107						R4-2309782

# Incheon, KR, May 22 – May 26, 2023

Agenda item:			8.19.3

Source:	Apple

Title:	WF for ATP test

Document for:	Approval

# Introduction

This TDoc summarizes the agreements in RAN4#107 for absolute physical layer throughput requirements with link adaptation.

# Absolute Physical Layer TP requirements

The Phy layer TP requirements have been agreed to be defined as follows:

- Average SNR of impairments results to achieve T% of maximum throughput + X dB margin

  - Use Gspan = [2.5] dB to check if the results are aligned

  - The maximum throughput is defined as with TBS corresponding to CQI index 15 with rank Y for 2Rx/4Rx UE, e.g., Y=2 for both 2Rx/4Rx UEs.

Test point T (%) selection

- For FR1 2x4 case of FDD and TDD mode: 50% agreed as test point.

- For FR1 2x2 case of FDD and TDD mode: 35% agreed as test point.

- For FR2-1 2x2 case of TDD mode: 35% agreed as test point.

Outlier results handling

- Discard outlier results (results farthest from the average) until span is within 2.5 dB.

X dB margin

- 0.8 dB applied for the agreed test points.

Release independence

- Release independent from Rel-17 based on UE declaration and mandatory for Rel-18