3GPP TSG-RAN WG4 Meeting # 107                                                     R4- 2309787			R4-22xxxxx

# Incheon, KR, May 22 – May 26, 2023

Title: 	Impact on RF requirements for duplex evolution

Source: 	CATT

Agenda item:	8.20.2.3

Document for:	Discussion

- Background

In the last RAN4 meeting, some views on the SBFD BS RF requirements were provided. WF [1] summarized the discussion status in last meeting. No agreements were reached, This contribution provides our further analysis based on the WF[1].

- RF Requirement Impact

  - OTA sensitivity within SBFD time slot

In the last meeting’s discussion, companies were generally ok with the following OTA sensitivity requirement formula.

PREFSENSdBm=-174dBm+10×log10(BW)+NF+IM+SNR+[ 1dB desens target]

But there were comments that if the 1dB desens should be the target for the requirement. Currently, 1 dB DESENS is the target for self-interference evaluation. Companies commented that there are also interferences from inter-sector and inter-site. To our understanding, the OTA sensitivity can be defined only for the self-interference scenario. Other scenarios can be discussed separately on how to guarantee the system performance.

Proposal 1: OTA sensitivity should be defined only for the self-interference scenario.

For the exact DESENS value in the formula, it depends on the SI analysis conclusions which are not available yet, especially for WA BS. This can be decided when there’re conclusions or the final values for each BS class can be discuss and decided in WI phase.

Proposal 2: The exact DESENS value can be discuss and decided in WI phase. The REFSENS formula can be written as following,

PREFSENSdBm=-174dBm+10×log10(BW)+NF+IM+SNR+ xdB desens

x value can be different for each BS class pending on the discussion.

There was a proposal that maximum TRP is needed for the OTA sensitivity requirement. We agree that that the TRP should be declared for the OTA sensitivity test, which is the same as the current FDD FR2 BS test. However, it may not need to be defined in the OTA sensitivity requirement. Following current FR2 FDD BS requirement and test approach could be ok.

Proposal 3: For OTA sensitivity test, following current FR2 FDD BS core requirement and test approach could be ok. No TRP or EIRP requirement is defined for the OTA sensitivity requirement.

  - New RF requirements for SBFD

In RAN4#106bis-e meeting, some requirements were proposed. Each proposal is analysed as follows.

- In-channel adjacent subband leakage ratio

In-channel adjacent subband leakage ratio may not be needed if only SI is considered when SBFD OTA sensitivity requirement is defined. However, it also contributes to inter-site CLI and co-site inter-sector CLI, the performance should be guaranteed. So this requirement may be needed.

- In-channel adjacent subband blocking, adjacent subband selectivity

Similar with in-channel adjacent subband leakage, adjacent subband selectivity impacts the co-existence with other BS. So it may need to be defined. For the blocking issue, the test case is the same with adjacent subband selectivity. Therefore, no separate subband blocking requirement is needed.

Proposal 4: In-channel adjacent subband leakage ratio and in-channel adjacent subband selectivity requirements are defined for SBFD capable BS. No separate sub-band blocking requirement is needed.

  - Transmit ON/OFF power requirement

For transition ON-OFF power and transition period requirement, we don’t see much necessity to define these requirements. In our understanding the SBFD and non-SBFD slots are configured statically, so no such ON/OFF power requirement is needed.

Observation1: SBFD and non-SBFD slots are configured statically in the real deployment.

Proposal 5: No transmit ON/OFF power requirement is defined for SBFD

  - Tx intermodulation requirement for co-location scenario

For Tx intermodulation requirement, legacy requirement uses 30 dB coupling loss which is too small for SBFD BS co-site deployment. However, if only the coupling loss assumption is the concern, it seems the adjacent sub-band rejection requirement can solve the problem because the Rx capability is defined then the coupling loss should guarantee that the Rx receiver is not blocked. There may be some other scenarios that the other co-location BS Tx signal is the interference signal to SBFD BS, then the IMD product should be kept within some level. If this is the scenario, the legacy Tx IMD requirement seems more stringent than the SBFD BS co-location scenario.

So we have the following observation.

Observation 2: The legacy Tx intermodulation requirement may be able to guarantee the SBFD Tx IMD performance.

- Summary

Proposal 1: OTA sensitivity should be defined only for the self-interference scenario.

Proposal 2: The exact DESENS value can be discuss and decided in WI phase. The REFSENS formula can be written as following,

PREFSENSdBm=-174dBm+10×log10(BW)+NF+IM+SNR+ xdB desens

x value can be different for each BS class pending on the discussion.

Proposal 3: For OTA sensitivity test, following current FR2 FDD BS core requirement and test approach could be ok. No TRP or EIRP requirement is defined for the OTA sensitivity requirement.

Proposal 4: In-channel adjacent subband leakage ratio and in-channel adjacent subband selectivity requirements are defined for SBFD capable BS. No separate sub-band blocking requirement is needed.

Observation1: SBFD and non-SBFD slots are configured statically in the real deployment.

Proposal 5: No transmit ON/OFF power requirement is defined for SBFD

Observation 2: The legacy Tx intermodulation requirement may be able to guarantee the SBFD Tx IMD performance.

Reference

[1] R4-2305918, “ WF on BS RF requirement impact for SBFD”, RAN4#106bis-e

[2] 3GPP TS 38.104, Base Station (BS) radio transmission and reception