3GPP TSG-RAN4 Meeting #107																	R4-2309794

Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Agenda item:			8.20.4

Source:	Moderator (CMCC)

Title:	Ad-hoc minutes for co-existence study

Document for:	Information

Introduction

Topics to be discussed in adhoc session:

- Confirm the NF modelling

- Check the coexistence cases and scenarios

- Check the template for collecting final simulation results

- Simulation results of case 1 for scenario 1 and 5, strive to reach consensus on the conclusion

- Topics to be discussed in adhoc session

How to model NF modeling into co-existence simulation

Issue 1-1-1: how to evaluate interference components (inter-subband and adjacent) in the NF modelling (Qualcomm, R4-2307314)

- Proposals

  - Option 1: the ACLR should be considered wholly in NF modelling

  - Option 2: the ACLR should be considered partially in NF modelling

  - Option 3: the ACLR should not be considered in NF modelling at all

  - Option 4: TBD

- Agreement:

  - Option 3

Coexistence cases and scenarios for simulation

Table 2-3: Coexistence cases

| Victim | Aggressor | Figures: Aggressor(left) and Victim(right) | Aggressor baseline | Priority |
| --- | --- | --- | --- | --- |
| NR TDD DL | SBFD (DU) | Case 1 | NR TDD DL | High |
| NR TDD UL | SBFD (DU) | Case 2 | NR TDD UL | Low |
| SBFD (DU) | NR TDD DL | Case 3 | No system in adjacent channel | High |
| SBFD(DU) | NR TDD UL | Case 4 |  | Low |

Table 2.1-1: Scenarios for SBFD co-ex study

| FR | Scenario No. | Deployment Scenario1 (Aggressor -> Victim) | Priority |
| --- | --- | --- | --- |
| FR1 (4GHz) | 1 | Urban Macro -> Urban Macro | High |
|  | 2 | Urban Hotspot -> Urban Hotspot | Note 4 |
|  | 3 | Indoor -> Indoor | Low |
|  | 4 | UMa-to-UMi | Note 5 |
|  | 5 | UMi-to-UMi | Note 6 |
| FR2 (30GHz) | 5 | Urban Macro -> Urban Macro | High |
|  | 6 | Urban Hotspot -> Urban Hotspot | Note 4 |
|  | 7 | Urban Micro -> Urban Micro | Low |
|  | 8 | Indoor -> Indoor | Low |
| Note 1: The Urban Macro is agreed as baseline scenario for SBFD co-ex study with high priority in RAN4#104-e, while it does not preclude other scenarios. Note 2: The Urban Hotspot uses the same assumption as Urban Macro, except that Urban Macro uses random dropping method for UE while Urban Hotspot uses cluster-based dropping method for UE. Both random dropping and cluster-based dropping for calibration. Note 3: Consider Urban Macro scenario first for calibration purpose. Note 4: Companies are encouraged to provide simulation results for Urban Hotspot scenario as 2nd priority. [Editor’s Note: Agreement 2.2.1 of R4-2302888] Note 5: Companies also encouraged to simulate Uma-to-UMi co-existence scenario as 2nd priority. [Editor’s Note: Agreement 2.2.3 of R4-2302888] Note 6: Use UMi simulation assumption R4-2305922 as starting point. Consider Tx power refer to 3GPP UMi output power. Detailed simulation assumptions will be discussed after RAN4#107 meeting. |  |  |  |

Template for collecting final simulaiton results

Issue 2-1: template table for collecting final simulation results

Template 1: simulation results for scenario X and case Y (aggressor: SBFD DU, victim: NR TDD UL/DL)

Note: template 1 is only applicable for case 1 and case 2

| Company | Observation point |  | Performance degradation |  |  | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Relative ACIR (dB) (Note 1) | Relative ACIR + step (dB) | Relative ACIR + step (dB) |  |
| Company A | 5% | SINR degradation (dB) |  |  |  |  |
|  |  | Throughput degradation (%) |  |  |  |  |
|  |  | 50% SINR degradation (dB) |  |  |  |  |
|  |  | Mean Throughput degradation (%) |  |  |  |  |
|  |  |  | Relative ACIR (dB) (Note 1) | Relative ACIR + step (dB) | Relative ACIR + step (dB) |  |
| Company B | 5% | SINR degradation (dB) |  |  |  |  |
|  |  | Throughput degradation (%) |  |  |  |  |
|  |  | 50% SINR degradation (dB) |  |  |  |  |
|  |  | Mean Throughput degradation (%) |  |  |  |  |
| Note 1: Relative ACIR is derived from legacy ACLR and ACS of legacy BS and UE. |  |  |  |  |  |  |

Template 2: simulation results for scenario X and case Y (aggressor: NR TDD UL/DL, victim: SBFD DU)

Note: template 2 is only applicable for case 3 and case 4

| Company | Victim | Observation point |  | Performance degradation (SINR in dB, Throughput in %) |  |  | Performance degradation reference (Note 2) | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Relative ACIR (dB) (Note 1) | Relative ACIR + step (dB) | Relative ACIR + step (dB) |  |  |
| Company A | SBFD DL | 5% | SINR degradation (dB) |  |  |  |  |  |
|  |  |  | Throughput degradation (%) |  |  |  |  |  |
|  |  |  | 50% SINR degradation (dB) |  |  |  |  |  |
|  |  |  | Mean Throughput degradation (%) |  |  |  |  |  |
|  | SBFD UL | 5% | SINR degradation (dB) |  |  |  |  |  |
|  |  |  | Throughput degradation (%) |  |  |  |  |  |
|  |  |  | 50% SINR degradation (dB) |  |  |  |  |  |
|  |  |  | Mean Throughput degradation (%) |  |  |  |  |  |
|  |  |  |  | Relative ACIR (dB) (Note 1) | Relative ACIR + step (dB) | Relative ACIR + step (dB) |  |  |
| Company B | SBFD DL | 5% | SINR degradation (dB) |  |  |  |  |  |
|  |  |  | Throughput degradation (%) |  |  |  |  |  |
|  |  |  | 50% SINR degradation (dB) |  |  |  |  |  |
|  |  |  | Mean Throughput degradation (%) |  |  |  |  |  |
|  | SBFD UL | 5% | SINR degradation (dB) |  |  |  |  |  |
|  |  |  | Throughput degradation (%) |  |  |  |  |  |
|  |  |  | 50% SINR degradation (dB) |  |  |  |  |  |
|  |  |  | Mean Throughput degradation (%) |  |  |  |  |  |
| Note 1: Relative ACIR is derived from legacy ACLR and ACS of legacy BS and UE. |  |  |  |  |  |  |  |  |
| Note 2: For legacy TDD interfering SBFD cases, companies are encouraged to report the performance degradation of legacy TDD UL/DL interfering TDD UL/DL with same assumptions. |  |  |  |  |  |  |  |  |

Simulation results for Case 1 (aggressor SBFD DU victim NR TDD DL (high priority)

Scenario 1 FR1 Urban Macro -> Urban Macro (High priority)

Tentative Agreement:

- All the simulation results for 100% grid shift show SINR/throughput degradation is acceptable.

- Observation 1: from Qualcomm in R4-2307314

Table 3 SINR and throughput degradation for UMa FR1 deployment when victim is TDD DL

| Source | Observation Point | Victim: Legacy TDD DL |  |
| --- | --- | --- | --- |
|  |  | Aggressor: SBFD DUD |  |
|  |  | SINR degradation (dB) | Throughput degradation (bps/Hz) |
| Qualcomm Inc. | 5% | 0 | 0 |
|  | 50% | -0.1 | 0.001 |
|  | 95% | -0.1 | -1 |

Observation 1: For FR1 and TDD DL as a victim, no SINR or throughput degradation is observed when adjacent network is SBFD-capable compared to when the adjacent network is a TDD DL network.

It is sufficient for RAN4 to study random UE deployments within the SBFD co-existence framework.

Figure 2 Interference power for different UE deployments

- Observation 2: from Ericsson in R4-2307702

Observation 1: Coexistence of an SBFD network with a DL legacy TDD network is possible in a scenario where users are uniformly distributed. In this case, the UE-to-UE CLI does not cause harmful impact against the DL of the legacy network. However, this scenario may hide coexistence issues, because if users are uniformly distributed over a wide area, the probability that two users active in UL and DL at the same time are dropped close enough to each other to generate UE-to-UE CLI is extremely low.

Table 2.3-1: FR1 simulation results without blocking

| Deployment scenario | Company | Case | Observation Point | Baseline (Mbps) | Throughput (Mbps) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Swept ACIR offset (dB) |  |  |  |  |  |  |  |
|  |  |  |  |  | 0 | 2 | 4 | 6 | 12 | 14 | 20 | 24 |
| 1 | Ericsson | 1 | 5% | 98.3 | 97.3 | 98.1 | 98.2 |  |  |  |  |  |
|  |  |  | 50% | 314 | 314 | 314 | 314 |  |  |  |  |  |
|  |  | 2 | 5% |  |  |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 3 | 5% |  |  |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 4 | 5% |  |  |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  |  |  |  |  |  |  |
| 2 | Ericsson | 1 | 5% | 101.8 | 86.9 | 87 | 87.5 | 90.4 | 94.8 | 95.8 | 97.3 |  |
|  |  |  | 50% | 314 | 310 | 311 | 311 | 312 | 312 | 313 | 314 |  |
|  |  | 2 | 5% |  |  |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 3 | 5% |  |  |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  |  |  |  |  |  |  |
|  |  | 4 | 5% |  |  |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  |  |  |  |  |  |  |

- Observation 3: from Huawei in R4-2307762

Table 1. Updated results for FR1 Uma SBFD (DU) co-ex with TDD from TDD DL perspective

| Aggressor | Victim | Observation Point | SINR degradation (dB) | Throughput degradation (%) |
| --- | --- | --- | --- | --- |
| NR SBFD {DU} 80MHz DL + 20MHz UL | NR TDD 100MHz DL | 5% | 0.08 | 0.1 |
|  |  | 50% | 0.01 | - |
|  |  | 95% | - | - |

- Observation 4: from CMCC in R4-2308194

| Deployment scenario number | Company | Case number | Observation point | Relative ACIR is derived from legacy or baseline assumptions for legacy TDD and SBFD BS and UE. |  |  |  |  |  |  | TDD-TDD with relative ACIR Note1 | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | -30dB | - 20dB | - 10dB | Relative ACIR | +10dB | +20dB | … |  |  |
| FR1 4GHz Uma-Uma | CMCC | 1 SBFDDL>TDDDL | 5% | 93.32 | 67.76 | 28.89 | 4.81 | 0.48 | 0.05 |  |  |  |
|  |  |  | 50% | 15.83 | 1.759 | 0.13 | 0.01 | 0.001 | 0 |  |  |  |
| Note 1: when SBFD as victim, it’s also suggested to report the TDD system throughput loss for the case when TDD interfere TDD using the same parameters as SBFD system. |  |  |  |  |  |  |  |  |  |  |  |  |

- Observation 5: from Nokia in R4-2308629

Observation 1: For coexistence case 1 and Urban Macro scenario, the DL SINR at the victim NR TDD DL remains almost unchanged regardless of whether NR TDD or SBFD is acting as aggressor technology

Table 1. Summary table with NR TDD DL relative throughput performance between aggressor baseline (DL TDD) and aggressor (SBFD) for coexistence case 1

| Scenario | Observation point | - 4dB | - 2dB | Relative ACIR | + 2dB | + 4dB |
| --- | --- | --- | --- | --- | --- | --- |
| Urban macro | 5th percentile | TBD | TBD | 0% | TBD | TBD |
|  | 50th percentile | TBD | TBD | +0.49% | TBD | TBD |

- Observation 6: from CATT in R4-2307391

- Table 11: SBFD adjacent channel co-existence simulation results

| Deployment scenario number | Company | Case number | Observation point | Relative ACIR is derived from legacy or baseline assumptions for legacy TDD and SBFD BS and UE. |  |  |  |  |  |  | TDD-TDD | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | … | - 4dB | - 2dB | Relative ACIR | + 2dB | + 4dB | … | with relative ACIR Note1 |  |
| FR1 Uma-Uma |  | 1 | 5% |  | 2.57 | 2.35 | 2.19 | 0.52 | 0.09 |  |  |  |
|  |  |  | 50% |  | 0.96 | 0.70 | 0.50 | 0.34 | 0.22 |  |  |  |
| Explanations: |  |  |  |  |  |  |  |  |  |  |  |  |
| - The -4/-2/+2/+4 are the offset based on that relative ACIR. - For TDD DL -> SBFD DL case: The relative and offset ACIR is derived from TDD gNB ACLR and SBFD UE ACS; |  |  |  |  |  |  |  |  |  |  |  |  |

- Observation 7: from Samsung in R4-2308785

| Deployment scenario number | Company | Case number | Observation point | Relative ACIR is derived from legacy or baseline assumptions for legacy TDD and SBFD BS and UE. |  |  | TDD-TDD with relative ACIR Note1 | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | / | Relative ACIR | / |  |  |
| FR1 Uma-Uma | Samsung | 1 | 5% | / | -0.25% | / | / | SBFD antenna configuration 1 with power boost. (option 1) |
|  |  |  | 50% | / | 0.24% | / | / |  |
|  |  | 1 | 5% | / | -0.75% | / | / | SBFD antenna configuration 2 |
|  |  |  | 50% | / | -0.52% | / | / |  |
|  |  |  | 50% | / | TDD DL SBFD UL: 0.02% SBFD DL: 1.03% | / | TDD DL TDD DL: 1.12% |  |

Observation 10: No performance degradation of legacy TDD DL was observed by introducing SBFD in adjacent channel for both FR1 and FR2 Urban Macro scenario.

Proposal 8: For SBFD system as aggressor, no performance degradation of legacy TDD DL relating to ACLR/ACS is observed.

- Observation 8: from ZTE in R4-2309176

Observation 2: the interference from FR1 SBFD to NR TDD DL @4GHz seems acceptable by reusing the existing requirement.

|  | 50% | 15.0 | 3.7 | -4.3 | 2.01 | 0.70 | 0.18 | 8.0 | 74% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Scenario 5 FR2 Urban Macro -> Urban Macro (high priority)

Tentative agreement:

- All the simulation results with 100% grid shift based on baseline assumption show SINR/throughput degradation is acceptable.

- Observation 1: from Qualcomm in R4-2307314

Table 5 SINR and throughput degradation for UMa FR2 deployment when victim is TDD DL

| Source | Observation Point | Victim: Legacy TDD DL |  |
| --- | --- | --- | --- |
|  |  | Aggressor: SBFD DUD |  |
|  |  | SINR degradation (dB) | Throughput degradation (bps/Hz) |
| Qualcomm Inc. | 5% | 0.03 | 0 |
|  | 50% | 0.1 | 0.01 |
|  | 95% | 0.1 | 0 |

Observation 4: For FR2, no SINR degradation is observed when the victim network is SBFD DL compared to legacy TDD DL network.

- Observation 2: from Ericsson in R4-2307702

Observation 11: In FR2 and coexistence case 1, similarly to what observed for FR1, it is important to study both urban macro and urban hotspot scenario. The uniform distribution of UEs may hide coexistence issues. Coexistence is possible when users are uniformly distributed, but when the users are clustered it is necessary to increase the ACIR to 26 dB, which is not feasible assuming ACS of legacy UE is 23 dB.

- Observation 3: from CATT in R4-2307391

- Table 11: SBFD adjacent channel co-existence simulation results

| Deployment scenario number | Company | Case number | Observation point | Relative ACIR is derived from legacy or baseline assumptions for legacy TDD and SBFD BS and UE. |  |  |  |  |  |  | TDD-TDD | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | … | - 4dB | - 2dB | Relative ACIR | + 2dB | + 4dB | … | with relative ACIR Note1 |  |
| FR2 Uma-Uma |  | 1 | 5% |  | 19.09 | 5.54 | 1.25 | 0.07 | 0.05 |  |  |  |
|  |  |  | 50% |  | 1.10 | 0.83 | 0.65 | 0.43 | 0.29 |  |  |  |

- Observation 4: from CMCC in R4-2308194

| Deployment scenario number | Company | Case number | Observation point | Relative ACIR is derived from legacy or baseline assumptions for legacy TDD and SBFD BS and UE. |  |  |  |  |  |  | TDD-TDD with relative ACIR Note1 | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | -20dB | - 10dB | Relative ACIR | +10dB | +20dB |  | … |  |  |
| FR2 30GHz Uma-Uma | CMCC | 1 SBFD DL>TDD DL | 5% | 66.812991 | 27.501274 | 4.8093525 | 0.4976720 | 0.0468379 |  |  |  |  |
|  |  |  | 50% | 2.5035963 | 0.1790337 | 0.0143104 | 0.0013455 | 0.0001327 |  |  |  |  |

| Deployment scenario number | Company | Case number | Observation point | Relative ACIR is derived from legacy or baseline assumptions for legacy TDD and SBFD BS and UE. |  |  |  |  |  |  | TDD-TDD with relative ACIR Note1 | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | -20dB | - 10dB | Relative ACIR | +10dB | +20dB |  | … |  |  |
| FR2 30GHz Uma-Uma | CMCC | 1 SBFD UL>TDD DL | 5% | 3.5177880e-05 | 3.5177901e-06 | 3.5177902e-07 | 3.5177908e-08 | 3.5178098e-09 |  |  |  |  |
|  |  |  | 50% | 1.8159290e-07 | 1.8159258e-08 | 1.8159320e-09 | 1.8159940e-10 | 1.8128898e-11 |  |  |  |  |

- Observation 5: from Samsung in R4-2308785

| Deployment scenario number | Company | Case number | Observation point | Relative ACIR is derived from legacy or baseline assumptions for legacy TDD and SBFD BS and UE. |  |  | TDD-TDD with relative ACIR Note1 | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | / | Relative ACIR | / |  |  |
| FR2 Uma-Uma | Samsung | 1 | 5% | / | 2.04% | / | / | SBFD antenna configuration 1 with power boost. (option 1) |
|  |  |  | 50% | / | 0.19% | / | / |  |
|  |  | 1 | 5% | / | 1.81% | / | / | SBFD antenna configuration 2 |
|  |  |  | 50% | / | 0.14% | / | / |  |

Observation 10: No performance degradation of legacy TDD DL was observed by introducing SBFD in adjacent channel for both FR1 and FR2 Urban Macro scenario.

Proposal 8: For SBFD system as aggressor, no performance degradation of legacy TDD DL relating to ACLR/ACS is observed.

- Observation 6: from ZTE in R4-2309176

Observation 4: the interference from FR2 SBFD to NR TDD DL @30GHz seems around 15%