3GPP TSG-RAN WG4 Meeting # 107												R4-2309964

Incheon, Korea, May 22-26, 2023

Agenda item:			8.23.4

Source:	Moderator (Ericsson)

Title:	Topic summary for [107][219] NR_pos_enh2_part1

Document for:	Information

# Introduction

The document contains discussion related to the positioning core requirements for the following 3 main topics:

- Topic # 1: General (AI 8.23.3.1)

- Topic # 2: RedCap positioning (AI 8.23.3.4)

- Topic # 3: PRS/SRS bandwidth aggregation (AI 8.23.3.5)

# Topic #1: General

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2307411 | CATT | Proposal 1: RAN1’s understandings on measurement definitions for positioning with bandwidth aggregation are correct. |
| R4-2307556 | CMCC | Proposal 1: for RAN1 LS on PRS and SRS bandwidth aggregation, RAN1 understanding that the legacy definition of DL RSTD, UL RTOA, UE Rx-Tx time difference, gNB Rx-Tx time difference is reused with the assumption that the subframe timings of the intra-band contiguous carriers are the same are confirmed. |
| R4-2307704 | vivo | Observation 1: No need to consider the collocated carriers for the definition of positioning measurements for the feature of the PRS and SRS bandwidth aggregation Proposal 1: The legacy definition of DL RSTD, UL RTOA, UE Rx-Tx time difference, gNB Rx-Tx time difference can be reused with the assumption that the subframe timings of the intra-band contiguous carriers are the same. |
| R4-2308666 | Huawei, HiSilicon | Proposal: RAN4 to confirm that legacy definition of DL RSTD, UL RTOA, UE Rx-Tx time difference, gNB Rx-Tx time difference can be reused with PRS/SRS CA. |
| R4-2308789 | Ericsson | Proposal 1: RSTD measurement definition is updated by incorporating the following text. “A UE capable of PRS bandwidth aggregation shall perform DL RSTD by aggregating PRS resources across multiple intra-band contiguous PFLs.” Proposal 2: UE Rx-Tx measurement definition is updated by incorporating the following text. “A UE capable of PRS/SRS bandwidth aggregation shall perform UE Rx-Tx time difference measurement by aggregating PRS resources across multiple DL intra-band contiguous PFLs and SRS in multiple UL intra-band contiguous PFLs. The PRS resource aggregation and SRS resource aggregation shall be performed on the same number of PFLs.” Proposal 3: UL-RTOA measurement definition is updated by incorporating the following text. “A base station capable of SRS resource aggregation shall perform TUL-RTOA measurement by aggregating SRS resources across multiple intra-band contiguous carriers.” Proposal 4: gNB Rx-Tx measurement definition is updated by incorporating the following text. “A base station capable of PRS/SRS bandwidth aggregation shall perform gNB Rx-Tx time difference measurement by aggregating SRS resources across multiple UL intra-band contiguous PFLs and PRS in multiple DL intra-band contiguous PFLs. The PRS resource aggregation and SRS resource aggregation shall be performed on the same number of PFLs.” |
| R4-2308468 | OPPO | Proposal 1: Confirm RAN1’s understanding that the legacy definition of DL RSTD, UL RTOA, UE Rx-Tx time difference, gNB Rx-Tx time difference is reused. |
| R4-2309677 | Nokia, Nokia Shanghai Bell | - RAN4 to propose to RAN1 to extend the measurement definitions in TS 38.215 with the following text: For DL RSTD measurement definition in subclause 5.1.29: If PRS bandwidth aggregation is configured, the measurement is performed over configured number of intra-band contiguous PFLs , with the assumption that the subframe timings of the intra-band contiguous PFLs are the same. - Note: multiple PRS resources from the same TRP which can be used to determine the start of subframe can be from multiple intra-band contiguous carriers.. For UE Rx-Tx time difference measurement definition in subclause 5.1.30: If PRS bandwidth aggregation and/or SRS bandwidth aggregation is configured, the measurement is performed over configured number of intra-band contiguous PFLs , with the assumption that the subframe timings of the intra-band contiguous PFLs are the same. - Note: multiple PRS resources from the same TRP and/or multiple SRS resources from the target UE which can be used to determine the start of subframe can be from multiple intra-band contiguous carriers.. For RTOA measurement definition in subclause 5.2.2: If SRS bandwidth aggregation is configured, the measurement is performed over configured number of intra-band contiguous PFLs , with the assumption that the subframe timings of the intra-band contiguous PFLs are the same. - Note: multiple SRS resources from the target UE which can be used to determine the start of subframe can be from multiple intra-band contiguous carriers.. For gNB Rx-Tx time difference measurement definition in subclause 5.2.3: If PRS bandwidth aggregation and/or SRS bandwidth aggregation is configured, the measurement is performed over configured number of intra-band contiguous PFLs, where the subframe timings of the intra-band contiguous PFLs are the same. - Note: multiple PRS resources from the measuring TRP and/or SRS resources from the target UE which can be used to determine the start of subframe can be from multiple intra-band contiguous carriers.. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1: RAN1 LS on bandwidth aggregation definition

RAN4 has received RAN1 LS in R1-2304082/R4-2307005 to confirm whether:

The legacy definition of DL RSTD, UL RTOA, UE Rx-Tx time difference, gNB Rx-Tx time difference is reused with the assumption that the subframe timings of the intra-band contiguous carriers are the same.

•	Note: multiple PRS/SRS resources which can be used to determine the start of subframe can be from multiple intra-band continuous carriers,

•	Note: no RAN1 spec impact

•	Send an LS to RAN4 to confirm RAN1’s understanding

Issue 1-1-1: RAN4 reply on definitions for positioning with bandwidth aggregation.

- Proposals

  - Option 1: CATT, CMCC, vivo, HW, OPPO

    - The legacy definition of DL RSTD, UL RTOA, UE Rx-Tx time difference, gNB Rx-Tx time difference can be reused with the assumption that the subframe timings of the intra-band contiguous carriers are the same.

  - Option 2: E///

    - RSTD measurement definition is updated by incorporating the following text.

      - “A UE capable of PRS bandwidth aggregation shall perform DL RSTD by aggregating PRS resources across multiple intra-band contiguous PFLs.”

    - UE Rx-Tx measurement definition is updated by incorporating the following text.

      - “A UE capable of PRS/SRS bandwidth aggregation shall perform UE Rx-Tx time difference measurement by aggregating PRS resources across multiple DL intra-band contiguous PFLs and SRS in multiple UL intra-band contiguous PFLs. The PRS resource aggregation and SRS resource aggregation shall be performed on the same number of PFLs.”

    - UL-RTOA measurement definition is updated by incorporating the following text.

      - “A base station capable of SRS resource aggregation shall perform TUL-RTOA measurement by aggregating SRS resources across multiple intra-band contiguous carriers.”

    - gNB Rx-Tx measurement definition is updated by incorporating the following text.

      - “A base station capable of PRS/SRS bandwidth aggregation shall perform gNB Rx-Tx time difference measurement by aggregating SRS resources across multiple UL intra-band contiguous PFLs and PRS in multiple DL intra-band contiguous PFLs. The PRS resource aggregation and SRS resource aggregation shall be performed on the same number of PFLs.”

  - Option 3: Nokia

For DL RSTD measurement definition in subclause 5.1.29:

If PRS bandwidth aggregation is configured, the measurement is performed over configured number of intra-band contiguous PFLs , with the assumption that the subframe timings of the intra-band contiguous PFLs are the same. - Note: multiple PRS resources from the same TRP which can be used to determine the start of subframe can be from multiple intra-band contiguous carriers.

For UE Rx-Tx time difference measurement definition in subclause 5.1.30:

If PRS bandwidth aggregation and/or SRS bandwidth aggregation is configured, the measurement is performed over configured number of intra-band contiguous PFLs , with the assumption that the subframe timings of the intra-band contiguous PFLs are the same. - Note: multiple PRS resources from the same TRP and/or multiple SRS resources from the target UE which can be used to determine the start of subframe can be from multiple intra-band contiguous carriers.

For RTOA measurement definition in subclause 5.2.2:

If SRS bandwidth aggregation is configured, the measurement is performed over configured number of intra-band contiguous PFLs , with the assumption that the subframe timings of the intra-band contiguous PFLs are the same. - Note: multiple SRS resources from the target UE which can be used to determine the start of subframe can be from multiple intra-band contiguous carriers.

For gNB Rx-Tx time difference measurement definition in subclause 5.2.3:

If PRS bandwidth aggregation and/or SRS bandwidth aggregation is configured, the measurement is performed over configured number of intra-band contiguous PFLs, where the subframe timings of the intra-band contiguous PFLs are the same. - Note: multiple PRS resources from the measuring TRP and/or SRS resources from the target UE which can be used to determine the start of subframe can be from multiple intra-band contiguous carriers.

- Recommended WF

  - Discuss the options and prepare a draft LS response, based on the agreement.

# Topic #2: RedCap positioning

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |

| R4-2307414 | CATT | Proposal 1: Define requirements for RedCap UE positioning with frequency hopping in RRC_CONNECTED and RRC_INACTIVE. Proposal 2: The requirements for redcap UE positioning in RRC_IDLE can be discussed in LPHAP part. Proposal 3: PRS requirements for both 1Rx and 2Rx RedCap UE without FH shall be defined for all the Rel-16/Rel-17 positioning features/techniques. Proposal 4: For 1Rx redcap UE without FH, reuse approximately Rel-17 accuracy requirements and relax the side condition for fading channel. Proposal 5: Define measurement requirements for the following cases separately: - Case 1: A single measurement is reported based on receiving multiple hops of the DL PRS or UL SRS for positioning - Case 2: One [or more] measurements are reported where each measurement is associated with one received hop Proposal 6: The existing Rel-17 measurement period and accuracy requirements can be used as baseline for each hop in case 2. Proposal 7: The measurement period requirements for case 1 can take Rel-17 requirements as baseline with the update on the definitions of Tavailable and Lavailable. |
| --- | --- | --- |
| R4-2307415 | CATT | Observation 1: For RSTD with 4 samples, reusing Rel-17 side condition cannot meet the Rel-17 measurement accuracy. And the better measurement performance is achieved with side condition <-3,-6,-6> which can be comparable to Rel-17 accuracy. Observation 2: For PRS RSRP with 4 samples, reusing Rel-17 side condition cannot meet the Rel-17 accuracy. It is feasible to approximately achieve Rel-17 measurement accuracy by using <-3,-10,-10>. Observation 3: For UE Rx-Tx with 4 samples, reusing Rel-17 side conditions cannot meet the Rel-17 measurement accuracy. Using <-3,-6,-6> will approximately meet the Rel-17 measurement accuracy. Observation 4: For RSTD and UE Rx-Tx with SCS 60 kHz, the measurement performance for cell 3 which has large time shift is not optimistic and needs more analysis. |
| R4-2307559 | CMCC | Proposal 1: PRS requirements for both 1Rx and 2Rx RedCap UE without FH shall be defined for all the Rel-16/Rel-17 positioning features/techniques. Proposal 2: PRS measurement requirements without FH in inacitve state, Kcarrier_PRS is updated as following: - If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier_PRS = Kcarrier_RedCap + 1Kcarrier_PRS=Kcarrier+1, where Kcarrier_RedCap Kcarrier is defined in clause 4.2B.2.4. - If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier_PRS =Nlayers+1, where N_layers is defined in clause 4.2.2.7 |
| R4-2308467 | OPPO | Proposal 1: The Rel-16 positioning features should be considered for all types of RedCap UE, include 1 Rx and 2 Rx with FH and without FH. Proposal 2: For applicable RRC states: - For RedCap UE without FH, define RRM requirements in RRC idle state. - For RedCap UE with FH, prioritize RRM requirements in RRC connected state. Proposal 3: MG-less based measurement: - For RedCap UE without FH, support MG-less based PRS measurement - For RedCap UE with FH, whether to support MG-less based measurement is up to RAN1 Proposal 4: For RedCap UE without FH and with FH, support reduced samples. Proposal 5: For lower Rx beam sweeping factor in FR2: - For 2 Rx RedCap UE without FH, support lower Rx beam sweeping factor. - For 1 Rx RedCap UE without FH, lower Rx beam sweeping factor is not applicable. - For RedCap UE with FH, not support lower Rx beam sweeping factor. Proposal 6: For RedCap UE without FH and with FH, not support TEG feature. Proposal 7: For RedCap UE without FH and with FH, support per-FR gap and MAC-CE based Pos gap. |
| R4-2308669 | Huawei, HiSilicon | Proposal 1: For RedCap UE positioning with FH, RAN4 to decide whether to define requirements for INACTIVE/IDLE after requirements for CONNECTED are stable. Proposal 2: PRS requirements for both 1Rx and 2Rx RedCap UE without FH shall be defined for all the Rel-16/Rel-17 positioning features/techniques. Proposal 3: For 1Rx RedCap UE without FH - For AWGN, re-use the Rel-17 side conditions also for reduced sample number - For fading channel, the lower Es/Iot side condition is defined as [-10]dB Proposal 4: RAN4 to define requirements for PRS measurement with FH for the case where UE hops between different repetitions of same resource within a single MG occasion. Proposal 5: RAN4 to discuss how to determine the effective BW with FH in a single MG occasion and the applicable accuracy requirements based on following parameters - Number of repetitions, which depends on PRS resource configuration - Switching time, which depends on UE capability - Number of overlapping RBs between hops, which is FFS Proposal 6: RAN4 to wait for further RAN1 conclusion e.g. on details on RedCap UE processing capabilities, before defining the exact measurement period for PRS measurement with FH. |
| R4-2308670 | Huawei, HiSilicon | Observation 1: For RSTD and UE Rx-Tx, with Es/Iot of -13dB and TDL-A channel, the first path detection performance is poor (detection rate <90%) with 1RX. Observation 2: For RSTD and UE Rx-Tx, for other cases than Es/Iot of -13dB and TDL-A channel, the TOA estimation error is larger with 1RX. Observation 3: For RSTD and UE Rx-Tx, with Es/Iot of -10dB and TDL-A channel, reasonable accuracy performance can be achieved. |
| R4-2308791 | Ericsson | Observation 1: Under AWGN propagation condition, accuracies of 4 sample RSTD and Rx-Tx measurements performed by 1Rx and 2Rx UEs under Rel. 17 side conditions are comparable to each other. For 1Rx UEs accuracy requirement can be relaxed by 1Tc. Observation 2: Under AWGN propagation condition, Rel. 17 4 sample RSTD and Rx-Tx measurement accuracy requirements can be met by 1Rx RedCap UE at higher side condition Es/Iot = (-3, -6, -6)dB. Observation 3: Under fading propagation conditions: - Rel. 17 4 sample accuracy (RSTD and UE Rx-Tx) cannot be met by 1Rx RedCap UE under Rel. 17 side condition. - Performance degradation is more pronounced for smaller number of PRBs. - Accuracies of 4 sample RSTD and Rx-Tx measurements performed by 1Rx and 2Rx UEs under Rel. 17 side conditions are comparable to each other for larger BWs. - Relaxed accuracy requirement can be defined only for BWs more than 104 PRBs for 15kHz SCS. - Rel. 17 4 sample accuracy (RSTD and UE Rx-Tx) cannot be met by 1Rx RedCap UE under higher side condition, i.e., Es/Iot = (-3, -6, -6)dB and Es/Iot = (0, -6, -6)dB. - Performance degradation is more pronounced for smaller number of PRBs. - Accuracies of 4 sample RSTD and Rx-Tx measurements performed by 1Rx under higher side condition and 2Rx UEs under Rel. 17 side conditions are comparable to each other for larger BWs. - Relaxed accuracy requirement can be defined only for BWs more than 104 PRBs for 15kHz SCS. Observation 4: PRS measurement requirement will be defined for RRC_CONNECTED and RRC_INACTIVE states for RedCap UE positioning without frequency hopping. Proposal 1: PRS requirements for both 1Rx and 2Rx RedCap UEs without FH shall be defined for all the Rel-16/Rel-17 positioning features/techniques. For 1Rx RedCap UE reduced Rx beam sweeping factor for positioning measurement is out of scope. Proposal 2: For AWGN channel condition, consider the following options to define accuracy requirements for 1Rx RedCap UE. Our preference is option 2 below. - Option 1: Reuse rel. 17 4 sample RSTD and UE Rx-Tx accuracy requirement defined for AWGN channel model to define accuracy requirement for 4 sample RSTD and UE Rx-Tx accuracy for 1Rx RedCap UE for AWGN channel model under higher side condition, i.e., Es/Iot = (-3, -6, -6)dB, for without FH case. - Option 2: Accuracy requirement for 4 sample RSTD and UE Rx-Tx measurements are defined by reusing Rel. 17 side conditions for without FH case. The 4 sample RSTD and UE Rx-Tx accuracy requirement can be relaxed by 1Tc in comparison to Rel. 17 4 sample RSTD and UE Rx-Tx accuracy requirement. Proposal 3: For fading channel conditions, consider the following options to define accuracy requirements for 1Rx RedCap UE. Our preference is option 2 below. - Option 1: For fading propagation condition, define 4 sample accuracy requirements for 15kHz (104 PRBs), 30kHz (24/48 PRBs), and 60kHz (24 PRBs) under higher side condition, i.e., Es/Iot = (-3, -6, -6)dB, for without FH case. The accuracy requirement can be relaxed in comparison to Rel. 17 accuracy requirement. - Option 2: Do not define accuracy requirement for 1Rx RedCap UE applicable to fading propagation conditions. Proposal 4: For positioning measurement core requirement for RedCap UEs without FH in RRC_INACTIVE state, Kcarrier_PRS in Rel. 17 RRC_INACTIVE core requirement is updated to Kcarrier_PRS_RedCap. Kcarrier_PRS_RedCap = 1 if RedCap UE is able to perform RRM and PRS measurements in parallel to each other, otherwise Kcarrier_PRS_RedCap is defined as: - If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier_PRS_RedCap=Kcarrier_RedCap+1, where Kcarrier_RedCap is defined in clause 4.2B.2.4. - If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier_PRS_RedCap=Nlayers+1, where Nlayers is defined in clause 4.2.2.7. Proposal 5: For positioning measurement core requirement for RedCap UEs without FH in RRC_CONNECTED state, CSSFPRS,i in Rel. 17 RRC_CONNECTED core requirement is updated to CSSFPRS,RedCap,i. The value of CSSFPRS,RedCap,i is the carrier-specific scaling factor for NR PRS-based positioning measurements in positioning frequency layer i as defined in clause 9.1A.5.2. Proposal 6: Rel. 17 TEG framework is supported for RedCap positioning. Proposal 7: RAN4 to uphold agreements from the last meeting and wait for further progress to be made in RAN1 and RF group discussion on RF switching time between hops. |
| R4-2308792 | Ericsson | Simulation results for 1Rx RedCap |
| R4-2308793 | Ericsson | Summary of 1Rx RedCap simulations |
| R4-2309132 | Qualcomm Incorporated | Simulation results for 1Rx RedCap UEs without frequency hopping – reduced number of samples |
| R4-2309133 | Qualcomm Incorporated | Observation 1: RAN4 agreed to define requirements for RedCap positioning in RRC_IDLE state. Proposal 1: - Define requirements for RedCap positioning without frequency hopping in RRC_IDLE state - FFS define requirements for RedCap positioning with frequency hopping in RRC_IDLE state Proposal 2: PRS requirements for both 1Rx and 2Rx RedCap UE without FH shall be defined for all the Rel-16/Rel-17 positioning features/techniques. Proposal 3: PRS requirements for both 1Rx and 2Rx RedCap UE with FH will be defined for all the Rel-16 positioning features. FFS which Rel-17 positioning features will be covered by the requirements with FH. Proposal 4: RAN4 to wait for more progress in RAN1 before starting to work on detailed PRS measurement period requirements with frequency hopping for RedCap UEs. Proposal 5: For defining DL PRS measurement requirements with FH within measurement gaps, prioritize the case where the multiple hops are all received within a single gap instance. Proposal 6: Reuse the Rel-17 side-conditions to define measurement accuracy requirements for 1Rx without FH in AWGN with reduced number of samples. Proposal 7: RAN4 to evaluate using a higher side-condition to define measurement accuracy requirements for 1Rx without FH in fading conditions. |
| R4-2309558 | MediaTek inc. | Proposal 1: PRS requirements for both 1Rx and 2Rx RedCap UE without FH shall be defined for all the Rel-16/Rel-17 positioning features/techniques. (Note: all the enhancement related to FR2 is not applicable to 1Rx). Proposal 2: The side conditions for fading channel for 1Rx RedCap UE without FH: relax Rel-17 side conditions and the accuracy requirements. |
| R4-2309603 | Nokia, Nokia Shanghai Bell | Proposal 1: RAN4 to define the core requirements for positioning of RedCap UE with frequency hopping in INACTIVE state. Proposal 2: We support Option 1 to define PRS requirements for both 1Rx and 2Rx RedCap UE without FH shall be defined for all the Rel-16/Rel-17 positioning features/techniques. Proposal 3: We support Option 1 to reuse Rel-17 side conditions and relax accuracy requirements. Proposal 4: RAN4 will start a discussion after RAN1 design of PRS/SRS configurations with FH is finalized. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1: General aspects/scenarios for RedCap positioning

Sub-topic description:

Open issues and candidate options before meeting:

In RAN4#106bis-e, following was agreed in the WF in R4-2306349:

- RedCap UE positioning without frequency hopping

  - Define the requirements in RRC_CONNECTED and RRC_INACTIVE states.

- RedCap UE positioning with frequency hopping

  - Define the requirements in RRC_CONNECTED state

  - FFS for requirements in RRC_INACTIVE state.

- FFS whether to define requirements in RRC_IDLE state

Issue 2-1-1: Applicability of RRC_IDLE state for Redcap positioning without FH:

- Proposals

  - Option 1: CATT

    - The requirements for redcap UE positioning in RRC_IDLE can be discussed in LPHAP part.

  - Option 2: OPPO, QC

    - For RedCap UE without FH, define RRM requirements in RRC idle state.

- Recommended WF

  - Discuss the options.

Issue 2-1-2: Applicable RRC states for Redcap positioning with FH:

- Proposals

  - Option 1: CATT, Nokia

    - Define requirements for RedCap UE positioning with frequency hopping in RRC_CONNECTED and RRC_INACTIVE.

    - Option 1A: CATT

      - The requirements for redcap UE positioning in RRC_IDLE can be discussed in LPHAP part.

  - Option 2: OPPO

    - For RedCap UE with FH, prioritize RRM requirements in RRC connected state.

  - Option 3: HW

    - For RedCap UE positioning with FH, RAN4 to decide whether to define requirements for INACTIVE/IDLE after requirements for CONNECTED are stable.

  - Option 4: QC

    - FFS define requirements for RedCap positioning with frequency hopping in RRC_IDLE state

- Recommended WF

  - Discuss the options.

Issue 2-1-3: Relation with Rel-16/Rel-17 positioning for PRS measurements without FH

- Proposals

  - Option 1: CATT, CMCC, HW, E///, QC, Nokia, MTK

    - PRS requirements for both 1Rx and 2Rx RedCap UE without FH shall be defined for all the Rel-16/Rel-17 positioning features/techniques.

    - Option 1A: E///

      - For 1Rx RedCap UE reduced Rx beam sweeping factor for positioning measurement is out of scope.

  - Option 2: OPPO

    - MG-less based measurement:

      - For RedCap UE without FH, support MG-less based PRS measurement

    - For RedCap UE without FH and with FH, support reduced samples.

    - For lower Rx beam sweeping factor in FR2:

      - For 2 Rx RedCap UE without FH, support lower Rx beam sweeping factor.

      - For 1 Rx RedCap UE without FH, lower Rx beam sweeping factor is not applicable.

    - For RedCap UE without FH and with FH, support per-FR gap and MAC-CE based Pos gap.

- Recommended WF

  - Discuss the options

Issue 2-1-4: Relation with Rel-16/Rel-17 positioning for PRS measurements with FH

- Proposals

  - Option 1: QC

    - PRS requirements for both 1Rx and 2Rx RedCap UE with FH will be defined for all the Rel-16 positioning features. FFS which Rel-17 positioning features will be covered by the requirements with FH.

  - Option 2: OPPO

    - MG-less based measurement:

      - For RedCap UE with FH, whether to support MG-less based measurement is up to RAN1

    - For RedCap UE without FH and with FH, support reduced samples.

    - For lower Rx beam sweeping factor in FR2:

      - For RedCap UE with FH, not support lower Rx beam sweeping factor.

    - For RedCap UE without FH and with FH, support per-FR gap and MAC-CE based Pos gap.

- Recommended WF

  - Discuss the options.

Issue 2-1-5: TEG framework for RedCap positioning

  - Option 1: OPPO

  - For RedCap UE without FH and with FH, not support TEG feature.

- Option 2: E///

  - Rel. 17 TEG framework is supported for RedCap positioning.

- Recommended WF

  - Discuss the options.

### Sub-topic 2-2: PRS measurements for RedCap without FH

Sub-topic description

Open issues and candidate options before meeting:

In RAN4#106bis-e, following was agreed in the WF in R4-2306349:

- Side conditions for 1Rx without FH

  - For AWGN channel, re-use the Rel-17 side conditions, and relax accuracy requirements

    - The agreement applies for 4 measurement samples case. FFS if it applies for lower number of samples.

  - FFS for fading channel

    - Option 1: Reuse Rel-17 side conditions and relax accuracy requirements

    - Option 2: Reuse approximately Rel-17 accuracy requirements and relax the side condition

Issue 2-2-1: Side conditions for 1Rx without FH

- Proposals

  - Option 1: CATT

    - For 1Rx redcap UE without FH, reuse approximately Rel-17 accuracy requirements and relax the side condition for fading channel.

  - Option 2: HW

    - For 1Rx RedCap UE without FH

      - For AWGN, re-use the Rel-17 side conditions also for reduced sample number

      - For fading channel, the lower Es/Iot side condition is defined as [-10]dB

  - Option 3: E///

    - For AWGN channel condition, consider the following options to define accuracy requirements for 1Rx RedCap UE. Our preference is option 2 below.

      - Reuse rel. 17 4 sample RSTD and UE Rx-Tx accuracy requirement defined for AWGN channel model to define accuracy requirement for 4 sample RSTD and UE Rx-Tx accuracy for 1Rx RedCap UE for AWGN channel model under higher side condition, i.e., Es/Iot = (-3, -6, -6)dB, for without FH case.

      - Accuracy requirement for 4 sample RSTD and UE Rx-Tx measurements are defined by reusing Rel. 17 side conditions for without FH case. The 4 sample RSTD and UE Rx-Tx accuracy requirement can be relaxed by 1Tc in comparison to Rel. 17 4 sample RSTD and UE Rx-Tx accuracy requirement.

    - For fading channel conditions, consider the following options to define accuracy requirements for 1Rx RedCap UE. Our preference is option 2 below.

      - For fading propagation condition, define 4 sample accuracy requirements for 15kHz (104 PRBs), 30kHz (24/48 PRBs), and 60kHz (24 PRBs) under higher side condition, i.e., Es/Iot = (-3, -6, -6)dB, for without FH case. The accuracy requirement can be relaxed in comparison to Rel. 17 accuracy requirement.

      - Do not define accuracy requirement for 1Rx RedCap UE applicable to fading propagation conditions.

  - Option 4: QC:

    - Reuse the Rel-17 side-conditions to define measurement accuracy requirements for 1Rx without FH in AWGN with reduced number of samples.

    - RAN4 to evaluate using a higher side-condition to define measurement accuracy requirements for 1Rx without FH in fading conditions.

  - Option 5: MTK

    - The side conditions for fading channel for 1Rx RedCap UE without FH: relax Rel-17 side conditions and the accuracy requirements.

  - Option 6: Nokia

    - We support Option 1 to reuse Rel-17 side conditions and relax accuracy requirements.

- Recommended WF

  - Discuss the options.

Issue 2-2-2: PRS measurement requirements in RRC inactive state without FH

- Proposals

  - Option 1: E///, CMCC

    - For positioning measurement core requirement for RedCap UEs without FH in RRC_INACTIVE state, Kcarrier_PRS in Rel. 17 RRC_INACTIVE core requirement is updated to Kcarrier_PRS_RedCap. Kcarrier_PRS_RedCap = 1 if RedCap UE is able to perform RRM and PRS measurements in parallel to each other, otherwise Kcarrier_PRS_RedCap is defined as:

      - If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier_PRS_RedCap=Kcarrier_RedCap+1, where Kcarrier_RedCap is defined in clause 4.2B.2.4.

      - If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier_PRS_RedCap=Nlayers+1, where Nlayers is defined in clause 4.2.2.7.

- Recommended WF

  - Check if Option 1 is agreeable?

Issue 2-2-3: PRS measurement requirements in RRC connected state without FH

- Proposals

  - Option 1: E///

  - For positioning measurement core requirement for RedCap UEs without FH in RRC_CONNECTED state, CSSFPRS,i in Rel. 17 RRC_CONNECTED core requirement is updated to CSSFPRS,RedCap,i. The value of CSSFPRS,RedCap,i is the carrier-specific scaling factor for NR PRS-based positioning measurements in positioning frequency layer i as defined in clause 9.1A.5.2.

- Recommended WF

  - Discuss the option.

Issue 2-2-4: Collection of simulation results for 1Rx without FH

- Proposals

  - Moderator suggests that in first round all the simulation results are collected and compiled e.g. in R4-2308973.

- Recommended WF

  - Discuss based on the compiled results.

### Sub-topic 2-3: PRS measurements for RedCap with FH

Sub-topic description

Open issues and candidate options before meeting:

Issue 2-3-1: When to start PRS measurements for RedCap with FH?

- Proposals

  - Option 1: HW, E///, QC, Nokia

    - RAN4 to wait for more progress in RAN1 before starting to work on detailed PRS measurement period requirements with frequency hopping for RedCap UEs.

- Recommended WF

  - Discuss the option.

Issue 2-3-2: Scenarios for PRS measurements for RedCap with FH

- Proposals

  - Option 1: CATT

    - Define measurement requirements for the following cases separately:

      - Case 1: A single measurement is reported based on receiving multiple hops of the DL PRS or UL SRS for positioning

      - Case 2: One [or more] measurements are reported where each measurement is associated with one received hop

- Recommended WF

  - Discuss the option.

Issue 2-3-3: Relation between FH and measurement gaps for PRS measurements for RedCap with FH

- Proposals

  - Option 1: HW

    - RAN4 to define requirements for PRS measurement with FH for the case where UE hops between different repetitions of same resource within a single MG occasion.

  - Option 2: QC

    - For defining DL PRS measurement requirements with FH within measurement gaps, prioritize the case where the multiple hops are all received within a single gap instance.

- Recommended WF

  - Discuss the options.

Issue 2-3-4: Effective BW in gap for PRS measurements for RedCap with FH

- Proposals

  - Option 1: HW

    - RAN4 to discuss how to determine the effective BW with FH in a single MG occasion and the applicable accuracy requirements based on following parameters

      - Number of repetitions, which depends on PRS resource configuration

      - Switching time, which depends on UE capability

      - Number of overlapping RBs between hops, which is FFS

- Recommended WF

  - Discuss the option.

Issue 2-3-5: PRS measurement period requirements for RedCap with FH

- Proposals

  - Option 1: CATT

    - The existing Rel-17 measurement period and accuracy requirements can be used as baseline for each hop in case 2.

    - The measurement period requirements for case 1 can take Rel-17 requirements as baseline with the update on the definitions of Tavailable and Lavailable.

- Recommended WF

  - Discuss the option.

# Topic #3: PRS/SRS BW aggregation

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |

| R4-2307430 | LG Electronics Inc. | Proposal 1: RAN4 to consider the PFL group concept which means aggregated PFLs. For example, in the RSTD case TRSTD,Total=j=1GPRSTD,j+G-1×maxPeffect, j where, j: Index of PFL group G: Total number of PFL groups Peffect,j: Periodicity of the PRS RSTD measurement in PFL group j PRSTD,j: Measurement period for PRS RSTD measurement in PFL group j Proposal 2: RAN4 to wait for more RAN1 progress to discuss the guard period. Proposal 3: Existing report mapping can be reused for RSTD and UE Rx-Tx. If necessary RAN4 can make a decision after performance evaluation. |
| --- | --- | --- |
| R4-2308043 | ZTE Corporation | Observation 1: RAN1 had several agreements for enabling PRS bandwidth aggregation between PRS in two or three different PFLs, and in the latest two meetings the conditions which should be satisfied for the aggregated PRS resources from a TRP across the aggregated PFLs were proposed. Proposal 1: RAN4 shall study the core requirements based on the current RAN1 progress as below: - In the same slot, in same symbols, by the same TRP associated with the same ARP, from the same RF chain (i.e. the same antenna) - The same number of symbols, symbol location within one slot, repetition factor, - The same numerology, i.e. the same CP and SCS - The same or different bandwidths - The same comb size - The same power per subcarrier - Aggregated PFLs are configured on the same aligned numerology grid - Phase continuity between aggregated PFLs Proposal 2: The legacy measurement period in R17 can be as baseline when defining the new measurement period for PRS bandwidth aggregation. Proposal 3: The PRS periodicities from different PFLs are the same and it is not the strong influence factor when defining the measurement period. Proposal 4: The referent PFL(r) can be defined, the measurement period requirement of a referent PFL(r) can be used as the reference of other PFLs’ measurement period requirement in the same group. |
| R4-2308468 | OPPO | Proposal 2: Consider TEG to defined measurement requirements for aggregated PFLs. Proposal 3: To define the measurement period requirements for aggregated PFLs, use the existing requirements in Rel-17 as the baseline and update the total number of PFLs, i.e. L. |
| R4-2308671 | Huawei, HiSilicon | Proposal 1: Requirements for PRS CA are applicable provided that LMF requests UE to perform joint measurement across aggregated PFL. Proposal 2: Requirements for PRS CA are applicable to PRS resource sets or PRS resources across PFLs that are linked. Other conditions can be discussed based on RAN1 progress. Proposal 3: Existing measurement period requirements can be used as baseline for defining PRS CA requirements, and aggregated PFLs are considered as one PFL. Proposal 4: RAN4 to discuss the impact of PRS resource dropping in one or more PFLs on the requirements based on further RAN1 agreements. Proposal 5: RAN4 to discuss possible impacts of SRS CA based on RAN1 progress. Proposal 6: RAN4 to update the report mapping for RSTD and UE Rx-Tx - FR1: support range of reporting granularity from 32Tc to [1]Tc - FR2: support range of reporting granularity from 32Tc to [0.25]Tc - Define new mapping table for reporting granularity of 0.5Tc and 0.25Tc |
| R4-2308794 | Ericsson | Proposal 1 : RAN4 to not define requirements for PRS bandwidth aggregation outside of the MG in RRC_CONNECTED state. Proposal 2: Rel. 17 gap-based measurement period requirement for RRC_CONNECTED mode is used as a baseline for defining measurement period requirement for PRS bandwidth aggregation within MG in RRC_CONNECTED mode.s Proposal 3: Rel. 17 measurement period requirement for RRC_INACTIVE mode is used as a baseline for defining measurement period requirement for PRS bandwidth aggregation in RRC_INACTIVE mode. Proposal 4: Update report mapping for RSTD and UE Rx-Tx with PRS/SRS bandwidth aggregation based on maximum supported aggregated SRS/PRS bandwidth and performance evaluation. |
| R4-2309134 | Qualcomm Incorporated | Observation 1: All the signalling options being considered by RAN1 to request PRS BW aggregation allow a mix of PRS resources to be measured with and without aggregation within one PFL. Proposal 1: To define the PRS measurement period requirement for RSTD and UE Rx-Tx when the LMF requests the UE to aggregate PRS resources across groups of PFLs (super-PFLs), as a baseline, the measurement period can be defined as a summation of two terms: - The first term equals the Rel-17 the measurement period requirement, with the modification that only PRS resources that are not aggregated, according to the LMF request and applicability conditions, are counted in Lavailable_PRS,i for each PFL. - The second term is the summation of the measurement delays for the super-PFLs. As a baseline, only PRS resources to be aggregated according to the LMF request and applicability conditions, are counted in Lavailable_PRS,i for each super-PFL. - FFS whether any additional modifications to the first term are needed due to new capabilities defined by RAN1 for PRS BW aggregation. - FFS the exact formula for the delay for each super-PFL based on RAN1 agreements on UE capabilities, etc. - FFS how to handle the scenario where the PRS RSTD reference is not aggregated - FFS any changes to the measurement requirement for PRS-RSRP and/or PRS-RSRPP when reported as supporting measurements for DL-TDOA and multi-RTT Proposal 2: Wait for further progress in RAN1 to finalize the definition of the measurement period requirement. Proposal 3: RAN4 to discuss requirements for a switching/guard period between UL transmissions within active BWP and transmissions outside carriers/BWP in support of positioning BW aggregation. |
| R4-2309677 | Nokia, Nokia Shanghai Bell | - RAN4 to wait on further RAN1 agreements related to maximum total aggregated bandwidth for PRS/SRS and on equal/unequal bandwidths of aggregated PFLs. - RAN4 to focus on MG assisted PRS measurements and wait on RAN1 agreements related to the support of PPW for gapless measurements in RRC_CONNECTED state. - RAN4 to investigate the case of PRS collision with SSB or other DL channels for one aggregated PFL and wait for further RAN1 agreements related to supported total aggregated BW and equal/unequal bandwidths of aggregated PFLs. - RAN4 to base RRM requirements for RRC_INACTIVE on the scenario that aggregated carriers with SRS for positioning are located outside the initial UL BWP. - RAN4 to define measurement reporting requirements including measurement delay with measurement gaps for RRC_CONNECTED and for RRC_INACTIVE states. - RAN4 to specify RSTD and UE Rx-Tx time difference measurement reports to be common for all CCs/PFLs. - RAN4 to investigate whether to specify PRS-RSRP and PRS-RSRPP measurement reports to be either common for all CCs/PFLs or individual for each CC/PFL as configured by LMF. - Proposal 9: RAN4 to define measurement accuracy requirements for PRS/SRS aggregation for two and three contiguous carriers/PFLs, respectively. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1: General aspects/scenarios for PRS/SRS BW aggregation

Sub-topic description:

Open issues and candidate options before meeting:

In RAN4#106bis-e, following was agreed in the WF in R4-2306349:

•	Define the core requirements for PRS BW aggregation in both RRC_INACTIVE and RRC_CONNECTED states.

•	Do not define requirements for RRC_IDLE state.

Issue 3-1-1: Applicable RRC states

- Proposals

  - Option 1: E///

    - RAN4 to not define requirements for PRS bandwidth aggregation outside of the MG in RRC_CONNECTED state.

- Recommended WF

  - Discuss the option.

Issue 3-1-2: Applicable bandwidth

- Proposals

  - Option 1: Nokia

    - RAN4 to wait on further RAN1 agreements related to maximum total aggregated bandwidth for PRS/SRS and on equal/unequal bandwidths of aggregated PFLs.

- Recommended WF

  - Discuss the option.

### Sub-topic 3-2: PRS measurement requirements for PRS/SRS bandwidth aggregation

Sub-topic description

Open issues and candidate options before meeting:

Issue 3-2-1: Conditions for PRS/SRS bandwidth aggregation requirements:

- Proposals

  - Option 1: ZTE

    - RAN4 shall study the core requirements based on the current RAN1 progress as below:

      - In the same slot, in same symbols, by the same TRP associated with the same ARP, from the same RF chain (i.e. the same antenna)

      - The same number of symbols, symbol location within one slot, repetition factor,

      - The same numerology, i.e. the same CP and SCS

      - The same or different bandwidths

      - The same comb size

      - The same power per subcarrier

      - Aggregated PFLs are configured on the same aligned numerology grid

      - Phase continuity between aggregated PFLs

  - Option 2: HW

    - Requirements for PRS CA are applicable provided that LMF requests UE to perform joint measurement across aggregated PFL.

    - Requirements for PRS CA are applicable to PRS resource sets or PRS resources across PFLs that are linked. Other conditions can be discussed based on RAN1 progress.

- Recommended WF

  - Discuss the options.

Issue 3-2-2: Impact of number of PFLs on PRS/SRS bandwidth aggregation requirements:

- Proposals

  - Option 1: LG

  - RAN4 to consider the PFL group concept which means aggregated PFLs. For example, in the RSTD case

TRSTD,Total=j=1GPRSTD,j+G-1×maxPeffect, j

where,

j: Index of PFL group

G: Total number of PFL groups

Peffect,j: Periodicity of the PRS RSTD measurement in PFL group j

PRSTD,j: Measurement period for PRS RSTD measurement in PFL group j

  - Option 2: ZTE

    - The PRS periodicities from different PFLs are the same and it is not the strong influence factor when defining the measurement period.

    - The referent PFL(r) can be defined, the measurement period requirement of a referent PFL(r) can be used as the reference of other PFLs’ measurement period requirement in the same group.

- Recommended WF

  - Discuss the options.

Issue 3-2-3: PRS measurement period for PRS/SRS bandwidth aggregation

- Proposals

  - Option 1: ZTE, HW, OPPO, E///

    - Existing measurement period requirements can be used as baseline for defining PRS CA requirements, and aggregated PFLs are considered as one PFL.

    - Option 1A: E///

      - Rel. 17 gap-based measurement period requirement for RRC_CONNECTED mode is used as a baseline for defining measurement period requirement for PRS bandwidth aggregation within MG in RRC_CONNECTED mode.

      - Rel. 17 measurement period requirement for RRC_INACTIVE mode is used as a baseline for defining measurement period requirement for PRS bandwidth aggregation in RRC_INACTIVE mode.

  - Option 2: QC

    - To define the PRS measurement period requirement for RSTD and UE Rx-Tx when the LMF requests the UE to aggregate PRS resources across groups of PFLs (super-PFLs), as a baseline, the measurement period can be defined as a summation of two terms:

      - The first term equals the Rel-17 the measurement period requirement, with the modification that only PRS resources that are not aggregated, according to the LMF request and applicability conditions, are counted in L_(available_PRS,i) for each PFL.

      - The second term is the summation of the measurement delays for the super-PFLs. As a baseline, only PRS resources to be aggregated according to the LMF request and applicability conditions, are counted in  L_(available_PRS,i) for each super-PFL.

      - FFS whether any additional modifications to the first term are needed due to new capabilities defined by RAN1 for PRS BW aggregation.

      - FFS the exact formula for the delay for each super-PFL based on RAN1 agreements on UE capabilities, etc.

      - FFS how to handle the scenario where the PRS RSTD reference is not aggregated

      - FFS any changes to the measurement requirement for PRS-RSRP and/or PRS-RSRPP when reported as supporting measurements for DL-TDOA and multi-RTT

    - Wait for further progress in RAN1 to finalize the definition of the measurement period requirement.

  - Option 3: Nokia

    - RAN4 to focus on MG assisted PRS measurements and wait on RAN1 agreements related to the support of PPW for gapless measurements in RRC_CONNECTED state.

    - RAN4 to define measurement reporting requirements including measurement delay with measurement gaps for RRC_CONNECTED and for RRC_INACTIVE states.

- Recommended WF

  - Discuss the options.

Issue 3-2-4: Impact of SRS CA for communication on PRS/SRS bandwidth aggregation

- In RAN4#106bis-e, following was agreed in the WF in R4-2306349:

  - Any impact of the CA for communication on the SRS aggregation (UL part) is up to RAN1 discussion.

- Proposals

  - Option 1: HW

    - RAN4 to discuss possible impacts of SRS CA based on RAN1 progress.

  - Option 2: Nokia

    - RAN4 to base RRM requirements for RRC_INACTIVE on the scenario that aggregated carriers with SRS for positioning are located outside the initial UL BWP.

- Recommended WF

  - Discuss the options.

Issue 3-2-5: Guard period between data and positioning period in UL

- Proposals

  - Option 1: QC

    - RAN4 to discuss requirements for a switching/guard period between UL transmissions within active BWP and transmissions outside carriers/BWP in support of positioning BW aggregation.

  - Option 2: LG

    - RAN4 to wait for more RAN1 progress to discuss the guard period.

- Recommended WF

  - Discuss the options.

Issue 3-2-6: TEG for PRS/SRS bandwidth aggregation

- Proposals

  - Option 1: OPPO

    - Consider TEG to defined measurement requirements for aggregated PFLs.

- Recommended WF

  - Discuss the option.

Issue 3-2-7: Impact of PRS resource dropping on PRS/SRS bandwidth aggregation requirement

- Proposals

  - Option 1: HW

    - RAN4 to discuss the impact of PRS resource dropping in one or more PFLs on the requirements based on further RAN1 agreements.

  - Option 2: Nokia

    - RAN4 to investigate the case of PRS collision with SSB or other DL channels for one aggregated PFL and wait for further RAN1 agreements related to supported total aggregated BW and equal/unequal bandwidths of aggregated PFLs.

- Recommended WF

  - Discuss the options.

### Sub-topic 3-3: PRS measurement report mapping for PRS/SRS bandwidth aggregation

Sub-topic description

Open issues and candidate options before meeting:

In RAN4#106bis-e, following was agreed in the WF in R4-2306349:

•	Identify whether to reuse the existing report mapping or update the report mapping for RSTD and UE Rx-Tx with PRS/SRS bandwidth aggregation based on:

o	Maximum supported aggregated SRS/PRS bandwidth and

o	RSTD and UE Rx-Tx measurement accuracies/performance evaluation.

Issue 3-3-1: Whether report mappings with PRS/SRS bandwidth aggregation need to be updated?

- Proposals

  - Option 1: LG

    - Existing report mapping can be reused for RSTD and UE Rx-Tx. If necessary RAN4 can make a decision after performance evaluation.

  - Option 2: HW

    - RAN4 to update the report mapping for RSTD and UE Rx-Tx

      - FR1: support range of reporting granularity from 32Tc to [1]Tc

      - FR2: support range of reporting granularity from 32Tc to [0.25]Tc

      - Define new mapping table for reporting granularity of 0.5Tc and 0.25Tc

  - Option 3: E///

    - Update report mapping for RSTD and UE Rx-Tx with PRS/SRS bandwidth aggregation based on maximum supported aggregated SRS/PRS bandwidth and performance evaluation.

- Recommended WF

  - Discuss the options.

Issue 3-3-2: RSTD and UE Rx-Tx time difference measurement report

- Proposals

  - Option 1: Nokia

- RAN4 to specify RSTD and UE Rx-Tx time difference measurement reports to be common for all CCs/PFLs.

- RAN4 to investigate whether to specify PRS-RSRP and PRS-RSRPP measurement reports to be either common for all CCs/PFLs or individual for each CC/PFL as configured by LMF.

- Recommended WF

  - Discuss the option.

### Sub-topic 3-4: Accuracy requirement for PRS/SRS bandwidth aggregation

In RAN4#106bis-e, following was agreed in the WF in R4-2306349:

- The number of PFLs for which PRS accuracy requirements for PRS/SRS bandwidth aggregation shall be defined can be discussed during the performance part of the WI.

- PRS measurement accuracy requirements for PRS/SRS BW aggregation will be discussed and defined during the performance part.

Issue 3-4-1: Number of carriers/PFLs for accuracy requirement

- Proposals

  - Option 1: Nokia

- RAN4 to define measurement accuracy requirements for PRS/SRS aggregation for two and three contiguous carriers/PFLs, respectively.

- Recommended WF

  - Based on agreement at RAN4#106bis-e (R4-2306349), it is recommended to postpone this issue until the performance part of the WI starts.