3GPP TSG-RAN WG4 Meeting #107                                                               R4-2310041

Incheon, Korea Republic, May 22nd – 26th, 2023

Agenda item:			8.13.6

Source:	Samsung

Title:	WF on FR2 HST RRM requirements (part 1)

Document for:	Approval

Topic #1: Simultaneous multi-panel operation for train roof-mounted FR2 high power devices

Sub-topic 1-1 General discussion on RRM requirement impact

Issue 1-1-5: The applicability and necessity of GBBR in FR2 HST - Agreements from Monday RRM session - Define the following requirements for Rel-18 enhanced FR2 CPE - 1) Requirements for UEs without mandatory support of GroupBasedBeamReporting-r17 - FFS if a new UE capability is needed to indicate support of simultaneous data reception - 2) Requirements for UEs with support of GroupBasedBeamReporting-r17 - FFS if GBBR (GroupBasedBeamReporting-r17) can be applicable for the case of RTD > CP. If it is not applicable, then the requirements will not be defined.

Issue 1-1-5: The applicability and necessity of GBBR in FR2 HST

- Agreement (Updated agreement from Thursday Ad-Hoc Session):

  - FR2 HST requirement shall be defined without GroupBasedBeamReporting-r17 supported and GBBR configured

    - FFS the performance degradation if GBBR is not configured.

New Issue related to Issue 1-1-5: Applicable condition for FR2 HST multi-RX in the spec, if without GBBR as prerequisite:

- Agreement from Thursday Ad-Hoc Session:

  - Applicable condition for FR2 HST multi-panel requirement, if without GBBR as prerequisite

    - FR2 PC6 UE shall support the multi-panel simultaneous reception capability

      - FFS new or existing (simultaneousReceptionDiffTypeD-r16) capability for multi-panel simultaneous reception

    - NW shall configure Rel-17 HST flag (highSpeedMeasFlagFR2) and bi-direction flag (i.e., highSpeedDeploymentTypeFR2)

Whether to consider/enhance CSI-RS based requirements for multi-panel RX in Rel-18 FR2 HST WI?

- Agreement from Thursday Ad-Hoc Session:

  - RAN4 shall preclude any enhancement for all the CSI-RS based requirements for FR2 HST multi-panel RX, which include:

    - Requirements for CSI-RS based RLM

    - Requirements for CSI-RS based BFD/CBD

    - Requirements for CSI-RS based L1-RSRP measurement

    - Measurement restriction for CSI-RS based L1-RSRP

Whether to consider/enhance SSB based CBD requirements for multi-panel RX in Rel-18 FR2 HST WI?

- Agreement:

  - RAN4 shall preclude any enhancement on the SSB based CBD requirements for FR2 HST multi-panel RX.

Whether to consider/enhance simultaneous L3+L3, L3+L1 and L1+L1 measurement for multi-panel RX in Rel-18 FR2 HST WI?

- Agreement from Thursday Ad-Hoc Session:

  - RAN4 shall preclude the simultaneous L3+L3 measurement for multi-panel RX in Rel-18 FR2 HST WI.

  - RAN4 shall preclude the simultaneous L3+L1 measurement for multi-panel RX in Rel-18 FR2 HST WI.

  - RAN4 shall consider the simultaneous L1+L1 measurement for multi-panel RX in Rel-18 FR2 HST WI, and new requirements shall be defined accordingly

    - FFS applicable condition of the enhancement for new requirement:

      - Restriction on SSB configuration for neighboring RRHs

      - Enhancement on only set1, or both set1 and set2.

      - The sweeping factor N1 can be reduced in FR2 HST multi-panel simultaneous reception

        - FFS how to reduce N1

Whether to consider/enhance simultaneous data+L1 measurement and data+L3 measurement for multi-panel RX in Rel-18 FR2 HST WI?

- Agreement from Thursday Ad-Hoc Session:

  - RAN4 shall preclude any enhancement on the scheduling restriction requirement for simultaneous L3 measurement and data reception in Rel-18 FR2 HST.

  - RAN4 shall consider data+L1 measurement for Rel-18 FR2 HST multi-panel RX:

    - FFS The necessity of scheduling restriction requirement enhancement for data+ L1 measurement in Rel-18 FR2 HST

    - RAN4 should introduce enhancement on the scheduling restriction requirement for L1-RSRP in Rel-18 FR2 HST

      - FFS applicable condition of the enhancement for new requirement

    - FFS whether RAN4 should preclude any enhancement on the scheduling restriction requirement for RLM in Rel-18 FR2 HST

Sub-Topic 1-2: MRTD and panel switching for FR2 HST multi-panel simultaneous reception

Issue 1-2-1: The impact to MRTD requirements

- Agreement from Thursday Ad-Hoc Session:

  - For Rel-18 FR2 PC6 UE, the new MRTD requirement shall be defined for simultaneous reception from two panels:

    - MRTD = [8]us

Issue 1-2-2: Turning on/off the multi-panel reception operations in FR2 HST

- Agreement from Thursday Ad-Hoc Session:

  - The necessity of a mechanism for turning on/off the multi-panel reception operation in FR2 HST:

    - No need for a new mechanism for turning on/off the multi-panel reception operation in FR2 HST;

    - Existing signaling and configuration shall be used.

Sub-topic 1-4: The impact to uplink spatial relation switch delay requirements

- Agreement from Thursday Ad-Hoc Session:

  - Keep current uplink spatial relation switch delay requirement for Rel-18 FR2 HST multi-panel RX:

    - It is RAN4’s understanding that UL spatial relation switch to the target associated to the unknown DL RS in FR2 HST scenarios shall not be considered:

      - Capture the above understanding in RAN4 WF.

RRM requirement impact summary

- Agreement:

  - Below green highlighted items from Thursday Ad-Hoc Session:

  - Below yellow highlighted items agreed in this WF

- Table 1. RRM requirement impact summary for FR2 HST multi-panel RX (informative)

| RRM Req. Category | Sub-Category | Proposed WF on RRM requirement impact | Work Split |
| --- | --- | --- | --- |
| RRC_IDLE state mobility | Cell Re-selection | Precluded |  |
| RRC_CONNECTED state mobility | Handover | Precluded |  |
|  | RRC Connection Mobility Control | Precluded |  |
| Timing | MRTD | New requirement required |  |
| Signalling characteristics | Requirements for SSB based RLM | New requirement required: - 8.1.2.2 Minimum requirement 8.1.2.3 Measurement restrictions for SSB based RLM |  |
|  | Requirements for CSI-RS based RLM | Precluded |  |
|  | Scheduling availability of UE during RLM | FFS 8.1.7.3 |  |
|  | Requirements for SSB based BFD | New requirement is required 8.5.2.2 Minimum requirement 8.5.2.3 Measurement restriction for SSB based beam failure detection |  |
|  | Requirements for SSB based CBD | Precluded |  |
|  | Requirements for CSI-RS based BFD/CBD | Precluded |  |
|  | Scheduling availability of UE during BFD/CBD | Precluded |  |
|  | Active TCI state switching delay | To be treated UL TX timing |  |
|  | Uplink spatial relation switch delay | Precluded |  |
| Measurement Procedure | NR intra-frequency measurements w/o gaps | Precluded |  |
|  | Measurement Requirements for SSB based L1-RSRP | New requirement is required - 9.5.4.1 SSB based L1-RSRP Reporting; - 9.5.5.1Measurement restriction for SSB based L1-RSRP |  |
|  | Measurement Requirements for CSI-RS based L1-RSRP | Precluded |  |
|  | Scheduling availability of UE during L1-RSRP measurement | New requirement required |  |
|  | L1-SINR measurements for Reporting | Precluded |  |

Topic #2: Definition of HST FR2 CA requirements and NW signaling for Rel-18 FR2 HST CA Scenario

Sub-topic 2-1 Discussion on the necessity of the requirements

Issue 2-1-1: Clarification on requirement for intra-frequency measurement for Rel-18 FR2 HST UE supporting intra-band CA

- Agreement on Monday RRM session:

  - Reuse legacy Rel-17 requirements for NR intra-frequency measurement for Rel-18 FR2 HST UE supporting intra-band CA.

Issue 2-1-2: PSS/SSS detection, Time index detection for inter-frequency measurements

PSS/SSS detection

- Agreement on Monday RRM session:

  - Specify the requirements for inter-frequency PSS/SSS detection

- Agreement:

  - RAN4 to use Rel-17 FR2-HST principles to define inter-frequency PSS/SSS detection, i.e., use beam sweeping scaling factor N1 of 2 and 6 for set1 and set2, respectively

Time index detection requirements

- Agreement

  - Specify the requirements for inter-frequency SSB index identification when deriveSSBIndexFromCellInter-r17 disabled.

- Agreement

  - RAN4 to use Rel-17 FR2-HST principles to define SSB index identification as a baseline, i.e., use beam sweeping scaling factor N1 of 2 and 6 for set1 and set2, respectively.

Issue 2-1-4: SCell activation delay

- Agreement

  - FFS the RRM impact for SCell activation delay requirement for Rel-18 FR2 HST UE

Issue 2-1-5: Inter-frequency measurement requirements in Idle mode

The necessity of specifying the requirements for inter-frequency measurement requirements in idle mode

- Agreement

  - Specify the requirements for inter-frequency measurement requirements in idle mode.

How to define the requirements for inter-frequency measurement requirements in idle mode

- Agreement

  - RAN4 to use Rel-17 FR2-HST principles to define inter-frequency measurement requirements in idle mode, i.e., use beam sweeping scaling factor N1 of 2 and 6 for set1 and set2, respectively

    - For Rel-18 PC6 UE supporting Rel-18 FR2 HST intra-band CA with [highSpeedMeasFlagFR2-r17] configured, RX sweeping factor is assumed as follows to derive inter-frequency measurement in IDLE mode: RX beam sweeping factor is 2 if [highSpeedMeasFlagFR2-r17] = set1; RX beam sweeping factor is 6 if [highSpeedMeasFlagFR2-r17] = set2

For Rel-18 PC6 UE supporting Rel-18 FR2 HST intra-band CA with [highSpeedMeasFlagFR2-r17] configured, for inter-frequency measurement in IDLE mode - The measurement requirement of inter-frequency NR cells is given in the below table Table 4.2.2.3-x: Tdetect,NR_Inter_HST, Tmeasure, NR_Inter_HST and Tevaluate, NR_Inter_HST for FR2 configured with highSpeedMeasFlagFR2-r17 note2 DRX cycle length [s]; Scaling Factor (N1); Tdetect,NR_Inter_HST [s] (number of DRX cycles); Tmeasure, NR_Inter_HST [s] (number of DRX cycles); Tevaluate, NR_Inter_HST [s] (number of DRX cycles). 0.32; N1Note3; 2.56 x N1 x M2 (8 x N1 x M2) Note1; 0.32 x N1 x M3 (1 x N1 x M3) Note1; 0.96 x N1 x M4 (3 x N1 x M4) Note1. 0.64; 5; 17.92 x N1 (28 x N1); 1.28 x N1 (2 x N1); 5.12 x N1 (8 x N1). 1.28; 4; 32 x N1 (25 x N1); 1.28 x N1 (1 x N1); 6.4 x N1 (5 x N1). 2.56; 3; 58.88 x N1 (23 x N1); 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). Note 1: When SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 Note 2: The support of HST Idle mode inter-frequency measurement enhancement is optional without capability signalling. Apply for UE declarating supports idle mode inter-frequency measurement enhancement for HST, otherwise Table 4.2.2.4-1 shall be used. Note 3: N1 = 2 when highSpeedMeasFlagFR2-r17= set1; N2 = 6 when highSpeedMeasFlagFR2-r17= set2..

Issue 2-1-6: Expected RRM requirement impact

- Agreement:

Table 2. RRM requirement impact summary for FR2 HST CA requirement (informative)

| RRM Req. Category | Proposed WF on RRM requirement impact | Work Split |
| --- | --- | --- |
| PSS/SSS detection, Time index detection for intra-frequency and inter-frequency measurements | For PSS/SSS detection, Time index detection for inter-frequency measurements: New requirement required For PSS/SSS detection, Time index detection for intra-frequency measurements: No impact identified |  |
| L1-RSRP measurement on SCell | No impact identified |  |
| SCell activation delay | FFS |  |
| Inter-frequency measurement requirements in Idle mode | New requirement required |  |
| SSB-based BFD and RLM on SCell | No impact identified |  |
| TCI state switching on SCell | No impact identified |  |

*Requirements were classified in the following five categories:

    - New requirement required: Need to define new requirements in Rel-18 FR2 HST

    - No impact identified: No impact on the existing requirement identified