3GPP TSG-RAN WG4 Meeting #107	R4-2310069

Incheon, KR, 22 – 26 May, 2023

Title: 	WF on NR Dual TxRx Multi-SIM

Agenda Item:	8.26.3

Source: 	vivo

Document for:	Approval

Topic #1: General aspects

Sub-topic 1-1 General aspects

Issue 1-1-1: Clarification on the scope

- Proposals

  - P1: Add the following note for the sentence “Case 2: Collisions between MUSIM gap and SMTC” (Apple Huawei vivo Qualcomm MTK)

    - Note: The scope collisions between MUSIM gap and SMTC will be limited to RRM procedures for which collisions between legacy measurement gaps and SMTC are taken into account in the existing requirements.

  - P2: The collision between SMTC for Handover/SCell activation is in the scope (Ericsson)

  - P3: Add a high-level clarification in RAN4 spec that during one-shot procedure such as SCell activation, SI update and so on, UE is not expected to enable MUSIM gaps unless existing RRM requirement for the corresponding one-shot procedure can be met (Apple)

  - P4: The issue is already covered by existing scenarios in section 2-4 (Huawei Nokia)

Recommendations: Covered by related issues in section 2.

Tentative agreement:

Close this issue

Issue 1-1-2: MUSIM overhead

- Proposals:

  - Option 1: Do not define overhead cap for MUSIM gaps (CMCC Apple Ericsson Huawei vivo Qualcomm)

  - Option 2: Define overhead cap for MUSIM gaps. (xiaomi oppo)

    - Option 2a: Measurement requirement does not apply when more than one MUSIM gap is configured with MGRP = [20] ms (xiaomi)

    - Option 2b: Measurement requirement does not apply when more than 2 gaps are configured with MGRP<=40ms in an FR. FFS other overhead cap rules. (oppo)

Recommendations: This topic has been discussed for a few meetings and suggest to follow majority view.

Tentative agreement:

Option 1.

Issue 1-1-3: Mandatory MUSIM gap patterns

- Proposals

  - P1: No need to discuss further whether to introduce mandatory MUSIM gap patterns (Apple oppo Huawei Nokia Qualcomm MTK)

  - P2: RAN4 to define the mandatory MUSIM gap patterns (CMCC Ericsson Nokia)

  - P3: No more discussion if there is no consensus (vivo)

Issue 1-1-4: General rule on properties for NW-A and NW-B procedures

- Proposals

  - P1: RAN4 to define the priorities for each procedure in either NW-A or NW-B in descending order as follow. The gaps or resources for higher priority procedures should be kept once the collision happens (Ericsson)

    - Level 1: One-shot RRM mobility procedures in NW-A, such as Handover/SCell activation/SI update;

    - Level 2: Periodic paging monitoring or one-shot procedure in NW-B Idle mode, such as On-demand SI reading;

    - Level 3: Measurements procedures for both NW-A and NW-B

  - P2: Add a high-level clarification in RAN4 spec that during one-shot procedure such as Scell activation, SI update and so on, UE is not expected to enable MUSIM gaps unless existing RRM requirement for the corresponding one-shot procedure can be met. (Apple)

  - P3: No need to define properties for procedures at NW A or NW B as suggested by P1 (Huawei Nokia vivo Qualcomm)

Recommendations: Based on the latest agreement it is not necessary to discuss these general principles.

Tentative agreement:

Close this issue

Issue 1-1-5: Others

- Proposals

  - P1: UE shall not request MUSIM gaps beyond the UE capacity considering the UEs current configuration (Nokia)

  - P2: UE shall not request more MUSIM gaps than it is capable of handling with the current measurement gap allocation (Nokia)

Topic #2: Collisions between gaps and priority rules

Sub-topic 2-1 MUSIM gap priority configuration

Issue 2-1-4-1: Constraints on MUSIM gap priority configuration from NW A

- Proposals

  - P1: NW A maintains the same relative priorities requested by the UE; The configured priority level may or may not be the same as that requested by UE. (vivo Apple xiaomi Huawei Nokia Qualcomm MTK)

    - P1-a: NW A will keep the same relative priority order indicated by a UE however when one or multiple or all MUSIM gap’s MGRP less than a threshold, NW A will not keep the relative order for those MUSIM gaps or all MUSIM gaps (vivo)

  - P2: When MUSIM gaps with equal priority is allowed, if UE requests two MUSIM gaps with the same priority X and if the network configures both gaps, then both gaps must be assigned a common priority X’. X’ may or may not be equal to X. (oppo Qualcomm)

  - P3: If network A cannot fulfill the priority configuration requested by UE for MUSIM gaps, it may choose not to configure one or more of the MUSIM gaps. (Qualcomm Nokia)

  - P4: When UE requesting MUSIM gap priority for all or a subset MUSIM gaps, the priority levels are different (xiaomi Nokia)

  - P5: No need to discuss further constraints on MUSIM gap priority configuration for NW A. (CMCC Ericsson)

  - P6: If equal priority is allowed, up two periodic MUSIM gaps can be configured with the same priority and inform such the configuration to RAN2 (oppo)

Issue 2-1-4-2: Constraints on MUSIM gap request from UE side

- Proposals

  - P1: There shall be a minimum MGRP defined for the requested MUSIM gap pattern (Nokia)

  - P2: When UE requests the MUSIM gaps, the MGRP of highest priority gap should be larger than 160ms; When UE requests only one MUSIM gap, the MGRP should be larger than 80ms (Ericsson)

  - P3: Do not define constraints on MUSIM gap request from UE side (Huawei vivo Qualcomm)

  - P4: Network A will configure the MUSIM gap priority requested by the UE under the following conditions (Qualcomm)

    - If the UE requests multiple MUSIM gaps, the MUSIM gap that the UE requests with the highest priority has MGRP larger than 160 ms.

    - If the UE requests only one MUSIM gap, the MUSIM gap has MGRP larger than 80 ms.

Issue 2-1-5: Priority setting for aperiodic MUSIM gaps

Note: Option 1 and 2 are agreements from GTW at RAN4 106bis

  - Option 1 (CMCC xiaomi Nokia Qualcomm vivo)

    - The priority level of aperiodic MUSIM gap can be configured by NW A

    - If the priority level is not configured by NW A then the aperiodic MUSIM gap by default has the highest priority level

    - The aperiodic MUSIM gap priority level can be optionally requested by UE from NW A

  - Option 2 (Apple ZTE oppo Huawei MTK):

    - Aperiodic MUSIM gap by default has the highest priority level.

    - The gap priority level is not explicitly configured by the NW

  - Option 3: The aperiodic MUSIM gap by default has the highest priority level. The priority level of aperiodic MUSIM gap can be configured by NW A (Ericsson)

Recommendations: Option 1 and 2 are agreements from RAN4 106bis. Suggest to down-select from option 1 and 2.

Issue 2-1-6: Order for applying the priority

- Proposals:

  - P1: when number of colliding MGs is larger than 2, collisions between gaps are resolved sequentially in order of decreasing priority, starting with the gap that has the highest priority (Apple oppo Huawei Qualcomm MTK vivo)

  - P2: when number of colliding MGs is larger than 2, RAN4 to postpone multiple gap collision issue until RAN4 has a clear understanding on MUSIM gaps’ priority. (Ericsson Nokia)

  - P3: When multiple gaps collide, it will be the gap with the highest priority that is used by the UE and other lower priority gaps are dropped. (Nokia)

  - P4: When at most 2 gap collide at each time instance however there are consecutive collisions, the priority rule should be applied with a chronological order. (vivo)

Issue 2-1-7: Further clarifications on MUSIM gap priority

- Proposals:

  - P1: The priorities among all configured gaps shall be comparable, including MUSIM and non-MUSIM gaps (Nokia)

Recommendations: This issue can be discussed under issue 2-2-2-1

Sub-topic 2-2 On collision between different MUSIM gaps

Issue 2-2-1: Definition of the collision between different MUSIM gaps

- Proposals

  - Option 1: The gap proximity condition for the Rel-17 concurrent gap collision should be reused for the collision between different MUSIM gap when priority rules are used to handle the collision between MUSIM gaps (CMCC Apple xiaomi ZTE oppo MTK vivo)

  - Option 2: Postpone the discussion till issue 2-2-2 is stable (Huawei)

  - Option 3: A collision between MUSIM gaps means a physical overlap in time domain between two MUSIM gaps and RAN4 does not define ‘proximity’ for collisions between MUSIM gaps. Considering the following cases: (Nokia)

    - Fully non-overlapping MUSIM gaps: All MUSIM gaps are disjoint in time.

    - Fully non-overlapping MUSIM gaps and non-MUSIM gaps: All MUSIM gaps and non-MUSIM gaps are disjoint in time.

    - Fully overlapping MUSIM gaps: Every MUSIM gap occasion of one MUSIM MG pattern is fully covered by every MUSIM gap occasion of another MUSIM MG pattern.

    - Fully overlapping MUSIM gaps and non-MUSIM gaps: Every MUSIM gap occasion of one MUSIM MG pattern is fully covered by every gap occasion of a non-MUSIM MG pattern.

    - Fully Partial overlapped MUSIM gaps: Every MUSIM gap occasion of one MUSIM MG pattern is partially overlapped by every MUSIM gap occasion of another MUSIM MG pattern.

    - Fully Partial overlapped MUSIM gaps and non-MUSIM gaps: Every MUSIM gap occasion of one MUSIM MG pattern is partially overlapped by every gap occasion of a non-MUSIM MG pattern.

    - Partially fully overlapped MUSIM gaps: every gap occasion of a MUSIM MG pattern is fully overlapped by another MUSIM MG pattern with different MGRP.

    - Partially fully overlapped MUSIM gaps and non-MUSIM gaps: Every MUSIM gap occasion of one MUSIM MG pattern is fully overlapped by a gap occasion of a non-MUSIM MG pattern with different MGRP.

    - Partially partial overlapped MUSIM gaps: Every MUSIM gap occasion of one MUSIM MG pattern is partially overlapped with a gap occasion of another MUSIM MG pattern with different MGRP.

    - Partially partial overlapped MUSIM gaps and non-MUSIM gaps: Every MUSIM gap occasion of one MUSIM MG pattern is partially overlapped with a gap occasion of a non-MUSIM MG pattern with different MGRP.

Recommendations: Since priority based solution are included in the MUSIM gap collision solution in issue 2-2-2, option 1 is supposed to be agreeable.

Tentative agreement:

Agree option 1.

Issue 2-2-2: Solutions for collision between different MUSIM gaps

- Proposals

  - Option 1: Priority based solution is used for collision handling between different MUSIM gaps (xiaomi MTK)

  - Option 2: Keep solution (keep all collided MUSIM gap) is used for collision handling between different MUSIM gaps (Huawei)

  - Option 3: Use both Priority based solution and Keep solution for collision handling between different MUSIM gaps (vivo Apple xiaomi ZTE oppo Huawei Qualcomm)

Agreements

  - Define two solutions for collision handling between different MUSIM gaps

    - 1) Priority based solution (i.e., network controls the MUSIM gaps priority)

    - 2) “Keep” solution (i.e., keep all collided MUSIM gaps)

  - FFS on the mechanism to select and/or switch between the solutions

Issue 2-2-2-1: Conditions when “keep solution” is used

- Proposals

  - Note: For P1 it needs to determine whether “equal priority” is allowed or not. Using P2 means there is no necessity to have equal priority between different MUSIM gaps.

  - P1: Use priority information provided by UE when UE requests MUSIM gaps to indicate when “keep solution” is used (vivo Apple oppo Qualcomm)

    - P1-1: “Keep solution” is used when MUSIM gaps have equal priority level. (vivo Apple oppo Qualcomm)

    - P1-2: “keep solution” is used when two MUSIM gaps has different priority, and the priority between them is less than or equal to a particular threshold in case there are concerns on “equal priority” (vivo)

  - P2: Introduce explicit bits in MUSIM gap request signalling to allow UE to indicate when “keep solution” is used (vivo Huawei Qualcomm)

    - P2-1: Use one bit to indicate “keep solution” are used to all MUSIM gaps (Huawei vivo)

    - P2-2: Introduce one bit for each MUSIM gaps to indicate whether “keep solution” will be used or not when it collides with other MUSIM gaps. (vivo)

  - P3: the MUSIM gaps are regarded as collision based the collision definition, and the collided MUSIM gaps are for paging reception, SSB measurement, or SI reading in the same frequency layer (xiaomi)

  - P4: The kept/merged solution is used for scenarios like paging (ZTE Ericsson)

  - P5: RAN4 shall define the conditions when colliding MUSIM gaps of lower priority are not dropped (Nokia)

Recommendations:

Tentative agreement:

Focus on option 1 and option 2:

Option 1: Use priority information when UE requests MUSIM gaps to indicate when “keep solution” is used, details are FFS

Option 2: Use explicit signalling to indicate when “keep solution” is used, details are FFS

Other solutions are not precluded

Issue 2-2-2-2: When priority based solution is used

- Proposals

  - Option 1: Priority based solution is used when collided MUSIM gaps have different priority levels (Apple ZTE oppo vivo)

  - Option 2: Conditions when Priority based solution is used and conditions when Keep solution is used are FFS (Huawei)

  - Option 3: Priority based solution is used when “keep solution” is not used, when “keep solution” is used is up to issue 2-2-2-2. (Huawei)

  - Option 4: Priority based solution is used when collided MUSIM gaps have different priority levels and the UE does not request that both gaps are kept (Qualcomm)

Recommendations:

Tentative agreement:

Priority based solution is used when “keep” solution is not used

Issue 2-2-2-3: UE behaviro after a MUSIM gap is dropped by using priority based rule

- Proposals

  - P1: A UE can be scheduled during a MUSIM gap occasion if that MUSM gap is dropped. (Nokia)

Recommendations: Suggest to agree P1

Tentative agreement:

A UE can be scheduled during a MUSIM gap occasion if that MUSM gap occasion is dropped

Sub-topic 2-3 On collision between MUSIM and legacy gaps

Issue 2-3-2: Solutions for collision between MUSIM gap and Type-1 MG or any configured gap without priority

- Proposals

  - P1: When a MUSIM gap collides with a legacy MG, requirements shall not apply if any one of the collided gaps is not assigned a priority. (vivo Apple xiaomi oppo)

  - P2: MUSIM gaps are assumed to have higher priority than a Type-1 MG when either MUSIM gaps or Type-1 MG (or both) are not assigned priorities by the network. (Qualcomm)

  - P3: Collision is be handled based on the MGRP of the collided gaps (vivo ZTE Ericsson Huawei MTK)

    - P3-1: RAN4 to prioritize the gap with longer MGRP when: 1. Any of the collision gaps is Type-1 MG; 2. NW-A doesn’t configure a priority associated with any of the collision gaps. (vivo Ericsson Huawei MTK)

    - P3-2: No requirements apply if the two gaps have same MGRP. (vivo Huawei)

    - P3-3: If the MGRPs of the collided MUSIM gap and Type-1 MG are the same, then prioritize MUSIM gap only if it is configured with the highest priority level; otherwise prioritize Type-1 MG (MTK)

  - P4: Introduce priority for Type-1 MG when MUSIM gaps are configured when also having Type-1 measurement gaps allocated (Nokia)

Sub-topic 2-4 On collision between MUSIM gaps and NW A signals

Issue 2-4-3: Collision between SMTC and MUSIM gaps for handover and Scell activation

- Proposals

  - P1: Collisions between other RRM procedures and MUSIM gaps are handled in the same way as collisions between RRM procedures and legacy MG, i.e., no special handling solution is defined. (Apple oppo Huawei Nokia Qualcomm MTK vivo)

  - P2: RAN4 to add a high level clarification for the collision between MUSIM gaps with Handover, SCell activation and SI update (Ericsson):

    - When NW-A’s RS resources for one-shot RRM procedure(Handover, SCell activation, SI update) collide with MUSIM gaps, MUSIM gaps should have lower priority.

    - When NW-A’s uplink signals for one-shot RRM procedure (Handover, SCell activation) collide with MUSIM gaps, MUSIM gaps should have lower priority, such as NW-A’s PRACH and CSI-RS reporting for SCell activation should be prioritized.

  - P3: Add a high-level clarification in RAN4 spec that during one-shot procedure such as Scell activation, SI update and so on, UE is not expected to enable MUSIM gaps unless existing RRM requirement for the corresponding one-shot procedure can be met. (Apple)

Recommendations: The following tentative agreement needs to be checked

Tentative agreements

- When MUSIM gaps are configured, UE is still required to meet Scell activation RRM requirements for NW-A  (FFS on whether to capture this conclusion into spec)

    - No test case will be defined to verify this case

- FFS on handover

Topic #3: On network A requirements

Sub-topic 3-1 On network A requirements

Issue 3-1-1: Principle on layer 1 and layer 3 measurement requirements after gap collision handling

- Proposals

  - P1: Frameworks of LBT failure in NR-U design can be used as starting point when discussing NW A L3/L1 requirement impact due to MUSIM gaps (Apple)

  - P2: Reuse the same principle of Rel-17 concurrent gaps WI to define network A L1/L3 measurement requirements when MUSIM gaps are configured, i.e., introduce a scaling factor like Kx = Ntotal /Navailable for network A requirements when MUSIM gaps are configured. (vivo CMCC xiaomi ZTE Ericsson oppo Huawei Nokia Qualcomm MTK)

  - P3: RAN4 to postpone the detail NW-A’s requirement discussion until RAN4 achieves the consensus on MUSIM gaps’ priority. (Ericsson Nokia)

  - P4: RAN4 to further consider the L1/L3 measurement requirement when the configured MUSIM gaps have longer MGRP. (Ericsson)

Recommendations: Companies are encouraged to check P1 and P2.

Issue 3-1-2: On parameters for L1/L3 measurement requirements

- Proposals

  - P1: (CMCC):

    - For L3 measurement, Navailable need to be updated to cover MUSIM gaps

    - For L1 measurement, Noutside_MG and Navailable need to be updated to cover MUSIM gaps

  - P2: The following parameters need to be updated to account for collisions with MUSIM gaps (Qualcomm MTK):

    - Kp for intra-frequency and inter-frequency measurements without gaps (Xiaomi)

    - Kgap for intra-frequency and inter-frequency measurements with gaps (Xiaomi)

    - Kgap_EUTRA for inter-RAT measurements

    - Kp_CSI-RS for CSI-RS L3 measurements

    - Kp,PRS,I for NR positioning measurements

    - CSSFintra for intra-frequency measurements

    - CSSFinter for intra-frequency measurements

    - CSSFinterRAT for intra-RAT measurements

    - P scaling factor for L1-RSRP and L1-SINR measurements (xiaomi)

  - P3: Suggest the following update on parameters for L1/L3 measurement requirements (ZTE)

  - On L3 measurements: the scaling factors shall be re-defined for MUSIM gaps impact with reusing the R17 con-MG principle, as below:

    - Intra-frequency (without gap):  Kp=Ntotal/Navailable

- Ntotal is the total number of SMTC occasions within the window W, including those overlapped with MGs and MUSIM gaps within the window.

- Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion and  non-dropped MUSIM gap occasions within the window W.

- Kp = 1 when Navailable = 0.

- The duration of the window W equals max{SMTC period, MGRP_max, MUSIM gap period}

    - Inter-frequency: Kgap=Ntotal/Navailable

- Ntotal is the total number of SMTC occasions that are covered by instances of the associated MG within the window W, including those overlapped with other MGs and MUSIM gaps within the window.

- Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated MG within the window W.

- The duration of the window W equals max{SMTC period, MGRP_max, MUSIM gap period}.

  - On L1 measurements: the scaling factors shall be re-defined for MUSIM gaps impact with reusing the R17 con-MG principle, as below:

    - Ntotal / Noutside_MG in FR1; Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0; Ntotal / Navailable in FR2 with Navailable > 0

- Ntotal is the total number of SSB resource occasions within the window, including those overlapped with measurement gap occasions,MUSIM gaps or SMTC occasions within the window, and

- Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor MUSIM gap occasion within the window W

- Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion, MUSIM gap nor any SMTC occasion within the window W

- TL1  is periodicity of the target SSB.

- The duration of the window W equals max{SMTC period, MGRP_max, MUSIM gap period}.

  - P4: (Huawei)

    - For L3 measurement outside MG, Kp in the requirements is updated

      - Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion or non-dropped MUSIM gap occasion within the window W.

    - For L3 and positioning measurement with MG, existing requirements can be re-used.

    - For L1 measurement outside MG, Navailable, Noutside_MG in the requirements are updated

      - Noutside_MG is the number of SSB resource occasions that are not overlapped with any MG nor MUSIM gap within the window W

      - Navailable is the number of SSB resource occasions that are not overlapped with any MG, MUSIM gap nor any SMTC occasion within the window W

  - P5: For L3 measurement without gap, SMTC should not be fully overlapping with MUSIM gap (oppo)

  - P6: (Qualcomm)

For intra-frequency and inter-frequency measurements without gaps in network A, modify the scaling factor Kp as follows:

- The duration of the window W is max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the SSB frequency layer

- Ntotal is the total number of SMTC occasions within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG and MUSIM gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

For inter-frequency and inter-frequency measurements with gaps in network A, modify the scaling factor Kgap as follows:

- The duration of the window W is max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the SSB frequency layer

- Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Navailable is the number of SMTC occasions that are covered by non-dropped instances of the associated MG within the window W, after accounting for measurement gap and MUSIM gap collisions

For inter-RAT measurements with gaps in network A, modify the scaling factor Kgap_EUTRA as follows:

- The duration of the window W is MGRP_max, where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within FR1

- Ntotal is the total number of associated measurement gap occasions within the window W, including those overlapped with other measurement gap and MUSIM gap occasions

- Navailable is the number of non-dropped associated measurement gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

For CSI-RS L3 intra-frequency measurements without gaps modify the scaling factor Kp_CSI-RS as follows:

- The duration of the window W is max(CSI-RS period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the CSI-RS frequency layer

- Ntotal is the total number of CSI-RS occasions within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Navailable is the number of CSI-RS occasions that are not overlapped with any non-dropped MG and MUSIM gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

For CSI-RS L3 inter-frequency measurements with gaps modify the scaling factor Kp_CSI-RS as follows:

- The duration of the window W is max(CSI-RS period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the CSI-RS frequency layer

- Ntotal is the total number of CSI-RS occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Navailable is the number of CSI-RS occasions that are covered by non-dropped instances of the associated MG within the window W, after accounting for measurement gap and MUSIM gap collisions

For NR positioning measurements with gaps modify the scaling factor Kp,PRS,i as follows:

- The duration of the window W is max(TPRS,i,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the positioning frequency layer

- Ntotal is the total number of associated measurement gap occasions covering PRS occasions within the window W, including those overlapped with other measurement gap and MUSIM gap occasions

- Navailable is the number of non-dropped associated measurement gap occasions covering PRS occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

Clarify the definition of CSSFintra for intra-frequency measurements so that dropped measurement gap occasions due to collisions with MUSIM gaps are not counted.

Clarify the definition of CSSFinter for inter-frequency measurements so that dropped measurement gap occasions due to collisions with MUSIM gaps are not counted.

Clarify the definition of CSSFinterRAT for intra-RAT measurements so that dropped measurement gap occasions due to collisions with MUSIM gaps are not counted.

For L1-RSRP and L1-SINR measurements modify the scaling factor P as follows:

- The duration of the window W is max(TL1,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and MUSIM gaps, if any, and/or per-FR measurement gaps within the same FR as the serving cell

- Ntotal is the total number of SSB resource occasions within the window W, including those overlapped with measurement gap and MUSIM gap occasions

- Noutside_MG is the total number of SSB resource occasions that do not overlap with measurement gap occasions, MUSIM gap occasions nor SMTC occasions within the window W

- Navailable is the number of SSB resource occasions that are not overlapped with any non-dropped MG and MUSIM gap occasions within the window W, after accounting for measurement gap and MUSIM gap collisions

Tentative agreement: postpone the detailed discussion until issue 3-1-1 is solved.

Topic #4: On network B requirements

Sub-topic 4-1

Issue 4-1-1: Network B requirements conditions

- Proposals

  - P1: Update the agreement on NW B requirements to include inactive state as: Define NW B measurement/cell reselection requirements in IDLE/inactive mode only (vivo CMCC Ericsson Huawei Nokia)

    - P1-1: The inactive state requirement should be the same as NW B’s Idle state (Ericsson Nokia)

  - P2: Add the condition “MUSIM gaps will not collide with other MUSIM gaps” when defining NW B requirements. (vivo)

  - P3: Do not add a condition stating that NW-B requirements only apply if “MUSIM gaps will not collide with other MUSIM gaps (Nokia)

  - P4: Re-discuss the conditions for the RAN4#106 agreement once network B requirements are clearer. Agree to continue discussion other conditions during or once NW B requirements are agreed. (Nokia)

  - P5: Postpone the discussion of additional conditions for defining Network B requirements until there is agreement on the framework for defining the requirements (issue 4-1-2). (Qualcomm)

Tentative agreement: Agree P1 and P1-1

Issue 4-1-2: Network B requirements framework

- Proposals

  - P1: The measurement/cell reselection requirements in IDLE/inactive mode for NW B could reuse the existing idle/inactive requirements as the baseline (vivo CMCC Apple xiaomi ZTE Huawei Qualcomm Ericsson)

    - P2-1: With DRX cycle replaced by max(DRX cycle, MGRP_max), where MGRP_max is the maximum MGRP among all configured MUSIM gaps. (vivo Apple xiaomi Huawei Qualcomm)

      - P2-1a: The value set of max(DRX cycle, MGRP_max) is {0.32, 0.64, 1.28, 2.56 5.12} (vivo)

    - P2-2: DRX cycle is replaced by max(DRX cycle, MGRP); MGRP is the MGRP of the periodic MUSIM gap with highest priority level (CMCC)

    - P2-3: The NW-B’s requirement should decouple with MUSIM gaps requested by UE.  RAN4 to introduce a relaxed NW-B’s IDLE mode requirement as follow. (Ericsson)

Table 1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra for NW-B

| DRX cycle length [s] | Scaling Factor (N1) |  |  | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2-1 | FR2-2 |  |  |  |
| 0.32 | 1 | 8 | 12 | 36 x N1 x M2 x R1 | 4 x N1 x M2 x R1 | 16 x N1 x M2 x R1 |
| 0.64 |  | 5 | 8 | 28 x N1 x R1 | 2 x N1 x R1 | 8 x N1 x R1 |
| 1.28 |  | 4 | 6 | 25 x N1 x R1 | 1 x N1 x R1 | 5 x N1 x R1 |
| 2.56 |  | 3 | 5 | 23 x N1 x R1 | 1 x N1 x R1 | 3 x N1 x R1 |
| Note: R1 = 2. |  |  |  |  |  |  |

\

  - P3: P2-1 and P2-2 is not reasonable. The UE measurement requirements for measurement/cell reselection requirements in IDLE/inactive mode for NW B, need to be based on a reasonable balance of the allocated MUSIM gap. (Nokia)

Recommendations: Continue discussion

Issue 4-1-3: Requirement when MGRP = 5.12s

- Proposals

  - P1: For MUSIM gap with 5.12s MGPR, new requirement for 5.12s should be defined. (vivo CMCC Apple Huawei Qualcomm)

    - P1-1: The new requirements for 5.12s could reuse corresponding requirements when DRX = 2.56s. (vivo Huawei)

  - P2: Not to define NW B measurement/cell reselection requirements in IDLE mode when MUSIM gap’s MGRP=5.12s (MTK)

Recommendations:

Issue 4-1-4: Limitation on MGRP of MUSIM gap can be requested under particular NW B DRX cycle configuration

- Proposals

  - P1: When RAN4 reuses existing IDLE mode cell reselection requirements for NW-B, the UE shall request MUSIM gaps with MGRP larger than 160ms when NW-B configures DRX cycle larger than 640ms. (Ericsson)

  - P2: When DRX value is much larger than the MGRP value, consider the following two options (vivo):

    - Option 1: when the MGRP of all configured MUSIM gaps are less than 320ms, the requirement of DRX = 0.32s is reused for NW B requirement irrespective of the DRX value configured by NW B.

    - Option 2: No change, the requirements are still based on max(DRX cycle, MGRP_max)

Recommendations:

Issue 4-1-5: Solutions when different MGRP are used for measurement

- Proposals

  - P1: When multiple MUSIM gaps with different MGRP configured for measurement, requirements could be option 1 by using max(DRX cycle, MGRP_max)  (using inter-frequency detection requirements as an example) (vivo):

    - Option 1. The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS38.304 [1] within Kcarrier_total * Tdetect,NR_Inter where Tdetect,NR_Inter (Tmeasure,NR_Inter/ Tevaluate,NR_Inter) depends on max(DRX cycle, MGRP_max) and Kcarrier_total is the total number of inter frequency layers to be measured of NW B.

  - P2: Clarify the need for performing inter-frequency measurement in NW-B; Clarify the need to for RAN4 to define UE requirements for NW-B inter-frequency measurements. (Nokia)

  - P3: RAN4 needs to study how the MGRP for a frequency layer is determined when the UE performs measurements on multiple frequency layers in NW B using single or multiple MUSIM gaps (MTK)

Recommendations:

Issue 4-1-6: Others NW B requirements

- Proposals

  - P1: NW B requirements for measurements of intra-frequency/ inter-frequency NR cells and inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion are not considered in Rel-18. (vivo)

Recommendations:

Tentative agreement:

P1

Issue 4-1-7: Network B requirements test case

- Proposals

  - P1: Do not define test cases to verify any new requirements in network B. (vivo Apple Qualcomm MTK)

  - P2: Do not exclude defining test cases to verify any new requirements in network B (Nokia)

Recommendations: