3GPP TSG-RAN WG4 Meeting # 107												             R4-2310085

Incheon, KR, May 22nd – May 26th , 2023

Agenda item:			8.34.4

Source:	Qualcomm Incorporated

Title:	WF on NR mobile IAB RRM requirements

Document for:	Approval

# Way Forward

Intra-frequency HO requirements

Reuse the UE intra-frequency HO requirements from TS 38.133 clauses 6.1.1.2 and 6.1.1.4 as baseline.

RRC Re-establishment

Re-use the UE RRC re-establishment requirements in TS 38.133 clause 6.2.1 for the mobile IAB as baseline.

Inter-frequency RRC re-establishment/RRC release with re-direction

Do not introduce requirements for inter-frequency RRC re-establishment and for RRC release with re-direction

Active TCI state switching

Reuse UE requirements for the active TCI state switching delay from TS 38.133 Clause 8.10 as baseline.

RLM Requirements

UE RLM requirements from TS 38.133 Clause 8.1 shall be taken as baseline and adapted as follows:

    - Discard requirements related to DRX: requirements depending on DRX cycle length, requirement at transitions

    - Discard requirements related to measurement gaps and collisions between RLM-RS and measurement gaps

    - Discard scheduling availability requirements related to FR1-FR2 inter-band CA and NR-DC (clause 8.1.7.4)

    - Discard Requirements when highSpeedMeasFlagFR2-r17 is configured

Link recovery procedure

UE link recovery procedures requirements from TS 38.133 Clause 8.5 shall be taken as baseline and adapted as follows:

    - Discard requirements related to DRX

    - Discard requirements related to measurement gaps

    - Discard requirements related to deactivated PSCells/SCells, inter-band CA, FR2-2, etc

    - Discard requirements when highSpeedMeasFlagFR2-r17 is configured

L1-RSRP measurement requirements

UE requirements defined in TS 38.133 Rel-15 Clause 9.5 shall be taken as baseline and adapted as follows:

    - Keep the beam sweeping factors for FR2

    - Discard requirements related to DRX

    - Discard requirements related to measurement gaps(gap sharing) and inter-frequency/inter-RAT measurements

Intra-frequency measurement requirements

UE intra-frequency measurement requirements defined in TS 38.133 Clause 9.2 should be taken as baseline for the L3 measurement requirements with the following amendments:

    - Keep the beam sweeping factors

    - Discard the DRX requirements

    - Discard the requirements for SFTD and measurements with gaps.

Handling of measurement gaps

Do not mention measurement gaps impacts in the specification

Handling of HST

Do not introduce HST requirements unless the WID is updated to include this scenario

CA Related Requirements

Do not introduce CA requirements

Timing related

Reuse the transmit timing and timing advance requirements from 38.174

Random access requirements

Re-use requirements from TS 38.174

CR split

| Requirements covered by CR | Company |
| --- | --- |
| Intra-frequency HO and RRC re-establishment | Ericsson |
| RLM and Link recovery procedures | Qualcomm |
| Active TCI state switching | Huawei |
| Measurements for mobile IAB -L3 and LI | Qualcomm |

Companies are invited to bring draft CRs in RAN4#108