3GPP TSG-RAN WG4 Meeting #107	R4- 2310242

Incheon, KR, May 22 – May 26, 2023

Source: 	Huawei, HiSilicon, China Unicom, Apple

Title:	TP for TR 38.896 on the A-MPR requirements for band n8

Agenda Item:	7.22.2

Document for:	Approval

# Background

According to the WID on R18 HPUE FDD [1], PC2 A-MPR for band n8 has been studied in [2][3][4]. Based on these contributions, the PC2 A-MPR requirements for NS_43 are proposed. Together with the simulation results, it’s proposed to capture the contributions into the TR 38.896 as shown in the text below.

# References

- RP-230352 Revised WID High power UE (power class 2) for NR FR1 FDD single band, China Unicom, RAN#99

- R4-2300716 PC2 A-MPR for band n8, Huawei, HiSilicon, RAN4#106

- R4-2304518 PC2 A-MPR for band n8 NS_43, Huawei, HiSilicon, RAN4#106bis-e

- R4-2309059 On PC2 A-MPR Proposal for NS_43, Apple, RAN4#107

# Text Proposal

##### ---Start of changes---

## 5.1	NR band n8

### 5.1.1	UE maximum output power

Table 5.1.1-1: UE output power for PC2

| NR band | Class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n8 | 26 | +2/-33 |
| NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. |  |  |

### 5.1.2	A-MPR requirements

### 5.1.2.1 NS_43

### 5.1.2.1.1 A-MPR simulation results from Huawei

The simulation results are obtained for NS_43 for both PC2 1Tx and PC3 1Tx, which are shown in Table 5.1.2.1.1-1 below. The absolute PC2 A-MPR values are plotted in the 3rd column, while the differences between PC2 and PC3 are plotted in the 2nd column, for which 0dB is assigned if A-MPR ≤ PC2 MPR.

Table 5.1.2.1.1-1: A-MPR simulation results for NS_43 with 1Tx, SCS=15kHz

| BW | DIFF A-MPR (PC2-PC3) | PC2 A-MPR |
| --- | --- | --- |
| 15MHz |  |  |
| 10MHz |  |  |
| 5MHz |  |  |

Additional simulation results are obtained for NS_43 PC2 1Tx with BW=10MHz and SCS=30kHz, which are shown in Table 5.1.2.1.1-2 below. The figures in the 2nd column show the differences of the A-MPR simulation results between PC2 and PC3, and the figures in the 3rd column plot only the PC2 A-MPR, for which 0dB is assigned if A-MPR ≤ PC2 MPR.

Table 5.1.2.1.1-2: A-MPR simulation results for NS_43 with 1Tx, SCS=30kHz

| BW | DIFF A-MPR (PC2-PC3) | PC2 A-MPR |
| --- | --- | --- |
| 10MHz |  |  |

It can be seen from the results in Table 1 that some A-MPR is needed for PC2 NS_18 for BW=10MHz with SCS=30kHz, although the PC3 A-MPR in the current specification is only allowed for SCS=15kHz.

### 5.1.2.1.2 A-MPR simulation results from Apple

Simulations have been conducted for all channel bandwidth from 5MHz, 10MHz and 15MHz. The A-MPR regions proposed in [3] were used as starting point. Selected simulation results can be found in Table 5.1.2.1.2-1 where allocations for DFT-s-OFDM and CP-OFDM for QPSK are provided.

Table 5.1.2.1.2-1: A-MPR simulation results for NS_43

### 5.1.2.1.3 A-MPR requirements

Based on the simulation results from previous sections, the PC2 A-MPR requirements for NS_43 are proposed below.

The existing A-MPR regions for PC3 as defined in TS38.101-1 are adapted for PC2, which are shown as the table below. The new table is applicable for both power classes without impact to the PC3 requirements.

Table 6.2.3.6-1: A-MPR regions for NS_43

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 5 MHz | 902.5 ≤ FC < 912.5 |  | > 15 | A1 |  |  |  |
| 10 MHz | FC = 910 |  | > 7.2 MHz/12/SCS | A2 |  | > 5.4 MHz/12/SCS | A4 |
|  |  |  | > 8.1 MHz/12/SCS | A3 |  | > 7.2 MHz/12/SCS | A5 |
| 15 MHz | FC = 907.5 | ≤ 1.8 MHz /12/SCS | > 0 | A6 | > 1.8 MHz/12/SCS < 6.12 MHz/12/SCS | ≥ 7.2 MHz/12/SCS | A6 |
|  |  | ≥ 12.24 MHz/12/SCS | > 0 | A6 | > 1.8 MHz/12/SCS < 6.12 MHz/12/SCS | < 7.2 MHz/12/SCS | A7 |
|  |  |  |  |  | ≥ 6.12 MHz/12/SCS ≤ 7.2 MHz/12/SCS | > 5.4 MHz/12/SCS | A7 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.6-2. NOTE 2: Only 15 kHz SCS is applicable for power class 3; both 15kHz and 30kHz SCS are applicable for power class 2. NOTE 3: Void |  |  |  |  |  |  |  |

The A-MPR values for PC2 1Tx are proposed in the tables below.

Table 6.2.3.6-2a: A-MPR for NS_43 (Power Class 2, 1Tx)

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) |  | A3 (dB) |  | A4 (dB) |  | A5 (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK | [1] | N/A | ≤ [1.5] | N/A |  |  |  | N/A |  | N/A |
|  | QPSK | ≤ [3.5] |  |  |  |  |  | ≤ [3.5] |  | ≤ [5.5] |  |
|  | 16 QAM | ≤ [3.5] |  |  |  |  |  | ≤ [3.5] |  | ≤ [5.5] |  |
|  | 64 QAM | ≤ [3.5] |  | ≤ [4.5] |  | ≤ [5] |  |  |  |  |  |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  |
| CP-OFDM | QPSK | ≤ [4] |  |  |  |  |  | ≤ [4] |  | ≤ [6] |  |
|  | 16 QAM | ≤ [4] |  |  |  |  |  | ≤ [4.5] |  | ≤ [6] |  |
|  | 64 QAM | ≤ [4] |  | ≤ [5] |  | ≤ [6.5] |  |  |  |  |  |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.6-2b: A-MPR for NS_43 (Power Class 2, 1Tx) Cont.

| Modulation/Waveform |  | A6 | A7 |  |
| --- | --- | --- | --- | --- |
|  |  | Outer / Inner (dB) | Outer (dB) | Inner (dB) |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [12] | ≤ [4] | [3] |
|  | QPSK | ≤ [12] | ≤ [4] | [3] |
|  | 16 QAM | ≤ [12] | ≤ [5] | [4] |
|  | 64 QAM | ≤ [12] | ≤ [5.5] | [5.5] |
|  | 256 QAM | ≤ [12] | ≤ [6.5] | [5.5] |
| CP-OFDM | QPSK | ≤ [12] | ≤ [6] | [4.5] |
|  | 16 QAM | ≤ [12] | ≤ [6] | [5] |
|  | 64 QAM | ≤ [12] | ≤ [6.5] | [6] |
|  | 256 QAM | ≤ [12] | ≤ [7] | [7] |

### 5.1.2.2 NS_43U

FFS.

##### ---End of changes---