3GPP TSG-RAN WG4 Meeting # 107												R4-2310310

Incheon, KR, May 22nd – May 26th , 2023

Agenda item:			8.31, 8.31.1, 8.31.2.1

Source:	Moderator (OPPO)

Title:	Topic summary for [107][147] NR_SL_enh2_UERF_part1

Document for:	Information

Introduction

This WF captures the agreement for the online session. Furthermore there are one LS and 4 TPs to be agreed within this email thread and further agreements are captured.

Agreed LS: R4-2310306

TPs:

R4-2310307	TP on system parameters for SL unlicensed operation for single CC

R4-2310378	TP for TR 38.786 on the MPR/A-MPR simulation assumptions

R4-2310308	on the SL-e TX requirement

R4-2310309	on the SL-e RX requirement

Topic #1: Reply LS on non-congituous RB sets transmission of PSFCH and S-SSB

The formal LS R4-2310306 has been agreed and sent out to RAN1.

Topic #2: System parameters for SL-U single CC

Open issues summary

Sub-topic 2-1 Spec change

Issue 2-1-1: Which clause is used to define SL-U requirements

- Proposals

  - Option 1: Suffix “J” “Sidelink on shared spectrum” is used for SL-U. (OPPO, VIVO)

  - Option 2: The SL-U RF requirements can be captured in suffix E in TS38.101-1 with the dedicated sub-suffix for each supporting features and prefer to use ‘Sidelink’ acronym instead of  ‘V2X’ acronym in TS38.101-1 and TS38.101-3. (Facebook, Nokia, LGE)

- Recommended WF

  - As per moderator understanding, option 2 works for single carrier SL-U. However, consider more features as SL-U CA, UL MIMO and TxD, option 1 might be more straight forward. Companies can propose comments on the additional features.

Agreement:

- Agree on Option 2.

  - If there is serious problem, go back to option 1.

  - RAN4 won’t change the existing suffix E name from V2X to sidelink.

Sub-topic 2-2 System Parameter

Issue 2-2-1: Channel spacing

- Proposals

  - Option 1: For SL-U bands n46, n96, n102, the nominal channel spacing is defined as follows: (VIVO)

    - For NR operating bands with 15 kHz channel raster,

      - Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2+{-5 kHz, 0 kHz, 5 kHz} for ∆FRaster equals 15 kHz

  - Option 2: To follow NR-U principle for channel spacing. (LGE, OPPO)

- Recommended WF

  - TBA

Agreement:

- Agree on Option 2.

Issue 2-2-2: 7.5kHz shift

- Proposals

  - Option 1: Reuse the channel raster definitions for bands n46, n96 and n102 as specified in clause 5.4.2.1 NR-ARFCN and channel raster for SL-U operation. (Facebook, VIVO, LGE, OPPO)

- Recommended WF

  - Agree on Option 1.

Agreement:

- No 7.5kHz shift is defined for bands n46, n96 and n102

Topic #3: Tx requirements for SL-U single CC

Open issues summary

Sub-topic 3-1 TX requirement

A revised TP R4-2310309 is used to captured the corresponding agreed requirement.

Sub-topic 3-2 MPR/A-MPR simulation

A revised TP R4-2310378 is used to captured the corresponding agreed requirement.

Topic #4: Rx requirements for SL-U single CC

Issue 4-1-1: RB configuration for REFSENS

- Proposals

  - Option 1:

- Table 3-1: Proposed RB configuration for REFSENS requirements for SL-U UE (Facebook)

|  |  | NR band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR SL-U Band | SCS kHz | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz | Duplex Mode |
| n46 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 100 | 132 |  |
| n96 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 100 | 132 |  |
| n102 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 100 | 132 |  |

  - Option 2:  (OPPO. LGE)

|  |  | NR band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR SL-U Band | SCS kHz | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz | Duplex Mode |
| n46 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 100 | 135 |  |
| n96 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 100 | 135 |  |
| n102 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 100 | 135 |  |

  - Option 3: To use the number of maximum transmission bandwidth configuration as defined in Table 5.3.2-1 for REFSENS calculation. (VIVO)

- Recommended WF

  - Option 1 and option 2 only has difference for 100MHz 60kHz RB numbers and can be check if alignment can be achieved. Further discuss option 3.

Issue 4-1-1: REFSENS requirement

- Proposals

  - Option 1: Based on RB configuration of issue 4-1-1 option 1. With the Sidelink RB allocation.

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm) | Duplex Mode |
| n46 | 15 | 20, 40 | -89.2 + 10log10(NRB/106) | TDD |
|  | 30 | 20, 40, 60, 80, 100 | -89.4 + 10log10(NRB/51) |  |
|  | 60 | 60, 80, 100 | -89.6 + 10log10(NRB/24) |  |
| n96, n102 | 15 | 20, 40 | -88.7 + 10log10(NRB/106) | TDD |
|  | 30 | 20, 40, 60, 80, 100 | -88.9 + 10log10(NRB/51) |  |
|  | 60 | 60, 80, 100 | -89.1 + 10log10(NRB/24) |  |
| NOTE 1: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. |  |  |  |  |

  - Option 2:

    - To use below equation for REFSENS calculation for SL-U band n46:

    - REFSENS=-174 dBm/Hz + 10*log10(Maximum Transmission Bandwidth Config.) + 13 dB NF and FE loss + 2.5 dB implementation margin -0.5 dB demod SNR – 3 dB diversity gain

    - Consider 0.5dB REFSENS relaxation for band n96 and n102 compared with the values for band n46 in SL-U.

| SL-U Operating Band | SCS kHz | 20 MHz (dBm) | 40 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| n46 | 15 | -89.2 | -86.1 |  |  |  |
|  | 30 | -89.4 | -86.2 | -84.3 | -83.1 | -82.1 |
|  | 60 | -89.6 | -86.4 | -84.5 | -83.1 | -82.1 |
| n96, n102 | 15 | -88.7 | -85.6 |  |  |  |
|  | 30 | -88.9 | -85.7 | -83.8 | -82.6 | -81.6 |
|  | 60 | -89.1 | -85.9 | -84.0 | -82.6 | -81.6 |

  - Option 3:

| NR SL-U Band | SCS kHz | Channel bandwidth / PREFSENS_V2X (dBm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz | Duplex Mode |
| n46 | 15 | -89.2 | -86.1 |  |  |  | HD |
|  | 30 | -89.4 | -86.2 | -84.4 | -83.1 | -82.1 |  |
|  | 60 | -89.6 | -86.4 | -84.6 | -83.4 | -82.1 |  |
| n96 | 15 | -89.2 | -86.1 |  |  |  | HD |
|  | 30 | -89.4 | -86.2 | -84.4 | -83.1 | -82.1 |  |
|  | 60 | -89.6 | -86.4 | -84.6 | -83.4 | -82.1 |  |
| n102 | 15 | -89.2 | -86.1 |  |  |  | HD |
|  | 30 | -89.4 | -86.2 | -84.4 | -83.1 | -82.1 |  |
|  | 60 | -89.6 | -86.4 | -84.6 | -83.4 | -82.1 |  |

  - Option 4: Sidelink RMCs for n46/96/102 are still under discussion.

- Recommended WF

  - TBA

Agreement:

- Use Option 2 as baseline

  - FFS on RB numbers for 100MHz with 60kHz SCS

Issue 4-1-1: REFSENS requirement

Agreement:

- Use the following table as baseline to derive REFSENS requirement

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm) | Duplex Mode |
| n46 | 15 | 20, 40 | -89.2 + 10log10(NRB/106) | [HD] |
|  | 30 | 20, 40, 60, 80, 100 | -89.4 + 10log10(NRB/51) |  |
|  | 60 | 60, 80, 100 | -89.6 + 10log10(NRB/24) |  |
| n96, n102 | 15 | 20, 40 | -88.7 + 10log10(NRB/106) | [HD] |
|  | 30 | 20, 40, 60, 80, 100 | -88.9 + 10log10(NRB/51) |  |
|  | 60 | 60, 80, 100 | -89.1 + 10log10(NRB/24) |  |

Sub-topic 4-2 Other RX requirements

A revised TP R4-2310309 is used to captured the corresponding agreed requirement.