3GPP TSG-RAN WG4 Meeting # 107												R4-2310398

Incheon, KR, May 22nd – May 26th , 2023

Title: 	WF on UE RF requirements for NR_HST_FR2_enh_UERF

Agenda Item:	8.13.6

Source: 	Samsung

Document for:	Approval

Topic #1: RF requirements for intra-band CA

- Agreement

  - R4-2307429 is endorsed.

Topic #2: RF requirement for simultaneous multi-panel operation

Issue 2-1: DL power level for 2AoA spherical coverage

- Proposals

  - Option 1: legacy requirement + X, X=0.0dB (Samsung, Nokia)

  - Option 2: legacy requirement + X, X=0.5dB (Huawei)

- Agreement

  - Align the simulation assumptions and RAN4 is targeted to agree on X value next meeting based on the simulation and analysis.

Issue 2-2: AoA area for 2AoA spherical coverage

- Agreement

  - All possible angular separations are applicable for core requirements as long as the two test points are from Area-1 and Area-2 respectively; 150° angular separation in theta of UE coordination is used for test verification.

Issue 2-3: impacted clause in TS 38.101-2

- Agreement

  - new clause with dedicated suffix 7.3J

- Table 4.3-1: Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| None | Single Carrier |
| A | Carrier Aggregation (CA) |
| B | Dual-Connectivity (DC) |
| C | Supplement Uplink (SUL) |
| D | UL MIMO |
| J | Multiple [AoA] |
| NOTE: Suffix D in this specification represents either polarized UL MIMO or spatial UL MIMO. RF requirements are same. If UE supports both kinds of UL MIMO, then RF requirements only need to be verified under either polarized or spatial UL MIMO. |  |

Topic #3: Simulation assumption

Simulation assumption for X value derivation in Issue 2-1 is agreed as following:

- antenna array

  - [Mg, Ng, M, N, P] = [1, 1, 4, 4, 2]

- antenna element pattern

  - Table 6.1.4.1 of TR38.854:

| Parameter | Values |
| --- | --- |
| Antenna element radiation pattern in θ’’ dim (dB) |  |
| Antenna element radiation pattern in ϕ’’ dim (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5dBi |

- beam assumption

  - highSpeedMeasFlagFR2-r17 = set2

- AoA pair

  - All possible AoA pairs as long as AoA1 and AoA2 are from Area-1 and Area-2 respectively

- DCI scheme

  - mDCI

- Interference

  - No polarization isolation between the two DL signals from the 2 TRPs

  - No “joint detect/decode” is considered, i.e., only consider two TRP-RX pairs: TRP1-RX1 and TRP2-RX2.

  - The signal from TRP2 to RX1 and TRP1 to RX2 is treated as interference.

- pass/fail criterion for each beam pair

  - min[SINR_AoA1, SINR_AoA2] >= -1dB

- other aspects for reference

  - agreements of FR2 HST RF WI which are applicable for this simulation

  - agreements of FR2 Multi-RX DL WI which are applicable for this simulation