3GPP TSG-RAN WG4 Meeting#107                                   R4-2310405

Incheon , KR, May 22 – May 26,2023

Source:	ZTE Corporation

Title:	TP for TS 38.876: Clause 7.1 and 7.1.1

Agenda Item:	8.14.2.1

Document for:	Approval

- Introduction

This contribution provides a text proposal to TS38.876 [1] on sub-clause 7.1 and 7.1.1.

- Reference

- R4-230xxxx, TS 38.876 Study on Air-to-ground network for NR, CMCC, approved.

Text Proposal

----- Start of TP -----

7	RF requirements

7.1	ATG CPE specific

7.1.1 ATG CPE power class and requirement type

Since the required power level for ATG might be varying in different aircraft types and also in different frequency ranges, it’s quite difficult to focus on one specific power class for ATG UE from 3GPP perspective, therefore 3GPP agreed to introduce the new capability for ATG UE to indicate the rated maximum output power at maximum modulation order and full PRB configurations and its capability report granularity as 1dB. The range of its power limit are defined as following:

- The lower limit of conductive MOP or TRP of ATG UE is 23 dBm

- The upper limit of conductive MOP or TRP of ATG UE is 40 dBm

In addition, considering the implementation freedom for ATG UE(e.g. for 2 GHz, to use omni antenna for ATG UE and for 4 GHz, to use antenna array for ATG UE), therefore 3GPP RAN4 agreed to introduce two ATG UE  types to distinguish the antenna types for ATG UE.

----- End of TP -----