3GPP TSG-RAN WG4 Meeting #107	R4-2310408

Incheon, Republic of Korea, May 22-26, 2023

Title: 	WF on system parameters for less than 5MHz channel bandwidth

Agenda Item:	8.15.6

Source: 	Nokia

Document for:	Approval

Issue 1-1: The value of A in N * 600kHz + M * 50 kHz + A kHz, N ϵ {1:2499}, M ϵ {1,3,5}

- Proposals

  - Option 1: 300

  - Option 2: 345

- Agreement

  - Agree on 300.

Issue 1-2: The value of B in N * 100 kHz + B kHz, N ϵ {9206:1:9232}

- Proposals

  - Option 1: 30

  - Option 2: 70

- Agreement

  - Do not consider 100kHz either Option 1 or Option 2.

Issue 1-3: Finer synchronization raster design for 3 MHz channel bandwidth

- Proposals

  - Option 1: Use N * 600 kHz + M * 50 kHz + A kHz, N ϵ {1:4998}, M ϵ {1,3,5} for 12 PRBs PBCH transmission bandwidth, use N * 100 kHz + B kHz, N ϵ {9206:1:9232} for 15 PRBs PBCH transmission bandwidth

  - Option 2: Use N * 600 kHz + M * 50 kHz + A kHz

- Agreement

  - Do not consider Option 1

Issue 1-4: Additional synchronization raster for 12 PRBs PBCH transmission bandwidth in band n100

- Proposals

  - Option 1: 920.6225 MHz

  - Option 2: 920.73 MHz

- Agreement

  - Agree on Option 2.

Issue 1-5: Additional synchronization raster for 20 PRBs PBCH transmission bandwidth in band n100

- Proposals

  - Option 1: 921.45 MHz

- Agreement

  - Additional synchronization raster of 921.45 MHz for 20 PRBs PBCH transmission bandwidth in band n100.

  - The additional sync raster is applicable for 20 PRBs PBCH and not applicable for PBCH with 12 or 15 PRBs in band n100.

Issue 1-6: Finer synchronization raster index for 3 MHz channel bandwidth

- Proposals

  - Option 1: GSCN = 22255 + 3*N + (M-1)/2, N=1:2499, M = {1,3,5}

  - Option 2: NREF = (N * 600 + M * 50 + 345)/5, N=1534:1538, M ϵ {1,3,5}

- Agreement

  - TBD based on finer synchronization raster design.