3GPP TSG-RAN4 Meeting #107	R4-2310427

# Incheon, Korea, 22nd – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1626 | rev | 1 | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DMRS bundling requirements correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh-Core |  |  |  |  |  | Date: |  |  | 2023-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For DMRS bundling operation, from the latest LS from RAN5 (in R4-2304029), it appears that Carrier Leakage is removed as part of the process prior to measurement of phase tolerance. However, it is understood that the TE receiver may observe some distortion of the remaining DC subcarrier signal phase. This may lead to the observation that the DC subcarrier does not achieve the existing phase tolerance requirements across consecutive slots. Therefore, it is proposed to exclude the DC subcarrier from the phase offset measurement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified in clause F.9.4 that the phase offset measurement shall exclude the subcarrier transmitted at the DC location. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Through no fault of the UE, the UE may fail the DMRS bundling requirements due to the measurement method of the phase tolerance by the Test Equipment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<start of change>

## F.9.4	Phase offset measurement

The phase offset measurement is based on the phase response of the Tx chain  as derived based on Annex F.4. The subcarrier at the DC location of the transmitted signal shall be excluded from the measured subcarriers.

The phase difference ∆φf for each measured subcarrier between a reference timeslot tref and the measurement timeslot tm is then calculated as defined below:

∆φf= φtm,f-φ(tref,f)

The phase offset between the reference and measurement timeslots are then calculated as the maximum over the results for all measured subcarriers as shown below:

PhaseOffset=maxf(∆φf)

<end of change>