3GPP TSG-RAN WG4 Meeting #107	R4-2310484

Incheon, KR, May 22nd – May 26th, 2023

Title: 	WF on NR coverage enhancement part 1

Agenda Item:	8.28.3

Source: 	Huawei, HiSilicon

Document for:	Approval

<Topic #1> Whether to introduce P-MPR report in FR1

<Agreement>:

-	Not to introduce such report since this is closely related to SAR implementation, which is sensitive to UE design.

<Topic #2> Whether to enable UE report on the ΔPPowerClass to indicate which power class requirements that the UE is referring to only when configured duty cycle is exceed

<Agreement>:

-	Introduce such report the ΔPPowerClass report based on the duty cycle scheme:

- The occasion of the report should be limited to when configured duty cycle is exceeded.

- The full power transmission capability corresponding to the current reference power class should also be reported to guarantee more reasonable UL scheduling.

<Topic #3> Whether to introduce ‘duration’ information (e.g., a UE can sustain UL at a previously or concurrently reported PCmax) report for FR1

<Agreement>:

-	RAN4 stops discussion for this ‘duration’ information report in Rel-18, but this does not mean RAN1 discussion is precluded.

<Topic #4> Whether to introduce “PHR report regarding potential UL on DL-only carriers”

<Agreement>:

-	RAN4 stops discussion for this “PHR report regarding potential UL on DL-only carriers” based on the following reasons:

-	Whether SRS carrier switching with PHR type 3 or downlink measurement report can have the same effect as advertised by the above report or not is not justified.

-	The target scenario under specific UE architecture e.g., example band combination of intra/inter-band CA with specific antenna arrangement has not be provided along with the details of such UE reporting in order to prove the potential gain since it has been claimed that such demand is rooted from various RF exposure capability of antenna.

-	But this does not mean RAN1 discussion is precluded.