3GPP TSG-RAN WG4 Meeting # 107												R4-2309950

Incheon, KR, May 22nd – May 26th , 2023

Agenda item:			5.2.5.2, 5.2.5.3

Source:	Moderator (Ericsson)

Title:	Topic summary for [107][205] NR_redcap

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

# Topic #1: Core part

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2307923 | ZTE Corporation | Proposal 1: UE may assume that the TX power of the SSB of the target cell’s active BWP is same as that of the measured SSBs that belong to the same PCI. Proposal 2: RAN4 need to clarify the definition of intra-frequency and inter-frequency handover in the spec. Proposal 3: Handover for a RedCap UE is defined as intra-frequency handover if the center frequency and subcarrier spacing (SCS) of the reference SSB of the serving cell is same as the center frequency and SCS of the reference SSB of the target cell, where: - The reference SSB of the serving cell is the SSB configured in the BWP-specific servingCellMO, if configured, else the SSB configured in the servingCellMO - The reference SSB of the target cell is the SSB configured in the firstActiveBWP of the target cell Proposal 4: UE is not required to perform intra-frequency cell detection if the intra-frequency cell is already detected, where an intra-frequency cell is regarded as already detected if it has been meeting the following conditions: -The MO with the same SSB frequency has already be configured, and -During 5s before BWP switching, - the UE has sent a valid measurement report for the MO with the same frequency, and - the SSB measured remains detectable according to the cee identification conditions specified in clause 9.2 and 9.3 |
| R4-2308420 | Vivo | Proposal 1: Handover to a BWP with unmeasured SSB of a known cell, ok to add the conditions that “UE may assume that the TX power of the SSB of the target cell’s active BWP is same as that of the measured SSBs that belong to the same PCI”. Proposal 2: Option 1a could be used as the further clarification on handover requirement. |
| R4-2309667 | Nokia, Nokia Shanghai Bell | - Option 1 is supported related to open issue on handover to a BWP with unmeasured SSB of a known cell. - Option 1a is supported related to open issue related to revision of the intra/inter-frequency definition in handover. - Proposal 4: Further discussion may be needed on the open issue related to intra-frequency cell measurement. |
| R4-2309708 | Qualcomm Incorporated | Proposal 1: Handover for a RedCap UE is defined as intra-frequency handover if the center frequency and subcarrier spacing (SCS) of the reference SSB of the serving cell is same as the center frequency and SCS of the reference SSB of the target cell, where: - The reference SSB of the serving cell is the SSB configured in the DL BWP of the serving cell - The reference SSB of the target cell is the SSB configured in the DL firstActiveBWP of the target cell Proposal 2: RAN4 to clarify the definition of intra-frequency and inter-frequency cell for RRC re-establishment procedure for RedCap UEs in the spec. Proposal 3: A neighbor cell for RRC re-establishment procedures for a RedCap UE is defined as an intra-frequency cell if the center frequency (and subcarrier spacing (SCS)) of the reference SSB of the serving cell is same as the center frequency (and SCS) of the reference SSB of the neighbor cell; else it is considered as inter-frequency cell, where: - The reference SSB of the target cell is the CD-SSB of the target cell - The reference SSB of the serving cell is - Option 1: SSB configured in the BWP-specific servingCellMO, if configured, else the SSB configured in the servingCellMO - Option 2: CD-SSB of the serving cell - Option 3: SSB within the active BWP - This option is preferred as it’s consistent with the intra/inter-frequency HO definition |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1

Sub-topic description:

Open issues and candidate options before meeting:

Issue 1-1-1: Handover to a BWP with unmeasured SSB of a known cell

- Proposals

  - Option 1 (ZTE, vivo, Nokia, Qualcomm): UE may assume that the TX power of the SSB of the target cell’s active BW is same as that of the measured SSBs that belong to the same PCI

- Recommended WF

  - Discuss the option.

Issue 1-1-2: Revision of the intra/inter-frequency definition in handover

- Proposals

Handover for a RedCap UE is defined as intra-frequency handover if the center frequency and subcarrier spacing (SCS) of the reference SSB of the serving cell is same as the center frequency and SCS of the reference SSB of the target cell, where:

    - The reference SSB of the target cell is the SSB configured in the firstActiveBWP of the target cell

  - Option 1 (ZTE, vivo, Nokia):

    - The reference SSB of the serving cell is the SSB configured in the BWP-specific servingCellMO, if configured, else the SSB configured in the servingCellMO

  - Option 2 (QC): The reference SSB of the serving cell is the SSB configured in the DL BWP of the serving cell

  - Option 3 (Nokia): Further discussions needed.

- Recommended WF

  - CRs are already submitted to capture the revised definition. Review the CR and provide comments directly to the CR if any.

Issue 1-1-3: Intra-frequency cell measurement

- Proposals

  - Option 1 (ZTE): UE is not required to perform intra-frequency cell detection if the intra-frequency cell is already detected, where an intra-frequency cell is regarded as already detected if it has been meeting the following conditions:

-The MO with the same SSB frequency has already be configured, and

-During 5s before BWP switching,

- the UE has sent a valid measurement report for the MO with the same frequency, and

- the SSB measured remains detectable according to the cee identification conditions specified in clause 9.2 and 9.3

- Recommended WF

  - Discuss the option.

Issue 1-1-4: Intra-frequency cell definition for RRC re-establishment

- Proposals

  - Option 1 (QC): A neighbor cell for RRC re-establishment procedures for a RedCap UE is defined as an intra-frequency cell if the center frequency (and subcarrier spacing (SCS)) of the reference SSB of the serving cell is same as the center frequency (and SCS) of the reference SSB of the neighbor cell; else it is considered as inter-frequency cell, where:

- The reference SSB of the target cell is the CD-SSB of the target cell

- The reference SSB of the serving cell is

  - Option 1: SSB configured in the BWP-specific servingCellMO, if configured, else the SSB configured in the servingCellMO

  - Option 2: CD-SSB of the serving cell

  - Option 3: SSB within the active BWP

This option is preferred as it’s consistent with the intra/inter-frequency HO definition

- Recommended WF

  - Check whether the same agreement from intra-frequency cell definition of HO can be applied for the RRC re-establishment.

# Topic #2: Performance part

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2307924 | ZTE Corporation | Proposal 1: Remove the offset from all not-at-cell-edge thresholds to avoid the double offset. Proposal 2: The offset of +1 dB should be applied for rsrp-ThresholdMsg3. |
| R4-2309650 | Ericsson | - Proposal #1: Remove the offset from s-SearchDeltaP-r16 and s-SearchDeltaP-Stationary-r17. - Proposal #2: Following options are considered; our preference is option 1: - Option 1: - Remove the offset from all not-at-cell-edge thresholds (i.e. s-SearchThresholdP-r16, s-SearchThresholdQ-r16, s-SearchThresholdP2-r17 and s-SearchThresholdQ2-r17). - Keep the existing offset (-1 dB) to Qrxlevmin and Qqualmin - Option 2: - Keep the existing offset (+1 dB) to all not-at-cell-edge thresholds (i.e. s-SearchThresholdP-r16, s-SearchThresholdQ-r16, s-SearchThresholdP2-r17 and s-SearchThresholdQ2-r17). - Remove the offset (-1 dB) from Qrxlevmin and Qqualmin - Proposal #3: Add +1 dB of offset to ThresholdSSB-SUL and rsrp-ThresholdMsg3. |
| R4-2309670 | Nokia, Nokia Shanghai Bell | - Clarify in the Reply LS to RAN2, that the offset of +1 dB is appropriate for the thresholds s-SearchThresholdP-r16 and s-SearchThresholdP2-r17 but will be removed from the thresholds s-SearchThresholdQ-r16 and s-SearchThresholdQ2-r17. RAN4 not to consider an offset for rsrp-ThresholdSSB-SUL as this threshold is not applicable to RedCap UE’s. RAN4 to send this information to RAN2 in the Reply LS. RAN4 to agree the offset of + 1dB for rsrp-ThresholdMsg3 in case of 1 Rx RedCap UE. RAN4 to send this information to RAN2 in the Reply LS. RAN4 to agree on the draft Reply LS to RAN2 in the Annex. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1

Sub-topic description:

Open issues and candidate options before meeting:

Issue 2-1-1: Offset for cell selection and not-at-cell-edge criteria thresholds

Background: Not-at-cell-edge thresholds are s-SearchThresholdP-r16, s-SearchThresholdQ-r16, s-SearchThresholdP2-r17 and s-SearchThresholdQ2-r17.

- Proposals

  - Option 1(E///, ZTE):

    - Remove the offset from all not-at-cell-edge thresholds (i.e. s-SearchThresholdP-r16, s-SearchThresholdQ-r16, s-SearchThresholdP2-r17 and s-SearchThresholdQ2-r17).

    - Keep the existing offset (-1 dB) to Qrxlevmin and Qqualmin

  - Option 2(Nokia):

    - Keep the existing offset (+1 dB) to all not-at-cell-edge thresholds (i.e. s-SearchThresholdP-r16, , s-SearchThresholdP2-r17).

    - Remove the offsets from s-SearchThresholdQ-r16 and s-SearchThresholdQ2-r17.

- Recommended WF

  - Discuss the options.

Issue 2-1-2: Offset for missing thresholds: rsrp-ThresholdMsg3

- Proposals

  - Option 1(ZTE, Ericsson, Nokia): Add +1 dB of offset to rsrp-ThresholdMsg3.

- Recommended WF

  - Discuss if option 1 can be agreed based on RAN2 recommendation.

Issue 2-1-3: Offset for missing thresholds: rsrp-ThresholdSSB-SUL

- Proposals

  - Option 1(Ericsson): Add +1 dB of offset to rsrp-ThresholdSSB-SUL.

  - Option 2 (Nokia):  not to consider an offset for rsrp-ThresholdSSB-SUL as this threshold is not applicable to RedCap UE’s.

    - Option 2a (Nokia): RAN4 to inform RAN2 that RAN4 will not to consider an offset for rsrp-ThresholdSSB-SUL as this threshold is not applicable to RedCap UE’s

- Recommended WF

  - Discuss the options and need for reply LS depends on the outcome. If option 1 is agreed, no need to send LS.