3GPP TSG-RAN WG4 Meeting # 107												             R4-2309978

Incheon, KR, May 22nd – May 26th , 2023

Agenda item:			8.34.4

Source:	Moderator (Qualcomm)

Title:	Topic summary for [107][233] NR_mobile_IAB

Document for:	Information

# Introduction

This email discussion contains the summary of the proposals on the RRM requirements to be defined for mobile IAB. The discussion is a continuation from RAN4#106-bis-e based on the WF agreed in RP-2306368.

# Topic #1: Title

Definition of RRM requirements for mobile IAB is discussed in this section.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2308039 | Nokia, Nokia Shanghai Bell | Proposal 1. Requirements for transitions to be excluded from the mobile IAB RLM requirements. Proposal 2. Only non-DRX requirements for link recover procedures apply for mobile IAB. Proposal 3. Requirement for beam failure recovery in SCell to be excluded from the mobile IAB requirements. Proposal 4. RAN4 does not need to introduce HST requirements for mobile IAB. Proposal 5. No RAN4 requirements for connection release with re-direction are needed. Proposal 6. Limited set of UE RRM requirements relevant for IAB-MT can be the baseline where basic requirements for the mobility need to be supported, others are FFS. |
| R4-2308325 | Huawei, HiSilicon | Observation 1: There is no explicit description about HST scenario or speed assumption in the WID for mIAB. Observation 2: The discussion about HSDN in RAN2 is about reusing HSDN framework to let UE prioritize the mIAB cell when UE determines itself onboard a mIAB Cell, which is for UE instead of mIAB. And it is irrelevant to HST scenario. Proposal 1: Do not define HST specific requirements for mobile IAB MT. Proposal 2: Requirements for RRC Connection Release with Redirection in TS 38.174 clause 12.1.1.3 can be reused for mobile IAB. Proposal 3: Do not define CA related requirements for Rel-18 mobile IAB, which could be further studied in future release. Proposal 4: The requirements already defined in TS 38.174 in Rel-16/17 can be reused directly. Observation 3: Some requirements introduced Rel-16/17 features are not needed for mobile IAB. Proposal 5: For requirements absent in TS 38.174, take Rel-15 requirements in TS38.133 as baseline. Requirement; Baseline; Whether can be reused directly or modification is needed?. Intra-frequency handover; TS 38.133 6.1.1.2 and 6.1.1.4; Only intra-frequency HO. RRC Re-establishment; TS 38.174 12.1.1.1; Reused. RACH; TS 38.174 12.1.1.2; Reused. Connection with redirection; TS 38.174 12.1.1.3; Reused. Transmit timing; TS 38.174 12.2.1; Reused. Timing advance; TS 38.174 12.2.3; Reused. RLM; TS 38.174 12.3.1; Reused. Link recovery; TS 38.174 12.3.2; Reused. TCI switching; TS 38.133 8.10; Reused. Intra-frequency measurement; TS 38.133 9.2; Without measurement gap Without DRX Without deactivated SCell requirements CSSF needs updating. L1-RSRP; TS 38.133 9.5; Without measurement gap Without DRX. Proposal 6: Define RRM requirements for Rel-18 mobile IAB-MT based on above Table I, and other requirements are precluded in Rel-18. |
| R4-2309644 | Ericsson | General aspects of RRM requirements: - Observation #1: Several scenarios/features for which RRM requirements are defined in TS 38.133 are not applicable for the mIAB. - Proposal #1: A list of RRM requirements explicitly identified for mIAB-MT are defined for the mIAB-MT. RRC Connection Mobility Control: - Observation #2: The RRC connection release with redirection to target NR carrier is analoguous to an inter-frequency handover. - Observation #3: It has been agreed to define only intra-frequency handover requirements since intra-frequency mobility is the most important and basic scenario for the mIAB. - Proposal #2: Do not define RRC connection release with redirection requirements for mIAB-MT. RLM: - Observation #4: It was agreed that in general RRM requirements for the mIAB-MT will be defined in non-DRX and without considering any measurement relaxations for power savings. - Observation #5: The measurement gaps will not be used by the mIAB-MT for the RRM measurements. - Proposal #3: The RLM requirements except the following in clause 8.1 of TS 38.133 are defined for the mIAB-MT: - Proposal 3A: - When highSpeedMeasFlagFR2-r17 is configured - Proposal 3B: - Aspects related to measurement gaps - Proposal 3C: - Requirements for RLM on PSCell - Proposal 3D: - Requirements for RLM when the PSCell is deactivated - Proposal 3E: - UE transitions between DRX and no DRX or when DRX cycle periodicity changes Link recovery procedure (LRP): - Proposal #4: The LRP requirements except the following in clause 8.5 of TS 38.133 are defined for the mIAB-MT: - Proposal 4A: - When highSpeedMeasFlagFR2-r17 is configured - Proposal 4B: - Aspects related to measurement gaps - Proposal 4C: - Requirements for LRP on PSCell and SCell - Proposal 4D: - Requirements for LRP when the PSCell is deactivated - Proposal 4E: - UE transitions between DRX and no DRX or when DRX cycle periodicity changes CA related requirements: - Observation #6: Only intra-frequency handover/mobility requirements will be defined and does not impact any other serving cell as it involves changing of the PCell of the IAB-MT. - Proposal #5: No CA related requirement to support mobility of the mIAB-MT is required. HST scenario: - Observation #7: The mIAB WID does not explicitly state that the HST scenario is within the scope of the WI. - Observation #8: According to the WID the main scenario comprises of the mIAB nodes mounted on vehicles. - Observation #9: mIAB applicability under HST scenario will require upscoping of the WID and therefore plenary level agreements. - Observation #10: Deployment scenario for the IAB in HST scenario may not be the same as used for the UEs and would require considerable discussion mIAB to the HST. - Proposal #6: HST scenario is NOT within the scope of mIAB WI. - Proposal #7: Do not specify requirements for mIAB under HST scenario. |
| R4-2308788 | Qualcomm Incorporated | Proposal 1: Reuse the UE intra-frequency HO requirements from TS 38.133 clauses 6.1.1.2 and 6.1.1.4. Proposal 2: Re-use the UE RRC re-establishment requirements in TS 38.133 clause 6.2.1 for the mobile IAB. Proposal 3: Also introduce inter-frequency re-establishment requirement (re-use existing UE requirements) as fallback case for inter-frequency mobility. Proposal 4: The UE requirements for the active TCI state switching delay from TS 38.133 Clause 8.10 should be reused. Proposal 5: UE RLM requirements from TS 38.133 Clause 8.1 shall be adapted and reused as follows: - Discard requirements related to DRX: requirements depending on DRX cycle length, requirement at transitions - Discard requirements related to measurement gaps and collisions between RLM-RS and measurement gaps - Discard scheduling availability requirements related to FR1-FR2 inter-band CA and NR-DC (clause 8.1.7.4) Proposal 6: UE link recovery procedures requirements from TS 38.133 Clause 8.5 shall be reused and adapted as follows: - Discard requirements related to DRX - Discard requirements related to measurement gaps Discard requirements related to deactivated PSCells/SCells, inter-band CA, FR2-2, etc Proposal 7: UE L1-RSR measurement requirements shall be based on the UE requirements defined in TS 38.133 Clause 9.5 and adapted as follows: - Keep the beam sweeping factors - Discard requirements related to DRX - Discard requirements related to measurement gaps(gap sharing) and interf-frequency/inter-RAT measurements Proposal 8: UE intra-frequency measurement requirements defined in TS 38.133 Clause 9.2 should be reused the L3 measurement requirements with the following amendments: - Discard the DRX requirements - Discard the requirements for SFTD and measurements with gaps. For all the requirements in which the requirements with gaps are discarded, a simple sentence stating that delays will longer if measurement gaps are configured can be included in the specs for consistency. Proposal 9: UE requirements for 4 step rand access procedure defined in TS 38.133 Clause 6.2.2.2 should be reused for the mobile IAB. Proposal 10: Do not introduce requirements for the RRC release with redirection. Proposal 11: Introduce requirements for the direct SCell activation at SCell addition and direct SCell activation at handover based on the UE requirements defined in TS 38.133 Clause 8.3.4 and 8.3.5. Proposal 12: Introduce HST requirements as optional such that this use case is also enabled. If this proposal cannot be agreed, optional support for HST should be kept FFS pending plenary discussion on this use case. This can be discussed in RAN#100 and it would not have any impact on the WI schedule. |

## Open issues summary

The open issues are mainly about how to define requirements that were agreed to be introduced in the previous meetings.

### Sub-topic 1-1

HO requirements

It was already agreed that only intra-frequency HO will be defined

Issue 1-1: Intra-frequency HO requirements

- Proposals

  - Option 1: Reuse the UE intra-frequency HO requirements from TS 38.133 clauses 6.1.1.2 and 6.1.1.4.

  - Option 2: others

- Recommended WF

  - Option 1

### Sub-topic 1-2

RRC Re-establishment requirements

Issue 1-2: RRC Re-establishment

- Proposals

  - Option 1: Re-use the UE RRC re-establishment requirements in TS 38.133 clause 6.2.1 for the mobile IAB

  - Option 2: others

- Recommended WF

  - Option 1

### Sub-topic 1-3

Inter-frequency RRC re-establishment

RRC release with re-direction is very similar to a inter-frequency HO to a unknown cell

Issue 1-3: Inter-frequency RRC re-establishment

- Proposals

  - Option 1: Introduce inter-frequency RRC re-establishment requirements, do not introduce RRC release with re-direction requirements

  - Option 2: Introduce RRC release with re-direction requirements

  - Option 3: Do not introduce either requirement

- Recommended WF

  - To be discussed

### Sub-topic 1-4

Active TCI state switching

Issue 1-4: Active TCI state switching

- Proposals

  - Option 1: reuse UE requirements for the active TCI state switching delay from TS 38.133 Clause 8.10

  - Option 2: others

- Recommended WF

  - Option 1

### Sub-topic 1-5

RLM Requirements

Issue 1-5: RLM Requirements

- Proposals

  - Option 1: UE RLM requirements from TS 38.133 Clause 8.1 shall be adapted and reused as follows:

    - Discard requirements related to DRX: requirements depending on DRX cycle length, requirement at transitions

    - Discard requirements related to measurement gaps and collisions between RLM-RS and measurement gaps

    - Discard scheduling availability requirements related to FR1-FR2 inter-band CA and NR-DC (clause 8.1.7.4)

    - Discard Requirements when highSpeedMeasFlagFR2-r17 is configured

  - Option 2: TBA

- Recommended WF

  - Option 1

### Sub-topic 1-6

Link recovery procedure

Issue 1-6: Link recovery procedure

- Proposals

  - Option 1: UE link recovery procedures requirements from TS 38.133 Clause 8.5 shall be reused and adapted as follows:

    - Discard requirements related to DRX

    - Discard requirements related to measurement gaps

    - Discard requirements related to deactivated PSCells/SCells, inter-band CA, FR2-2, etc

    - Discard requirements when highSpeedMeasFlagFR2-r17 is configured

  - Option 2: TBA

- Recommended WF

  - Option 1

### Sub-topic 1-7

L1-RSRP Measurements

Issue 1-7: L1-RSRP measurement requirements

- Proposals

  - Option 1: UE L1-RSRP measurement requirements shall be based on the UE requirements defined in TS 38.133 Rel-15 Clause 9.5 and adapted as follows:

    - Keep the beam sweeping factors for FR2

    - Discard requirements related to DRX

    - Discard requirements related to measurement gaps(gap sharing) and inter-frequency/inter-RAT measurements

  - Option 2: TBA

- Recommended WF

  - Option 1

### Sub-topic 1-8

Intra-frequency measurements

Issue 1-8: Intra-frequency measurement requirements

- Proposals

  - Option 1: UE intra-frequency measurement requirements defined in TS 38.133 Clause 9.2 should be reused the L3 measurement requirements with the following amendments:

    - Keep the beam sweeping factors

    - Discard the DRX requirements

    - Discard the requirements for SFTD and measurements with gaps.

  - Option 2: TBA

- Recommended WF

  - Option 1

### Sub-topic 1-9

Handling of measurement gaps

Issue 1-9: Handling of measurement gaps

- Proposals

  - Option 1: Include a sentence stating that delays will longer if measurement gaps are configured for the requirements where measurement gaps were discarded because measurement gaps can be configured in practice

  - Option 2: Do not mention anything about measurement gaps

- Recommended WF

  - To be discussed

### Sub-topic 1-10

HST requirements

Issue 1-10: Handling of HST

- Proposals

  - Option 1: Do not introduce any HST requirements

  - Option 2: Introduce HST requirements as optional

  - Option 3: clarify the handling of HST in RAN#100

- Recommended WF

  - Option 3

### Sub-topic 1-11

CA Related Requirements

Issue 1-11: CA Related Requirements

- Proposals

  - Option 1: Introduce  requirements for the direct SCell activation at SCell addition and direct SCell activation at handover based on the UE requirements defined in TS 38.133 Clause 8.3.4 and 8.3.5.

  - Option 2: Do not introduce any CA requirements

- Recommended WF

  - To be discussed

### Sub-topic 1-12

Timing related requirements

Issue 1-12: Timing related

- Proposals

  - Option 1: Reuse the transmit timing and timing advance requirements from 38.174 (no need to do anything)

  - Option 2: TBA

- Recommended WF

  - Option 1

### Sub-topic 1-13

RACH requirements

Issue 1-13: Random access requirements

- Proposals

  - Option 1: Re-use requirements from TS 38.174 (no need to do anything)

  - Option 2: TBA

- Recommended WF

  - Option 1