3GPP TSG-RAN WG4 Meeting # 107												R4-2310443

Incheon, KR, May 22nd – May 26th, 2023

Agenda item:			8.15.6

Source:	Man Hung Ng (Nokia)

Title:	Topic summary for [107][304] NR_FR1_lessthan_5MHz_BW_BSRF

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

Summary for contributions submitted under agenda items 8.15.1 and 8.15.4 for NR support for dedicated spectrum less than 5MHz for FR1.

List of candidate target of discussion for 1st round and 2nd round:

- 1st round: Discussion and agreement on open issues listed below.

- 2nd round: Continue discussion and agreement on open issues listed below.

# Topic #1: BS RF requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2307235 | Nokia, Nokia Shanghai Bell | Proposal 1: To use the following formula for 3 MHz channel bandwidth for band n100, assuming a 17 dBi maximum antenna gain and 4 dB losses: 49.3 dBm/3MHz + (fDL-921.1) x 40/3 dB Proposal 2: To specify the additional narrow band blocking requirement for 3 MHz channel bandwidth for band n100 as follows: - Interfering signal mean power: -39 dBm - Wanted signal mean power (dBm): PREFSENS + 6 dB - Interfering Signal: 3 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB - Interfering RB centre frequency offset 874.4 MHz -(255 kHz +m*180), m=0, 1, 2, 3, 4, 7, 10, 13 Proposal 3: To specify the requirement also using the same 3MHz channel bandwidth for both wanted and interfering signals as follows: - Interfering signal mean power: -39 dBm - Wanted signal mean power (dBm): PREFSENS + 6 dB - Interfering Signal: 3 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB - Interfering centre frequency offset: 360 kHz for CW, 960 kHz for 1 RB Observation 1: PUSCH SNR [dB] Vs BLER@5% MCS; 16 (Receiver dynamic range); 4 (15PRB, Reference sensitivity); 4 (9PRB, In-channel selectivity). 2RX; 5.5534; -3.7722; -3.5778. 1RX; 8.4428; -1.2332; -1.0391. |
| R4-2307845 | ZTE Corporation | Proposal 1: Observation 1: the updated simulation results for ICS requirement is shown as following table: PRBs; SCS (kHz); SNR@95% throughput (dB). G-FR1-A1-20; 9; 15; -0.8. Observation 2: the updated simulation results for dynamic range requirement is shown as following table: CBW (MHz); PRBs; SCS (kHz); SNR@95% throughput (dB). G-FR1-A2-15; 3; 15; 15; 8.8. |
| R4-2308540 | Ericsson | Proposal 1: Reuse -0.9 dB agreed SNR value for G-FR1-A1-7. Proposal 2: Specify G-FR1-A1-20 and G-FR1-A2-15 as described in the way forward [4], removing []. Proposal 3: Use -0.9 dB and 8.6 dB SNR values for respectively FRCs G-FR1-A1-20 and G-FR1-A2-15. Observation 1: |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1

Sub-topic description:

Open issues and candidate options before meeting:

Issue 1-1: Conducted requirements

- Proposals

  - Proposal 1: To use the following formula for 3 MHz channel bandwidth for band n100, assuming a 17 dBi maximum antenna gain and 4 dB losses:

    - 49.3 dBm/3MHz + (fDL-921.1) x 40/3 dB

  - Proposal 2: To specify the additional narrow band blocking requirement for 3 MHz channel bandwidth for band n100 as follows:

    - Interfering signal mean power:		-39 dBm

    - Wanted signal mean power (dBm):		PREFSENS  + 6 dB

    - Interfering Signal:				3 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB

    - Interfering RB centre frequency offset 874.4 MHz -(255 kHz +m*180), m=0, 1, 2, 3, 4, 7, 10, 13

  - Proposal 3: To specify the requirement also using the same 3MHz channel bandwidth for both wanted and interfering signals as follows:

    - Interfering signal mean power:		-39 dBm

    - Wanted signal mean power (dBm):		PREFSENS  + 6 dB

    - Interfering Signal:				3 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB

    - Interfering centre frequency offset: 360 kHz for CW, 960 kHz for 1 RB

- Recommended WF

  - Pending the responses on R4-2305902 and R4-2305903, it is proposed to use the above for RAN4 discussion.

### Sub-topic 1-2

Sub-topic description

Open issues and candidate options before meeting:

Issue 1-2: Fixed Reference Channels

- Proposals

  - Proposal 1: Reuse -0.9 dB agreed SNR value for G-FR1-A1-7.

  - Proposal 2: Specify G-FR1-A1-20 and G-FR1-A2-15 as described in the way forward [4], removing [].

  - Proposal 3: Use -0.9 dB and 8.6 dB SNR values for respectively FRCs G-FR1-A1-20 and G-FR1-A2-15.

- Recommended WF

  - Discuss if the above proposals can be agreed.

# Topic #2: CR drafting

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2307234 | Nokia, Nokia Shanghai Bell | Proposal 1: Draft big CR to TS 38.104 Observation 1: |
| R4-2307236 | Nokia, Nokia Shanghai Bell | Proposal 1: Draft CR to TS 38.104 Observation 1: |
| R4-2307844 | ZTE Corporation | Proposal 1: Draft CR to TS 38.104 Observation 1: |
| R4-2308541 | Ericsson | Proposal 1: Draft CR to TS 38.104 Observation 1: |
| R4-2309640 | Huawei, HiSilicon | Proposal 1: Draft CR to TS 38.104 Observation 1: |
| R4-2309641 | Huawei, HiSilicon | Proposal 1: Draft CR to TS 38.104 Observation 1: |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1

Sub-topic description:

Open issues and candidate options before meeting:

Issue 2-1: R4-2307234

- Proposals

  - Option 1: Post meeting email approval

  - Option 2: TBA

- Recommended WF

  - Post meeting email approval

### Sub-topic 2-2

Sub-topic description

Open issues and candidate options before meeting:

Issue 2-2: R4-2307236

- Proposals

  - Option 1: Endorse the draft CR

  - Option 2: Revise the draft CR

  - Option 3: Postpone the draft CR

- Recommended WF

  - TBA

### Sub-topic 2-3

Sub-topic description

Open issues and candidate options before meeting:

Issue 2-3: R4-2307844

- Proposals

  - Option 1: Endorse the draft CR

  - Option 2: Revise the draft CR

  - Option 3: Postpone the draft CR

- Recommended WF

  - TBA

### Sub-topic 2-4

Sub-topic description

Open issues and candidate options before meeting:

Issue 2-4: R4-2308541

- Proposals

  - Option 1: Endorse the draft CR

  - Option 2: Revise the draft CR

  - Option 3: Postpone the draft CR

- Recommended WF

  - TBA

### Sub-topic 2-5

Sub-topic description

Open issues and candidate options before meeting:

Issue 2-5: R4-2309640

- Proposals

  - Option 1: Endorse the draft CR

  - Option 2: Revise the draft CR

  - Option 3: Postpone the draft CR

- Recommended WF

  - TBA

### Sub-topic 2-6

Sub-topic description

Open issues and candidate options before meeting:

Issue 2-6: R4-2309641

- Proposals

  - Option 1: Endorse the draft CR

  - Option 2: Revise the draft CR

  - Option 3: Postpone the draft CR

- Recommended WF

  - TBA

…