3GPP TSG-RAN WG4 Meeting #108	R4-2313921

Toulouse, France, August 21 – August 25, 2023

Title: 	Way Forward for 256 QAM BS Demod

Agenda Item:	8.6.4.1

Source: 	Nokia, Nokia Shanghai Bell

Document for:	Approval

# 1 General

### Sub-topic 1-1: PUSCH Requirements

Issue 1-1: PUSCH Requirements

Agreement: The demodulation performance requirements for UL 256QAM will be defined only for PUSCH

### Sub-topic 1-2: Bands for FR2 256 QAM

Issue 1-2: Bands for FR2 256 QAM

Agreement: Interested Companies to use 30 GHz for simulation frequency for FR2-1

Agreement: Introduce only one FR2-1 PUSCH with 256QAM demodulation requirements up to 39 GHz (aligned with RF).

Issue 1-3: BS Declaration

Agreement: Align with FR1: The PUSCH requirement with FR2-1 UL 256QAM is only applied for BS declared to support it. Introduce the BS declaration for FR2-1 UL 256QAM

# 2 PUSCH requirements

### Sub-topic 2-1: EVM Impact

Issue 2-1: EVM Impact

Agreement: The impact of EVM can be considered in impairment results, and interested companies are encouraged to provide ideal simulations with and without TxEVM for alignment at RAN4#108bis, TxEVM value to be 3.5%.

Note: Test requirement parameters do not capture TxEVM and TEs don’t add additional TxEVM during test.

### Sub-topic 2-2: SNR Limit

Issue 2-2: SNR Limit

RAN4 will attempt to keep simulation parameters to reach an SNR operating point less than 20dB, but will await simulation results from interested companies at RAN4#108-bis.

Request input from TE vendor on maximum testable SNR at RAN4#108bis

### Sub-topic 2-3: Phase Noise Model

Issue 2-3: Phase Noise Model

Interested companies are encouraged to bring simulations to RAN4#108-bis with and without phase noise model, and provide information of phase noise model, with PT-RS enabled and disable.

- Alignment to happen without phase model (therefore with priority)

- Companies encouraged to deliver simulation results for some cases of Phase Noise model, with PT-RS enabled and disabled.

### Sub-topic 2-4: Channel

Issue 2-4: Channel

The following candidate channel models can be considered for simulation at RAN4 #108 bis and down selected at later meetings for requirements.

- TDLA 30-35

- TDLD 30-35

### Sub-topic 2-5: Rank

Issue 2-5: Rank

Agreement: Rank 1

### Sub-topic 2-6: Carrier BW and SCS

Issue 2-6: Carrier BW for 60 kHz SCS and Issue 2-7: Carrier BW for 120 kHz SCS

Interested companies are encouraged to bring simulation on the following config: 60 kHz SCS/50 MHz, 120 kHz SCS/50 MHz, 100 and 200 MHz, discussion for further downselection based on simulation results at for RAN4#108-bis.

### Sub-topic 2-7: DMRS

Issue 2-8: Additional DMRS

Interested companies to provide simulation results with single (1+0) and additional (1+1) DMRS for RAN4#108-bis.

Issue 2-9: DMRS Mapping Type

Agreement: Mapping Type B

### Sub-topic 2-8: PTRS

Issue 2-10: PTRS

Interested companies to provide simulation results with both enabled and disable for RAN4#108-bis.

Configuration: K_PTRS: 2 and L_PTRS =1

### Sub-topic 2-9: Waveform Type

Issue 2.11: Waveform Type

Agreement: CP-OFDM

### Sub-topic 2-10: MCS

Issue 2-12: MCS

RAN4 considers the following candidate MCS before down selection for requirements definition at RAN4#108 bis (all in Table 2): MCS 20, 21 and 22 with a priority on MCS 20

### Sub-topic 2-11: Testing Metric

Issue 2-13: Testing Metric

Agreement: 70% of Max Throughput

### Sub-topic 2-12: Frequency Domain Resource Allocation

Issue 2-14: Frequency Domain Resource Allocation

Agreement: Full RB allocation of the applicable BW

### Sub-topic 2-13: Frequency Hopping

Issue 2-15: Frequency Hopping

Agreement: Frequency hopping disabled

### Sub-topic 2-14: HARQ Transmissions

Issue 2-16: Number of HARQ Transmissions

Agreement: 4

### Sub-topic 2-15: TPMI Index

Agreement: Not applicable, due to Rank 1

### Sub-topic 2-16: Code Block Group

Issue 2-18: Code Block Group

Agreement: disabled

### Sub-topic 2-17: Limited buffer rate matching

Issue 2-19: Limited buffer rate matching

Agreement: disabled

# 3 Work Plan

### Sub-topic 3-1: Work Plan

Issue 3-1: Work Plan

- Proposals

  - Option 1:

| RAN4#108: |  |
| --- | --- |
|  | Discussion and agreement on work plan. Discussion on performance requirements scope. Initial discussion on simulation assumptions |
| RAN4#108-bis |  |
|  | Discussions on performance requirements scope. Discussions on simulation assumptions. Discussions on work split. Initial round of simulation results collection and alignment. |
| RAN4#109 |  |
|  | Finalise discussions on performance requirements scope. Finalise discussions on simulation assumptions. Second round of simulation results collection and alignment. Initial draft CRs. |
| RAN4#110 |  |
|  | Final round of simulation results collection and alignment. CRs submitted. |

Agreement: Option 1

## A.4 Reference

[1] R4-2314257, Topic summary for [108][321] NR_RF_FR2_req_Ph3_Demod, Nokia, Nokia Shanghai Bell, RAN WG4 #108