# 3GPP TSG-RAN WG4 Meeting # 108                                                            R4-2314643

Toulouse, France, 21 - 25 Aug 2023                                         was R4-2313728

Source:	T-Mobile USA, Dish Network

Title:	TP for TR 38.896 to add PC2 for n66

Agenda item:			7.22.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 38.896 to add PC2 for n66 including RSD requirements based on the WF in [1].

This revision changes the text in 5.x.2 from “N/A” to “Re-use PC3 A-MPR.”

# 2. Reference

[1] R4-2306544	WF on FDD HPUE Basket WIs, China Unicom

# 3. Text Proposal

<Start of changes>

## 5.x	NR band n66

### 5.x.1	UE maximum output power

Table 5.x.1-1: UE output power for PC2

| NR band | Class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n66 | 26 | +2/-3 |

### 5.x.2	A-MPR requirements

Re-use PC3 A-MPR.

### 5.x.3	Reference sensitivity requirements

For PC2 single band n66, both 1Tx and 2Tx requirements are considered. Therefore, for the Reference Sensitivity Degradation requirements, not supporting Tx diversity (i.e. 1Tx) and supporting Tx diversity (i.e. 2Tx) are needed to be defined.

For PC2 single band n66 not supporting Tx diversity (i.e. 1Tx), the Reference Sensitivity Degradation from PC3 to PC2 are defined in Table:

Table 5.x.3-1: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n66 | Skyworks(R4-2300652) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
|  | Huawei, HiSilicon(R4- 2304517) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Average |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |

For PC2 single band n8 supporting Tx diversity (i.e. 2Tx), several companies shared the analysis, and we summarize the required sensitivity degradation levels from companies as shown in 5.x.3-2:

Table 5.x.3-2: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n66 | Skyworks(R4-2300652) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
|  | Huawei, HiSilicon(R4- 2304517) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Average |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |

<End of changes>