3GPP TSG-RAN WG4 Meeting # 108bis	R4-2315483

# Xiamen, China, October 09 – October 13, 2023

Agenda item:	5.26.8.2

Source:	Apple

Title:	On UE demod and CSI requirements for NR NTN enhancement

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

In Rel-18 a new WI for NR NTN enhancements was approved in RAN#94e. The revised WID [1] was approved in RAN#101. In this contribution we present our views on the scope for UE demodulation ad CSI reporting requirements for NTN enhancements.

# 2. Discussion

The performance part objective for eNTN from [1]:

The related core part objective for support of NTN deployment in > 10GHz bands is:

The UE demod performance part requirements are to be discussed for NTN deployment in > 10GHz.

- R18 NR NTN enhancements is to support NTN deployments in bands > 10GHz in FDD mode.

- For R18 NR NTN enhancements demod part we need to introduce demod and CSI reporting requirements for bands > 10GHz in FDD mode.

The UE demod and CSI reporting requirements in FR2 are only defined for TDD bands for TN deployment. The existing demodulation and CSI reporting requirements in FR2 cannot be applicable for NTN supporting > 10GHz bands.

- For TN we have requirements in FR2 for TDD duplex mode alone.

- Existing demodulation and CSI reporting requirements for FR2 cannot be directly applicable to R18 NR NTN.

We propose to introduce a minimum set of requirements for demodulation and CSI reporting for eNTN for FR2 in FDD bands.

For UE demodulation, we should introduce new PDSCH demodulation requirements as existing requiremetns are only for TDD. We propose to introduce a minimum set of requirements with UE mandatory features – Mapping Type A, MMSE-IRC receiver, for rank 1 transmission.

- Define a minimum set of PDSCH demodulation requirements for eNTN covering mandatory UE features – mapping Type A, MMSE-IRC receiver, rank 1 transmission.

The PDCCH demodulation requirements in FR2 are defined for TDD but should also be applicable to FDD mode. We should evaluate the feasibility of using the same requirements from Rel-15 for PDCCH in FR2 for eNTN, without having to introduce a new set of requirements.

- Evaluate feasibility of reusing FR2 PDCCH demod requirements for eNTN.

For PBCH demod requirements in FR2, we have defined requirements for SSB SCS of 120KHz and 240KHz – Case-D and Case-E. The same SSB patterns are also used for NTN for >10GHz. Hence, there is no need to define new PBCH requirements for eNTN.

- The PBCH demod requirements for FR2-1 are defined for SSB SCS of 120KHz and 240KHz. The same SSB SCS/patterns are supported for eNTN.

- Reuse the existing PBCH demod requirements for FR2-1 for eNTN.

For CSI reporting requirements, we have CQI, PMI, RI reporting requirements for FR2-1. Given the round trip delay with NTN, we don’t think it is very critical to test CSI reporting requirements, as CSI feedback from the UE is likely outdated and not used. This is especially true with PMI reporting. There might be some value in verifying the CQI mapping. CQI reporting in fading propagation conditions would not be testable due to long RTT. Hence, we should further discuss if we should have CQI reporting in AWGN for eNTN.

- CSI feedback from UE is likely outdate and not used – especially for reported PMI.

- CQI reporting in NTN might still be useful.

- CQI reporting in fading conditions is not practical/ testable due to long RTT.

- Further discuss if CQI reporting in AWGN should be introduced for NTN.

In Rel-17 RAN4 introduced requirements for NTN in FR1. There we assumed that for UE demodulation requirements we don’t model and Doppler shift, timing drift, sampling frequency offset – since these are compensated by the UE prior to baseband processing and not part of verifying UE demod requirements. We propose to use the same methodology for R18 eNTN demod requirements.

- Reuse the methodology used in R17 NTN demod requirements for R18 eNTN 
– Assume UE compensation for Doppler shift prior to DL demodulation, i.e. no Doppler shift model
–  Do not consider timing drift and sampling frequency offset for UE demod requirements

For NTN operation in >10GHz SCS of 60KHz, 120KHz are supported. The supported channel BWs are:

- 50, 100 and 200 MHz channel bandwidths are supported for 60 kHz SCS.

- 50, 100, 200 and 400 MHz channel bandwidths are supported for 120 kHz SCS.

For demodulation and CSI requirements for eNTN, we think it is sufficient to define requirements with 120KHz SCS with 100MHz CBW.

- Define UE demod and CSI reporting requirements for 120KHz SCS with 100MHz CBW.

Given the link budget in NTN, we don’t think it is practical to employ high modulation order. We propose to define requirements with QPSK, 16QAM for PDSCH.

- Define PDSCH demod requirements for QPSK, 16QAM.

In Rel-17 NTN, we introduced simplified channel models for NTN-TDLA and NTN-TDLC for NLOS and LOS propagation conditions respectively. We propose to use those as a starting point for requirements for eNTN.

- Consider NTN channel models NTN-TDLA (NLOS) and NTN-TDLC (LOS) for R18 NTN requirements.

The Doppler used for R17 NTN demod requirements was 200Hz, and that was derived based on maximum frequency error of 0.1 ppm after compensation. Using the same logic, the Doppler for FR2 bands would be 3000Hz,which is pretty large to define any meaningful demod requirements. We should further discuss a practical value for Doppler for eNTN FR2 requirements.

- Using the logic as FR1 NTN to use 0.1 ppm as the Doppler would lead to 3000Hz Doppler in FR2, which is not practical to define requirements.

- Further discuss Doppler to be used for NTN requirements for FR2 bands.

The HARQ enhancements introduced in R17 NTN are also applicable to NTN in bands >10GHz. In Rel-17 we define requirements for-

- 16, 32 HARQ processing

- Disabled HARQ feature

There were some testability issues identified with the test with disabled HARQ. Hence, we would like to further discuss if we need to introduce requirements with disabled HARQ feature for eNTN.

- Define PDSCH demod requirements with 16, 32 HARQ processes.

- Further discuss if requirements with disabled HARQ are introduced.

# 3. Conclusion

In this paper, we provide our views on the scope for UE demodulation ad CSI reporting requirements for NTN enhancements. Our observations and proposals are captured below:

- R18 NR NTN enhancements is to support NTN deployments in bands > 10GHz in FDD mode.

- For R18 NR NTN enhancements demod part we need to introduce demod and CSI reporting requirements for bands > 10GHz in FDD mode.

- For TN we have requirements in FR2 for TDD duplex mode alone.

- Existing demodulation and CSI reporting requirements for FR2 cannot be directly applicable to R18 NR NTN.

- Define a minimum set of PDSCH demodulation requirements for eNTN covering mandatory UE features – mapping Type A, MMSE-IRC receiver, rank 1 transmission.

- Evaluate feasibility of reusing FR2 PDCCH demod requirements for eNTN.

- The PBCH demod requirements for FR2-1 are defined for SSB SCS of 120KHz and 240KHz. The same SSB SCS/patterns are supported for eNTN.

- Reuse the existing PBCH demod requirements for FR2-1 for eNTN.

- CSI feedback from UE is likely outdate and not used – especially for reported PMI.

- CQI reporting in NTN might still be useful.

- CQI reporting in fading conditions is not practical/ testable due to long RTT.

- Further discuss if CQI reporting in AWGN should be introduced for NTN.

- Reuse the methodology used in R17 NTN demod requirements for R18 eNTN 
– Assume UE compensation for Doppler shift prior to DL demodulation, i.e. no Doppler shift model
–  Do not consider timing drift and sampling frequency offset for UE demod requirements

- Define UE demod and CSI reporting requirements for 120KHz SCS with 100MHz CBW.

- Define PDSCH demod requirements for QPSK, 16QAM.

- Consider NTN channel models NTN-TDLA (NLOS) and NTN-TDLC (LOS) for R18 NTN requirements.

- Using the logic as FR1 NTN to use 0.1 ppm as the Doppler would lead to 3000Hz Doppler in FR2, which is not practical to define requirements.

- Further discuss Doppler to be used for NTN requirements for FR2 bands.

- Define PDSCH demod requirements with 16, 32 HARQ processes.

- Further discuss if requirements with disabled HARQ are introduced.

# Reference

- RP-232669, “Revised WID: NR NTN (Non-Terrestrial Networks) enhancements”, Thales.