3GPP TSG-RAN WG4 Meeting # 108bis	R4-2315800

Xiamen, China, October 09 – October 13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On Network energy savings conformance testing

Agenda item:	5.34.2	BS conformance testing requirements [Netw_Energy_NR-Perf]

Document for:	Approval

Introduction

RAN4#108bis meeting continue discussion and works for core part of the NES work item [1], that should be finalized on RAN#102 (Dec. 2023). On top of that RAN4#108bis according to agreed work plan starts discussion on BS conformance with completion target RAN#104 (June 2024)

In this contribution we initiate discussion on BS conformance testing for network energy savings.

Discussion

2.1 Testing of BS Network energy savings

As outcome of Network energy savings WI power consumed for BS transmitter should be lowered to receive energy savings benefits. When BS functionality with switching ON/OFF is introduced to assist cell DTX, the energy saving consumed at the BS needs to be verified. However, in NR BS test specifications (TS 38.141-1 and TS 38.141-2) there isn’t exist any type of test that would allow to verify energy savings.

Observation 1:  In NR BS test specifications (TS 38.141-1 and TS 38.141-2) there isn’t exist any type of test that would allow to verify energy savings.

Currently, the BS RF transmitter tests in NR test specifications (TS 38.141-1 and TS 38.141-2) aim for measuring the transmitted signal quality (e.g., unwanted emission and EVM measurement). Hence the test models (test signals) in the BS RF transmitter tests are configured to be transmitted either with minimum or maximum BS output power. Therefore, measuring the BS transmitted power of these test signals would not provide sufficient information on the energy saving features, and cannot be used to test energy savings features.

Observation 2:  Currently the BS RF transmitter tests in NR test specifications are designed for measuring the transmitted signal quality (e.g., unwanted emission and EVM measurement).

On the other hand, the current BS transmitter OFF power test aims for measuring the output power level when all transmitter chains are OFF, which is not the case for energy saving features where parts of the transmitter chains would still be operating, and the output power level required during transmitter OFF would not be the same as during cell DTX. Therefore, the current BS transmitter OFF power test is not sufficient for testing of energy saving features.

Observation 3:  The current BS transmitter OFF power test is not sufficient for testing of energy saving features.

Lack of BS RF or RRM test in NR specifications (TS 38.141-1/2 and TS 38.133) that would allow to evaluate real energy savings from the BS side. Without such test, it would not be possible to verify the various features proposed in this WI can achieve energy savings from the BS side. For example, it would not be possible to verify the BS switch-off the TRX chains and PAs efficiently and effectively as expected during the micro/light/deep sleep duration, which is made available with the various features proposed in Network energy savings WI.

Observation 4: Without introduction specific dedicated test, it would not be possible to verify the various features proposed in NES WI can achieve energy savings from the BS side.

Thus, we propose to consider introduction of new testing for network energy savings for BS conformance testing.

Proposal 1:  It is proposed to consider introduction of new testing for Network energy savings for BS conformance testing.

Conclusion

In this contribution we initiated discussion on BS conformance testing for network energy savings. We have made following observations and proposal:

Observation 1:  In NR BS test specifications (TS 38.141-1 and TS 38.141-2) there isn’t exist any type of test that would allow to verify energy savings.

Observation 2:  Currently the BS RF transmitter tests in NR test specifications are designed for measuring the transmitted signal quality (e.g., unwanted emission and EVM measurement).

Observation 3:  The current BS transmitter OFF power test is not sufficient for testing of energy saving features.

Observation 4: Without introduction specific dedicated test, it would not be possible to verify the various features proposed in NES WI can achieve energy savings from the BS side.

Proposal 1:  It is proposed to consider introduction of new testing for network energy savings for BS conformance testing.

- References

- RP-223540, New WID: Network energy savings for NR, Huawei