3GPP TSG-RAN WG4 Meeting # 108-bis	R4-2315989

# Xiamen, China, Oct.9th – Oct.13th, 2023

Title: 	Discussions on Rel-18 DSS performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	5.36.2

Document for:	Discussion

Background

RAN1 has almost concluded the core part of Rel-18 DSS enhancement. RAN4 is responsible for defining the corresponding UE demodulation and CSI requirements. This contribution provides our over views on test scenario and test setup.

- Discussions

General

Two features are introduced by RAN1, which can be briefly summarized as UE support PDCCH reception overlapping with LTE CRS and support two overlapping CRS rate-matching patterns. Such two new features have large impact on the performance and UE’s baseband processing compared to legacy PDCCH/PDSCH features. Hence we propose to define the corresponding performance requirements.

Proposal 1: RAN4 to define following demodulation requirements:

PDCCH overlapping with LTE CRS

PDSCH with two overlapping CRS rate-matching patterns configured

PDCCH requirements

General test setup and receiver assumption

RAN1 has agreed following:

Agreement Reception of NR PDCCH candidates that overlap with LTE CRS REs is supported by Rel18 UEs PDCCH candidates and PDCCH-DMRS RE mapping are based on that of R15 from UE side.

We can observe that PDCCH candidate and PDCCH DMRS RE mapping are not changed. From BS side, to avoid the interference from PDCCH to LTE CRS, it’s high likely for BS to puncture the PDCCH data and DMRS REs overlapped with LTE CRS, RAN1 also considered this implementation as typical scenario for evaluation. Therefore, RAN4 should also follow the RAN1’s common understanding to define the test setup that TE puncture the PDCCH REs and PDCCH data. For the receiver assumption, RAN4 to assume that UE sets the LLR corresponding to CRS REs to zero which is also considered as typical UE implementations from RAN1.

Proposal 2: RAN4 to define following test setup and receiver assumption:

Transmission side: TE puncture the PDCCH data and DMRS REs overlapped with LTE CRS

Receiver side: UE sets the LLR corresponding to CRS REs to zero

Work scope:

For the feature of supporting PDCCH reception overlapping LTE CRS, RAN1 defined following capabilities:

Reception of a NR PDCCH candidate in REs that overlap with LTE CRS: candidate value set {a) when at least one symbol of the NR PDCCH candidate is not overlapped with LTE CRS , b) when some or all of symbols of NR PDCCH candidate overlap with LTE CRS} Note: it is RAN1 understanding that the feature is supported by UE performing channel estimation with a regular legacy DMRS pattern in frequency dimension, i.e., no change to UE assumption on PDCCH DMRS RE positions/pattern in a symbol that are used for the purpose of channel estimation. For component 2, RAN1 considers support value b) in component 2 only if RAN4 performance requirements for value b) are not defined

We can observe that candidate value a) has been confirmed by RAN1, which means UEs reporting a) only support partial overlapping, implicitly expressing that such UE is most likely to use clean symbol for channel estimation to achieve better performance.  Hence, we propose to define the cases to verify UE’s performance for such advanced channel estimation

Candidate value b) has not been confirmed by RAN1 yet, which depends on the RAN4’s input. Companies concern that channel estimation performance is seriously degraded if all PDCCH symbols are interfered by LTE CRS. Our understanding is that RAN4 should evaluate the performance of this scenario (All PDCCH symbols are overlapped with LTE CRS symbols) and compare the performance with candidate value a). If the performance difference is acceptable, RAN4 will not define the requirements of b) and inform RAN1 that such capability is suggested to be introduced, otherwise, RAN4 to inform RAN1 that performance of value b) is unacceptable and such capability is suggested to be dropped.

Proposal 3: RAN4 to evaluate performance of following scenarios

Scenario a): At least one PDCCH symbol is not overlapped with LTE CRS.

Scenario b): All PDCCH symbols are overlapped with LTE CRS

- Proposal 4: RAN4 to define the performance requirements at least for scenario a). Regarding scenario b):

If the performance difference between scenario a) and scenario b) is acceptable, RAN4 will not define the requirements of b) and inform RAN1 that such capability is suggested to be introduced, otherwise, RAN4 to inform RAN1 that performance of value b) is unacceptable and such capability is suggested to be dropped.

Channel estimation

As denoted above, if a UE report value a), it's most likely for the UE to perform channel estimation by using clean symbol to achieve better performance. Hence, for scenario 1, such channel estimation should be assumed for minimum requirements definition.  For scenario b), all PDCCH symbols are overlapped with LTE CRS symbols, legacy channel estimation should be assumed for minimum requirements definition purpose.

Proposal 5: RAN4 to consider following channel estimation methods

Scenario a): Time domain: Use clean symbol; Frequency domain: Regular DMRS pattern

Scenario b): Time domain: Legacy channel estimation ; Frequency domain: Regular DMRS pattern

Detailed assumption

A UE capability that UE supports recept NR PDCCH candidates overlapping with LTE CRS REs with multiple non-overlapping CRS rate matching patterns is introduced. Based on our understanding, different CRS rate matching patterns with different vShift will result in same number of interferenced PDCCH REs and DMRS REs. Therefore we don’t see the necessity to define requirements for such capability

Proposal 6: Don’t define requirements for NR PDCCH overlapping with multiple non-overlapping CRS rate matching patterns

For the PDCCH reception location, following capabilities are defined:

Reception of NR PDCCH candidates that overlap with LTE CRS REs on the X-th symbols of an NR slot. Candidate values for X: {only 2nd symbol, 1st and 2nd symbols}

First symbol usually has LTE PCFICH and PHICH which can bring large interference to NR PDCCH, so RAN1 introduced the separate capability that UE only supporting receive PDCCH symbol overlapping with CRS symbols on 2nd symbol. For simplicity, it’s proposed to only consider scenario that PDCCH and LTE CRS are overlapped on 2nd  symbol since it’s very difficult to define the LTE interference model for 1st symbol. Meanwhile, consideing 1 and 2 CORESET symbols are typical configurations for RAN4 performance requirements definition, so we propose the following:

Proposal 7: Don’t consider the scenario that PDCCH and CRS are overlapped on the 1st symbol.

Proposal 8: Consider following CORESET configuration:

Scenario a): CORESET: 2nd and 3rd symbols, LTE: 4 CRS Ports

Scenario b): CORESET: 2nd symbol, LTE: 4 CRS Ports

Others

Based on our simulation results, the span of performance difference between scenario a) and b) for different Aggregation level is very large. So it’s proposed to cover all Aggregation levels for evaluation purpose. (Not for requirements definition purpose)

Proposal 9: RAN4 to cover all PDCCH Aggregation levels for evaluation purpose. (Not for requirements definition purpose)

Initial simulation results

We provide the initial simulation results for both scenarios as follows. The simulation assumptions are shown in Appendix.

Figure 2-1: BLER-SNR curve

Table 2-1: Target SNR@ 1% of BLER summary

| AL | 1 | 2 | 4 | 8 | 16 |
| --- | --- | --- | --- | --- | --- |
| Scenario a) (Partial overlapping) | 15.9 | 7.0 | 1.5 | -2.5 | -6.0 |
| Scenario b) (Full overlapping) | N/A | N/A | 4.9 | -1.2 | -4.1 |

We have following observations:

Observation 1: For scenario b), the target SNR can’t be achieved for AL1 and 2, for AL4,6,8, there 1.9dB~3.3dB performance degradation compared to scenario a)

We propose to take parameters listed in Table 5-1 as baseline for evaluation.

Proposal 10: Take parameters listed in Table 5-1 as baseline for evaluation.

PDSCH requirements

For PDSCH requirements, we propose to reuse all parameters of Rel-15 LTE-NR coexistence test. Besides, one additional CRS pattern with vShift=1 should be configured. The proposed test parameters and test cases are captured in Table 2-1 to 2-3:

Table 2-2: Proposed test parameters for PDSCH requirements

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD, TDD |
| Active DL BWP index |  |  | 1 |
| NR UL transmission with a 7.5 kHz shift to the LTE raster |  |  | true |
| PDCCH configuration | Symbols with PDCCH |  | Symbol# 2 |  |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 3 |
|  | Length (L) |  | 9 for Test 1-1 11 for Test 1-2 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Position of the first DM-RS for downlink |  | 3 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CRS for rate matching1 (Note 1) | LTE carrier centre subcarrier location |  | Same as NR carrier centre subcarrier location |
|  | LTE carrier BW | MHz | 10 |
|  | Number of antenna ports |  | 4 |
|  | v-shift |  | 0 |
| CRS for rate matching1 (Note 1) | LTE carrier centre subcarrier location |  | Same as NR carrier centre subcarrier location |
|  | LTE carrier BW |  | 10 |
|  | Number of antenna ports |  | 4 |
|  | v-shift |  | 1 |
| Number of HARQ Processes |  |  | 4 for FDD, 8 for TDD |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 for FDD, Specific to each TDD UL-DL pattern for TDD |
| Note 1: No MBSFN is configured on LTE carrier |  |  |  |

Table 2-3: Proposed test cases for PDSCH TDD requirements

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 TDD | 10 / 15 | QPSK, 0.30 | FR1.15-1 | TDLA30-10 | 4x2, ULA Low | 70 | TBD |
| 1-2 | R.PDSCH.1-1.2 TDD | 10 / 15 | QPSK, 0.30 | FR1.15-1 | TDLA30-10 | 4x2, ULA Low | 70 | TBD |

Table 2-4: Proposed test cases for PDSCH FDD requirements

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-7.1 FDD | 10 / 15 | QPSK, 0.30 | TDLA30-10 | 4x2, ULA Low | 70 | TBD |
| 1-2 | R.PDSCH.1-7.2 FDD | 10 / 15 | QPSK, 0.30 | TDLA30-10 | 4x2, ULA Low | 70 | TBD |

Proposal 11: Use test parameters listed in Table 2-2 to 2-4 to define PDSCH requirements

- Conclusion

In this meeting, we provide our views on how to define PDCCH and PDSCH requirements for Rel-18 eDSS. The observations and proposals are:

Proposal 1: RAN4 to define following demodulation requirements:

PDCCH overlapping with LTE CRS

PDSCH with two overlapping CRS rate-matching patterns configured

Proposal 2: RAN4 to define following test setup and receiver assumption:

Transmission side: TE puncture the PDCCH data and DMRS REs overlapped with LTE CRS

Receiver side: UE set the LLR corresponding to CRS REs to zero

Proposal 3: RAN4 to evaluate performance of following scenarios

Scenario a): At least one PDCCH symbol is not overlapped with LTE CRS.

Scenario b): All PDCCH symbols are overlapped with LTE CRS

Proposal 4: RAN4 to define the performance requirements at least for scenario a). Regarding scenario b):

If the performance difference between scenario a) and scenario b) is acceptable, RAN4 will not define the requirements of b) and inform RAN1 that such capability is suggested to be introduced, otherwise, RAN4 to inform RAN1 that performance of value b) is unacceptable and such capability is suggested to be dropped.

Proposal 5: RAN4 to consider following channel estimation methods

Scenario a): Time domain: Use clean symbol; Frequency domain: regular DMRS pattern

Scenario b): Time domain: Legacy channel estimation ; Frequency domain: regular DMRS pattern

Proposal 6: Don’t define requirements for NR PDCCH overlapping with multiple non-overlapping CRS rate matching patterns

Proposal 7: Don’t consider the scenario that PDCCH and CRS are overlapped on the 1st symbol.

Proposal 8: Consider following CORESET configuration:

Scenario a): CORESET: 2nd and 3rd symbols, LTE: 4 CRS Ports

Scenario b): CORESET: 2nd symbol, LTE: 4 CRS Ports

Proposal 9: RAN4 to cover all PDCCH Aggregation levels for evaluation purpose. (Not for requirements definition purpose)

Observation 1: For scenario b), the target SNR can’t be achieved for AL1 and 2, for AL4,6,8, there 1.9dB~3.3dB performance degradation compared to scenario a)

Proposal 10: Take parameters listed in Table 5-1 as baseline for evaluation.

Proposal 11: Use test parameters listed in Table 2-2 to 2-4 to define PDSCH requirements

- Reference

[1]    R1-2308521 Updated RAN1 UE features list for Rel-18 NR after RAN1#114

- Reference

Table 5-1: Simulation assumptions

| Parameters | Values |
| --- | --- |
| Carrier frequency | 2 GHz |
| SCS | 15 kHz |
| Bandwidth | 20MHz |
| LTE Bandwidth | 20MHz |
| Channel model | TDL-C 300-100 |
| Correlation | Low |
| Number of BS antennas | 4Tx |
| Number of UE antennas | 2 Rx |
| DCI payload (excluding CRC) | 60 bits |
| Interleaving | Non-Interleaved |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 PRBs |
| CRS | single 4 port CRS pattern |
| Aggregation level | 1,2,4,8,16 |
| CORESET | Scenario a): CORESET: 2nd and 3rd symbols, LTE: 4 CRS Ports Scenario b): CORESET: 2nd symbol, LTE: 4 CRS Ports |
| Channel estimation | Scenario a): Use clean symbol Scenario b): Use legacy channel estimation |
| Transmitter | DMRS REs and data REs overlapped with CRS are punctured |
| Receiver | Puncture the CRS REs |
| Power ratio of LTE-CRS RE/NR PDCCH-DMRS RE | 3dB |