3GPP TSG-RAN WG4 Meeting #108	R4-2316035

Toulouse, France, August 21 – August 25, 2023

Title: 	Discussion on the impacts on RRM test cases for R18 SL relay

Source: 	Huawei, HiSilicon

Agenda item:	5.32.2

Document for:	Discussion

Introduction

In RAN4, the impacts on RRM core requirements for R18 sidelink relay enhancement were discussed, and the following agreements have been captured in WF [1].

In this contribution, we will provide the discussion on the impacts on RRM performance for R18 sidelink relay enhancement.

Discussion

For R18 SL relay, the remote UE is enhanced to support multipath scenario, and the impacts of supporting multipath scenario on the relay UE selection/reselection requirements and interruption requirements were discussed in RAN4. RAN4 achieved the following agreements for R18 SL relay enhancement.

Agreements in RAN4#107 [1] Issue 1-1: The requirements for the selection/reselection of the relay UE - The legacy requirement on selection/reselection of relay UE is applicable to R18 scenarios. Issue 1-2: Interruption time in multipath scenario - The existing interruptions due to RRC reconfiguration and SL DRX are applicable to multipath scenario (Uu and U2N path). FFS whether and how to capture this in the specification. Issue 1-3: Measurement accuracy - Discovery signal measurements accuracy (clause 10.4.5 in TS38.133) can be applied. No need to define new requirement. Agreements in RAN4#108 [2] Issue 1-1: Interruption time in multipath scenario - Clarify interruption requirements caused by the remote UE on its serving cell in TS 38.133 Issue 1-2: Selection/reselection of relay UE in UE-to-UE relay scenario - Applicability of requirements on selection/reselection of relay UE by the remote UE in U2U relay scenario should be explicitly captured in clause 12.10 of TS 38.133.

It can be observed that the existing RRM requirements in R17 can also be applied for R18 enhanced SL relay. Only some applicability rules or clarifications needs to be added in current RRM specification. There is no new RRM requirements defined for R18 enhanced SL relay. Accordingly, there is no need to introduce new RRM test cases to verify the RRM requirements for R18 SL relay enhancements.

Proposal 1: For R18 SL relay enhancements, there is no need to introduce new RRM test cases.

Conclusions

In this paper we provided our analysis on RRM impacts for R18 sidelink relay enhancement. The followings are provided.

Proposal 1: For R18 SL relay enhancements, there is no need to introduce new RRM test cases.

Reference

- R4-2310059, “WF on R18 SL Relay RRM requirements”, LG Electronics

- R4-2314352, “WF on SL Relay RRM Requirements”, LG Electronics