3GPP TSG-RAN WG4 Meeting # 108-bis	R4-23xxxxx

Xiamen, China, October 09 – October 13, 2023

Title: 	WF on n5B BCS1

Agenda Item:	4.1.1.1

Source: 	AT&T, Verizon, [Qualcomm], [Murata], [Skyworks], [Apple], …

Document for:	Approval

# CA_n5B MSD for 25MHz Aggregated CBW

Way forward:

Take the average of the MSD values provided by companies that are relatively consistent and do not consider the outlier MSD values at this meeting. Agree to keep the average values in brackets and allow additional input at RAN4 #109 to confirm the MSD values.

| Proposal | CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCS0 Ref | CA_n5B | 15/15 | 10MHz + 10MHz | 10 (RBSTART = 0) | 10 (RBSTART = 42) | 30.8 | 26.1 | FDD |
| Qualcomm | CA_n5Bx | 15/15 | 5MHz + 20MHz | 4 (RBSTART = 0) | 16 (RBSTART = 90) | 43.4 | 20.6 | FDD |
| Murata | CA_n5Bx | 15/15 | 5MHz + 20MHz | 4 (RBSTART = 0) | 16 (RBSTART = 90) | [42.5] | [18.2] | FDD |
| Skyworks | CA_n5BX | 15/15 | 5MHz + 20MHz | 4 (RBSTART = 0) | 16 (RBSTART = 90) | 46.4 | 26.2 | FDD |
| Average | CA_n5BX | 15/15 | 5MHz + 20MHz | 4 (RBSTART = 0) | 16 (RBSTART = 90) | [44.1] | [21.7] | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE X: Applicable only to BCS 1 for both 5MHz + 20MHz and 10MHz + 15 MHz. Only 5MHz + 20MHz test point is defined. |  |  |  |  |  |  |  |  |