3GPP TSG-RAN WG4 Meeting # 108bis 														R4-23XXXX

Xiamen, China, October 09 - October 13, 2023

Agenda item:			5.13.8

Source:	Moderator (CMCC)

Title:	Ad hoc minutes for [108bis][212] NR_ATG

Document for:	Approval

Introduction

Summary of RRM core requirements maintenance and RRM performance requirements for Rel-18 NR ATG, including agenda 5.13.5 and 5.13.6 are in R4-2317204. The agreed way forward in previous meetings are R4-2314307, R4-2310159, R4-2306344, R4-2303226, R4-2220361, R4-2217256 and R4-2214737.

The adhoc session will focus on some maintenance issues first and then discuss issues of performance requirements.

Maintainance

Last meeting agreements for phase antenna array impact: Issue 4-1-1: Phase antenna array impact for L3 measurements - Do not introduce scaling factor N and the existing measurement requirements can be reused for ATG UEs [with omnidirectional antennas] - Introduce scaling factor N for L3 measurements for ATG UEs with [antenna arrays] - N = 3 for the case when network assistance on ATG cells reference locations is provided - N = 4 otherwise Issue 4-1-2: Phase antenna array impact for L1 measurements - Do not introduce scaling factor N. Existing measurement requirements can be reused, provided that beam steering based on the positioning is capable for the ATG UE.

Issue 5-1: Phase antenna array impact for L3 measurements

- Proposals

  - For the applicability of scaling factor

  - Option 1: The scaling factor should be also applied for L3 measurements with MG. (CMCC)

  - For the specific value of scaling factor

  - Option 1: (CMCC, HW, ZTE)

    - For intra-frequency L3 measurements without MG of ATG UE with antenna array

      - N = 3 for the case when network assistance on ATG cells reference BS locations is provided

      - N = 4 otherwise

    - For inter-frequency L3 measurements (also for RRC IDLE/INACTIVE mode) of ATG UE with antenna array

      - N = 2 for the case when network assistance on ATG cells reference BS locations is provided

      - Option 1-1: N = 3 otherwise (CMCC, ZTE)

      - Option 1-2: N =4 otherwise (HW)

- Discussion

  - CMCC: for inter-frequency, UE does not need to measure serving cell direction, N can reduce by 1.

  - Qualcomm: UE can have neighbour cell with same serving cell direction.

  - ZTE: same view as CMCC. For ATG deployment, since location of ATG cells are fixed. If UE does not need to measure neighbour cell, the scaling factor can be reduced.

  - Qualcomm: same as FR2, sweeping beam is based on UE implementation.

  - LGE: difference between ATG and FR2 UE is that network will provide neighbour BS reference locations.

  - CMCC: ATG with extreme large ISD is difficult to have multiple neighbour cells for UE to monitor.

  - ZTE: last meeting, N=3 comes from one beam of serving cell and two beams of neighbour cells. It is natural N=2 if neighbour BS location is provided.

- For further offline discussion during this week:

    - For inter-frequency L3 measurements (also for RRC IDLE/INACTIVE mode) of ATG UE with antenna array

      - N = 2 for the case when network assistance on ATG cells reference BS locations is provided

      - N= 4 otherwise

Issue 2-2: Tsearch in HO requirements for UE with antenna array

- Proposals

  - For HO requirements, when network assistance on ATG unknown target cell’s reference BS locations is provided,

    - Option 1-1:  no need to scale the Tsearch (CMCC, ZTE)

  - For HO requirements, when network assistance on ATG unknown target cell’s reference BS locations is not provided,

    - Option 1-2: the Tsearch should be scaled with scaling factor N=3. (CMCC)

    - Option 1-3: the Tsearch should be scaled with scaling factor N=[3 or 4]. (ZTE)

- Discussion

  - Nokia: unknown cell means UE does not measure this cell before, it still needs to perform beam sweeping. Tsearch needs to be scaled.

  - Qualcomm: agree with Nokia. Same scaling factor should be applied to HO Tsearch.

  - ZTE: If BS location is provided, UE can perform beam steering. For unknown cell, UE perform beam steering instead of beam sweeping. No scaling is needed.

  - CMCC: Same view as ZTE.

  - Huawei: For the case location is provided, agree with N=1. For the case location is not provided, agree with N=4

  - ZTE: For the case location is provided, N equals to the cell UE needs to measure. During HO, UE only needs to measure one target cell.

  - Nokia: To compromise, for the case location is provided, N=2.

- Tentative Agreement:

  - For HO requirements, when network assistance on ATG unknown target cell’s reference BS locations is provided,

    - the Tsearch should be scaled with scaling factor N= 2

  - For HO requirements, when network assistance on ATG unknown target cell’s reference BS locations is not provided,

    - the Tsearch should be scaled with scaling factor N=4

Issue 2-3: Tmeasure in CHO requirements for UE with antenna array (not discussed in adhoc session)

- Proposals

  - Option 1: For intra-frequency CHO requirements, when network assistance on ATG cells reference BS locations is provided to UE with antenna array, the scaling factor of Tidentify intra with index or Tidentify_intra_without_index should be decreased to N=2, and N=3 otherwise. (CMCC)

- Recommended WF

  - To be discussed

Issue 2-4: Requirement applicability for CHO requirement

- Proposals

  - Option 1: For CHO requirements, it should be applicable even if UE doesn’t have the valid and applicable parameters of BS location information and Koffset for target NR cell. (CMCC)

- Discussion

  - CMCC: in ATG, one is reference location for CHO, i.e. the cell centre, the other one is the BS reference location for neighbour cell assistance.

  - Nokia: We agreed that for ATG BS location can be optionally provided. So even without BS location, CHO can still work with beam sweeping, i.e. scaling factor N.

- Tentative agreement:

  - Agree with option 1 in principle and check CR for final agreement.

Issue 2-5: Tidentify_intra_NR in RRC Re-establishment delay requirement for UE with antenna array

- Proposals

  - Option 1: For UE with antenna array, the scaling factor should be considered for unknown cells of intra-frequency and inter-frequency in RRC Re-establishment delay requirement requirements. The scaling factor of L3 measurements should be used. (CMCC)

    - For intra-frequency, N = 3 for the case when network assistance on ATG cells reference BS locations is provided, N = 4 otherwise.

    - For inter-frequency, N = 2 for the case when network assistance on ATG cells reference BS locations is provided, N = 3 otherwise.

  - Option 2: The consideration of phase array antenna should be extended to the requirement of Tidentify_intra_NR and Tidentify_inter_NR, i within RRC re-establishment as: (ZTE)

Table 6.2X.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 5 x N2 x TSMTC) | MAX (800 ms, 10 x N2 x TSMTC) |
| < -8 | FR1 | N/A | 800Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. Note 2: If [omnidirectional antennas] is assumed at UE, N = 1. If [Phase antenna array] is assumed at UE, - When [network assistance on ATG cells reference location] of the target cell is provided to UE, N = 3; - Otherwise, N = [3 or 4]. |  |  |  |

Table 6.2X.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell

| Serving cell SSB Ês/Iot (dB) | FR of target NR cell | Tidentify_inter_NR, i [ms] |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 6 x N2 x TSMTC, i) | MAX (800 ms, 13 x N2 x TSMTC, i) |
| < -8 | FR1 | N/A | 800Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. Note 2: If [omnidirectional antennas] is assumed at UE, N = 1. If [Phase antenna array] is assumed at UE, - When [network assistance on ATG cells reference location] of the target cell is provided to UE, N = 2; - Otherwise, N = [3 or 4]. |  |  |  |

- Recommended WF

  - Firstly, for the UE with antenna array, discuss whether the scaling factor is needed for known NR cell and unknown NR cell separately

  - Secondly, discuss the scaling number

- Discussion

  - CMCC: for known cell, scaling factor is not needed.

  - Ericsson: Same view as CMCC.

- Tentative agreement:

  - Scaling factor is not needed for known cell

  - FFS on scaling factor for unknown cell

Issue 2-6: Tidentify_NR in RRC connection release with redirection requirement for UE with antenna array (not discussed in adhoc session)

- Proposals

  - Option 1: For UE with antenna array, the scaling factor of L3 inter-frequency measurements are suggested to be used. N = 2 for the case when network assistance on ATG cells reference BS locations is provided, N = 3 otherwise. (CMCC)

  - Option 2: The consideration of phase array antenna should be extended to the requirement of Tidentify-NR within RRC connection release with redirection to NR as below: (ZTE)

Table 6.2X.3.2.1-1: Time to identify target NR cell for RRC connection release with redirection to NR

| FR of target NR cell | Tidentify-NR |
| --- | --- |
| FR1 | MAX (680 ms, 11 x N2 x Trs) |
| Note 1: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell. Note 2: If [omnidirectional antennas] is assumed at UE, N = 1. If [Phase antenna array] is assumed at UE, - When [network assistance on ATG cells reference location] of the target cell is provided to UE, N = 2; - Otherwise, N = [3 or 4]. |  |

- Recommended WF

  - Firstly, for the UE with antenna array, discuss whether the scaling factor is needed

  - Secondly, discuss the scaling number

Issue 3-1: validity timer for ATG

- Proposals

  - Option 1:  remove the brackets and keep the validity timer statement, since running validity timer for NTA,adjUE is a sign that the UE has received data in SIB regarding ATG gNB positions and is ready to transmit in UL. (Ericsson)

The ATG UE shall meet the Te_ATG requirement for an initial transmission provided that at least one SSB is available at the ATG UE during the last 160 ms. [and the ATG UE has a validity time running for NTA,UE-specific.] The reference point for the ATG UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus NTA+NTA-offset+NTA,adjUE×Tc.

The timing advance is initiated by ATG UE configured with only PCell [served by gNB upon initiating a validity timer for NTA,adjUE. ] The timing advance can be adjusted with MAC message that implies the adjustment of the timing advance, as defined in clause 5.2 of TS 38.321 [7].

- Discussion

  - CMCC: not sure whether validity time will be introduced in RAN2 spec.

  - Ericsson: OK to keep [] for now.

- Agreement:

  - Keep [] for validity timer and wait for RAN2 conclusion.

Issue 5-2:  If any additional UE capability is needed for ATG UEs with [antenna arrays] (not discussed in adhoc session)

- Proposals

  - Option 1: No capability is needed for ATG UE to support antenna arrays in FR1 4GHz. (Ericsson)

  - Option 2: Define a UE capability to differentiate UE with [omnidirectional antennas] and UE with [antenna arrays]. (HW)

  - Option 3: For the band n79 capable UE, it works with phase antenna array by default at band n79. (ZTE)

    - Regarding the antenna assumption, phase antenna array is tied with band n79, and omni-antenna is tied with other ATG bands.

- Recommended WF

  - To be discussed

Performance requirements

Issue 6-1-1: Measurement accuracy

- Proposals

  - Option 1: Reuse the Es/Iot side condition and measurement accuracy requirement of legacy TN network for ATG.(CMCC)

  - Option 2: The accuracy requirement of L1 measurement should be identified for ATG. Regarding L3 measurement accuracy, reuse the accuracy requirement of legacy FR1 TN for the non-phase array antenna assumption; it is better to identify the accuracy requirement for the phase array antenna assumption. (ZTE)

- Recommended WF

  - reuse the L3 and L1 accuracy requirement of legacy FR1 TN for the non-phase array antenna assumption

  - Further discuss whether to identify the incremental accuracy requirement for the phase array antenna assumption

- Discussion

  - ZTE: For phase array antenna, we consider possible different accuracy requirements.

  - Ericsson: What is the expectation for different accuracy, tightened or what?

- Agreement:

  - Reuse the L3 and L1 accuracy requirement of legacy FR1 TN

Issue 6-2-1: General test configuration

- Proposals

  - Option 1:  (CMCC)

    - Configuration 1:15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, only for UE with omnidirectional antennas

    - Configuration 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, for both UE with omnidirectional antennas and for UE with antenna array

  - Option 2: (ZTE)

    - Testing band:

      - Band n79 should be configured for the sake of phase array antenna relevant enhancements testing;

      - At least one band within {n1, n3, n34, n39, n41, n78} is considered to test other ATG enhancement besides the phase array antenna relevant enhancements testing;

    - Duplex:

      - Similar as in legacy, both FDD and TDD should be configured.

    - SCS:

      - Similar as in legacy, both 15kHz, 30kHz should be configured.

      - Not consider 60kHz since some RRM requirement is vacant for 60kHz.

    - TDD pattern:

      - TDD pattern ‘30D4S6U’, S=’14GP’ should be prioritized in the test.

  - Option 3: RAN4 to discuss additional test cases considering the new TDD pattern for ATG (if agreed). (Ericsson)

- Discussion

  - Qualcomm: proposal of testing band needs to be clarified.

  - ZTE: the bands are intended to differentiate omnidirectional antenna and phase array antenna.

  - Intel: what band is targeted for TDD with 20MHz

  - Moderator: n34 (2000MHz) and n39 (1900MHz)

  - Huawei: whether 20MHz is only specified for RedCap?

- Agreement:

    - Configuration 1:15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, only for UE with omnidirectional antennas

    - Configuration 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, for UE with antenna array

    - Configuration 3: 30 kHz SSB SCS, [20MHz] bandwidth, TDD duplex mode, for UE with omnidirectional antennas

    - FFS on TDD pattern

Issue 6-2-2: Test scope

- Proposals

  - Option 1: Only define test case for non-DRX mode in connected mode (HW, Ericsson)

  - Option 2: Define test cases for both DRX mode and non-DRX mode. (CMCC)

  - Option 3: RAN4 not to define test cases for SCC. (Ericsson)

  - Option 4: A general principle is to only define the test cases for those requirements that are different from the TN deployment or not involved in the TN deployment. (ZTE)

- Discussion

  - HW: DRX mode is for power saving, ATG is not sensitive to power.

  - QC: How about the test case list? Do we repeat all the test cases?

  - Ericsson: We can select some cases for requirements with scaling factor N.

  - CMCC: prefer to pick some test cases for DRX configuration.

  - HW: RRM test case is not for the purpose of DRX functionality.

  - Nokia: UE can only be either omnidirectional or phase antenna array.

- Agreement

  - RAN4 not to define test cases for SCC

  - Only define test case for non-DRX mode in connected mode

  - For requirements with scaling factor, consider to select some configurations for RRM test cases.

For information:

  - Companies can show your support in the table below

| Test scope RRC_IDLE state mobility | Test number | Supporter |
| --- | --- | --- |
| Cell reselection to intra-frequency neighbour cell | ATG 1-1 | CMCC (Neighbour cell reference BS location is provided) HW, Ericsson |
| Location based cell reselection to intra-frequency neighbour cell | ATG 1-2 | CMCC (Neighbour cell reference BS location is provided) |
| Cell reselection to inter-frequency neighbour cell | ATG 1-3 | CMCC (Neighbour cell reference BS location is not provided) HW, Ericsson |
| Cell reselection to inter-frequency NR cell for UE configured with [enhanced requirements] | ATG 1-4 | CMCC (Neighbour cell reference BS location is provided) HW, Ericsson, ZTE |
| Location based cell reselection to inter-frequency neighbour cell | ATG 1-5 | CMCC (Neighbour cell reference BS location is provided) |

| Test scope RRC_CONNECTED state mobility | Test number | Supporter |
| --- | --- | --- |
| Intra-frequency HO with known target cell | ATG 2-1 | CMCC, HW, Ericsson |
| Intra-frequency HO with unknown target cell | ATG 2-2 | CMCC (Neighbour cell reference BS location is provided) HW, Ericsson |
| Inter-frequency HO with known target cell | ATG 2-3 | CMCC, [HW] |
| Inter-frequency HO with unknown target cell | ATG 2-4 | CMCC (Neighbour cell reference BS location is not provided) HW, Ericsson |
| Intra-frequency location based CHO | ATG 2-5 | CMCC (Neighbour cell reference BS location is not provided) HW, Ericsson |
| Inter-frequency location based CHO | ATG 2-6 | CMCC (Neighbour cell reference BS location is provided) HW, Ericsson |
| Intra-frequency RRC Re-establishment with known target cell | ATG 2-7 | CMCC HW, Ericsson |
| Intra-frequency RRC Re-establishment with unknown target cell | ATG 2-8 | CMCC (Neighbour cell reference BS location is not provided) |
| Inter-frequency RRC Re-establishment with known target cell | ATG 2-9 | CMCC |
| Inter-frequency RRC Re-establishment with unknown target cell | ATG 2-10 | CMCC (Neighbour cell reference BS location is provided) HW, Ericsson |
| Inter-frequency RRC Re-establishment in FR1 without serving cell timing | ATG 2-11 | HW |
| 4-step RA type contention based random access test | ATG 2-12 | CMCC, HW, Ericsson |
| 4-step RA type Non-Contention based random access test | ATG 2-13 | CMCC, HW, Ericsson |
| 2-step RA type contention based random access test | ATG 2-14 | CMCC, Ericsson |
| 2-step RA type Non-Contention based random access test | ATG 2-15 | CMCC, Ericsson |
| RRC Connection Release with Redirection | ATG 2-16 | CMCC (Neighbour cell reference BS location is provided) HW, Ericsson |

| Test scope Timing | Test number | Supporter |
| --- | --- | --- |
| UE transmit timing test | ATG 3-1 | CMCC, Ericsson, ZTE |
| Timing advance adjustment delay and accuracy | ATG 3-2 | CMCC, Ericsson, ZTE |

| Test scope Signalling characteristics | Test number | Supporter |
| --- | --- | --- |
| Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode | ATG 4-1 | CMCC, HW, Ericsson |
| Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode | ATG 4-2 | CMCC, HW, Ericsson |
| Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode | ATG 4-3 | CMCC |
| Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode | ATG 4-4 | CMCC |
| Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode | ATG 4-5 | CMCC, HW, Ericsson |
| Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode | ATG 4-6 | CMCC, HW, Ericsson |
| Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode | ATG 4-7 | CMCC |
| Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode | ATG 4-8 | CMCC |
| Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode | ATG 4-9 | CMCC, HW, Ericsson |
| Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode | ATG 4-10 | CMCC |
| Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode | ATG 4-11 | CMCC, HW, Ericsson |
| Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode | ATG 4-12 | CMCC |
| DCI-based and Timer-based Active BWP Switch | ATG 4-13 | CMCC, HW, Ericsson |
| RRC-based Active BWP Switch | ATG 4-14 | CMCC, HW, Ericsson |
| UE specific CBW change | ATG 4-15 | CMCC, HW, Ericsson |
| Pathloss reference signal switching delay | ATG 4-16 | CMCC |

| Test scope Measurement procedure | Test number | Supporter |
| --- | --- | --- |
| Intra-frequency measurements event triggered reporting tests without gap under non-DRX | ATG 5-1 | CMCC (Neighbour cell reference BS location is provided) HW, Ericsson, ZTE (under phase antenna array assumption) |
| Intra-frequency measurements event triggered reporting tests without gap under DRX | ATG 5-2 | CMCC (Neighbour cell reference BS location is not provided), ZTE (under phase antenna array assumption) |
| Intra-frequency measurements event triggered reporting tests with per-UE gaps under non-DRX | ATG 5-3 | CMCC (Neighbour cell reference BS location is not provided) HW, Ericsson, ZTE (under phase antenna array assumption) |
| Intra-frequency measurements event triggered reporting tests with per-UE gaps under DRX | ATG 5-4 | CMCC (Neighbour cell reference BS location is provided), ZTE (under phase antenna array assumption) |
| Intra-frequency measurements event triggered reporting tests without gap under non-DRX with SSB index reading | ATG 5-5 | CMCC (Neighbour cell reference BS location is provided), ZTE (under phase antenna array assumption) |
| Intra-frequency measurements SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading | ATG 5-6 | CMCC (Neighbour cell reference BS location is provided), ZTE (under phase antenna array assumption) |
| Inter-frequency measurements event triggered reporting tests for FR1 without SSB time index detection when DRX is not used | ATG 5-7 | CMCC (Neighbour cell reference BS location is not provided) HW, Ericsson, ZTE (under phase antenna array assumption) |
| Inter-frequency measurements event triggered reporting tests for FR1 without SSB time index detection when DRX is used | ATG 5-8 | CMCC (Neighbour cell reference BS location is provided), ZTE (under phase antenna array assumption) |
| Inter-frequency measurements event triggered reporting tests for FR1 with SSB time index detection when DRX is not used | ATG 5-9 | CMCC (Neighbour cell reference BS location is provided) HW, ZTE (under phase antenna array assumption) |
| Inter-frequency measurements event triggered reporting tests for FR1 with SSB time index detection when DRX is used | ATG 5-10 | CMCC (Neighbour cell reference BS location is not provided), ZTE (under phase antenna array assumption) |
| Inter-frequency measurements event triggered reporting tests for FR1 without gap when DRX is used | ATG 5-11 | CMCC (Neighbour cell reference BS location is provided), ZTE (under phase antenna array assumption) |
| Inter-frequency measurements event triggered reporting tests for FR1 without gap when DRX is not used | ATG 5-12 | CMCC (Neighbour cell reference BS location is provided) HW, ZTE (under phase antenna array assumption) |
| SSB based L1-RSRP measurement when DRX is not used | ATG 5-13 | CMCC, HW, Ericsson, ZTE (under phase antenna array assumption) |
| SSB based L1-RSRP measurement when DRX is used | ATG 5-14 | CMCC, ZTE (under phase antenna array assumption) |
| CSI-RS based L1-RSRP measurement when DRX is not used | ATG 5-15 | CMCC, HW, Ericsson, ZTE (under phase antenna array assumption) |
| CSI-RS based L1-RSRP measurement when DRX is used | ATG 5-16 | CMCC, ZTE (under phase antenna array assumption) |
| L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is used | ATG 5-17 | CMCC, ZTE (under phase antenna array assumption) |
| L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used | ATG 5-18 | CMCC, HW, ZTE (under phase antenna array assumption) |
| L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used | ATG 5-19 | CMCC, HW, ZTE (under phase antenna array assumption) |
| SA intra-frequency CGI identification of NR neighbor cell in FR1 | ATG 5-20 | CMCC |

| Test scope Measurement Performance requirements | Test number | Supporter |
| --- | --- | --- |
| SS-RSRP intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-1 | CMCC, ZTE (under phase antenna array assumption) |
| SS-RSRP inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-2 | CMCC, ZTE (under phase antenna array assumption) |
| SS-RSRQ intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-3 | CMCC, ZTE (under phase antenna array assumption) |
| SS-RSRQ inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-4 | CMCC, ZTE (under phase antenna array assumption) |
| SS-SINR intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-5 | CMCC, ZTE (under phase antenna array assumption) |
| SS-SINR inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-6 | CMCC, ZTE (under phase antenna array assumption) |
| L1-RSRP measurement SSB based L1-RSRP measurement | ATG 6-7 | CMCC |
| L1-RSRP measurement CSI-RS based L1-RSRP measurement on resource set with repetition off | ATG 6-8 | CMCC |
| L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off | ATG 6-9 | CMCC |
| L1-SINR measurement with SSB based CMR and dedicated IMR | ATG 6-10 | CMCC |
| L1-SINR measurement with CSI-RS based CMR and dedicated IMR | ATG 6-11 | CMCC |
| CSI-RSRP intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-12 | CMCC |
| CSI-RSRP inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-13 | CMCC |
| CSI-RSRQ Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-14 | CMCC |
| CSI-RSRQ Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-15 | CMCC |
| CSI-SINR intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-16 | CMCC |
| CSI-SINR Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell | ATG 6-17 | CMCC |

Issue 6-2-3: RMC (not discussed in adhoc session)

- Proposals

  - Option 1: The legacy NR RMCs defined in section A.3.1 of TS 38.133 are reused for ATG test cases. (Ericsson)

- Recommended WF

  - Option 1 can be the baseline

Issue 6-2-4: OCNG (not discussed in adhoc session)

- Proposals

  - Option 1: Following legacy NR OCNGs are reused for ATG test cases: (Ericsson)

    - Generic OCNG pattern for all unused REs defined in A.3.2.1.1 in TS 38.133.

    - Generic OCNG pattern for unused REs in the same bandwidth as CORESET defined in A.3.2.1.3 of TS 38.133.

    - Generic OCNG pattern for all unused REs outside SSB slot(s) defined in A.3.2.1.4 in TS 38.133

- Recommended WF

  - Option 1 can be the baseline.

Issue 6-2-5: Test method for UE with antenna array (not discussed in adhoc session)

- Proposals

  - Option 1: RAN4 to discuss whether and how to define test cases for an ATG UE with antenna array (HW)

  - Option 2: RAN4 to discuss how to model the L3 measurement test with beam sweeping in FR1 4GHz. (Ericsson)

- Recommended WF

  - To be discussed

Issue 6-2-6: whether to define test cases for TCI switching delay requirements (not discussed in adhoc session)

- Proposals

  - Option 1: RAN4 to discuss the test cases for TCI switching delay requirements for MAC-, DCI and RRC-based TCI state switching delay requirements.  (Ericsson)

- Recommended WF

  - To be discussed

Issue 6-2-7: GNSS setup (not discussed in adhoc session)

- Proposals

  - Option 1: GNSS is viable via AT command for all test cases, and GNSS only changed during the test for location-based CHO.  (HW)

- Recommended WF

  - To be discussed

Issue 6-2-8: UE mobility assumption (not discussed in adhoc session)

- Proposals

  - Option 1: For the location-based cell re-selection tests, location-based CHO tests and UL transmit timing tests, the UE mobility should be assumed with 1200km/h. For the other tests, UE could be assumed with no mobility. (CMCC)

- Recommended WF

  - To be discussed

Issue 6-2-9: Neighbour cell configuration (not discussed in adhoc session)

- Proposals

  - Option 1: For the FDD cell re-selection test and intra/inter-frequency measurement test for UE with omnidirectional antenna, configure 1 neighbour cell. For the TDD cell re-selection test and and intra/inter-frequency measurement test for UE with antenna array, choose some test cases to configure 2 neighbour cells. (CMCC)

- Recommended WF

  - To be discussed

Issue 6-2-10: Channel model (not discussed in adhoc session)

- Proposals

  - Option 1: Use the AWGN with residual doppler channel model for RRM test cases.  (CMCC)

- Recommended WF

  - To be discussed