3GPP TSG- RAN4 Meeting # 108bis	R4-2315668

# Xiamen, China, October 09 – October 13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification on interruption in paging reception for HD-FDD RedCap UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Core |  |  |  |  |  | Date: |  |  | 2023-10-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current paging requirements on monitoring of paging occasions for CG-SDT with HD-FDD Redcap UEs are not alligned with RAN2. An LS reply is sent to RAN2 with the following conclusion made in RAN4[R4-2314464] RAN4 will further update requirements for the case of partial collisions of POs with CG-SDT occasions for HD-FDD RedCap UE within the SI modification period based on RAN2 LS There are no existing RRM requirements for the case when all available POs are colliding with CG-SDT occasions for HD-FDD RedCap UE within the SI modification period. RAN4 is not planning to cover this scenario in Rel-17 or Rel-18 specifications.. |
| s |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | For RedCap UE in HD-FDD mode, - If paging occasions partially overlap with CG-SDT transmission, the UE is required to monitor for SI change indication in any paging occasion at least once per modification period [2] during SDT if the initial downlink BWP on which the SDT procedure is ongoing is associated with a CD-SSB. - There is no requirement in case all available paging occasions overlap with the CG-SDT transmissions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect implementation for RedCap UE in HD-FDD mode. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1B.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

5.1B.2.6	Maximum interruption in paging reception

The requirements in clause 4.2B.2.6 shall apply for RedCap UEs.

For RedCap UE in HD-FDD mode, if paging occasions partially overlap with CG-SDT transmission, the UE is only required to monitor for SI change indication in any paging occasion at least once per modification period [2] during SDT if the initial downlink BWP on which the SDT procedure is ongoing is associated with a CD-SSB.

<End of Change 1>