3GPP TSG-RAN WG4 Meeting #109                                                                                         R4-2318493

Chicago, US, November 13 – 17, 2023

Source: 	CATT

Title: 	Discussion on co-existence study for above 10GHz bands

Agenda item:	8.26.2

Document for:	Approval

- Introduction

This contribution provides our views on co-existence study for above 10GHz bands based on our co-existence simulation result.

- Discussion

According to WF [1], the co-existence simulation scenarios are show as below:

| No. | Combination | Aggressor | Victim | Frequency band | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | 27 GHz | ACLR NTN UE to be varied/defined ACS TN gNB fixed |
| 2 | TN with NTN | TN UL | NTN UL | 27 GHz | ACLR TN UE fixed ACS NTN SAN to be varied/defined |
| 3 | TN with NTN | NTN UL | TN DL | 27 GHz | ACLR NTN UE to be varied/defined ACS TN UE fixed |
| 4 | TN with NTN | TN DL | NTN UL | 27 GHz | ACLR TN gNB fixed ACS NTN SAN to be varied/defined |
| 5 | TN with NTN | TN DL | NTN DL | 17 GHz | ACLR TN gNB fixed ACS NTN UE to be varied/defined |
| 6 | TN with NTN | NTN DL | TN DL | 17 GHz | ACLR NTN SAN to be varied/defined ACS TN UE fixed |
| 7 | TN with NTN | NTN DL | TN UL | 17 GHz | ACLR NTN SAN to be varied/defined ACS TN gNB fixed |
| 8 | TN with NTN | TN UL | NTN DL | 17 GHz | ACLR TN UE fixed ACS NTN UE to be varied/defined |
| NOTE 1: For coexistence between Ka-Band DL and adjacent TN bands, there are no 3GPP defined/specified TN bands. NOTE 2: To deprioritize scenario 7 and 8 for SAN elevation angle as 90-degree cases. Scenario 7 and 8 should still be studied for SAN elevation angle as 25-degree cases (computed from the center of the central beam). |  |  |  |  |  |

Referring to our co-existence simulation result in excel [2], we plot required ACIR curve of case 1 to case 8 for VSAT as following.

Figure 2-1: Case 1 Aggressor NTN UL to Victim TN UL (GEO, LEO-1200, LEO-600)

Figure 2-2: Case 2 Aggressor TN UL to Victim NTN UL (GEO, LEO-1200, LEO-600)

Figure 2-3: Case 3 Aggressor NTN UL to Victim TN DL(GEO, LEO-1200, LEO-600)

Figure 2-4: Case 4 Aggressor TN DL to Victim NTN UL(GEO, LEO-1200, LEO-600)

Figure 2-5:  Case 5 Aggressor TN DL to Victim NTN DL(GEO, LEO-1200, LEO-600)

Figure 2-6: Case 6 Aggressor NTN DL to Victim TN DL(GEO, LEO-1200, LEO-600)

Figure 2-7: Case 7 Aggressor NTN DL to Victim TN UL(GEO, LEO-1200, LEO-600)

Figure 2-8: Case 8 Aggressor TN UL to Victim NTN DL(GEO, LEO-1200, LEO-600)

We summarized required ACIR for above 8 cases in Table 2-2.

Table 2-2: Required ACIR for Ka-band NTN co-existence

| Case |  | Case1 | Case2 | Case3 | Case4 | Case5 | Case6 | Case7 | Case8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency |  | 27GHz | 27GHz | 27GHz | 27GHz | 17GHz | 17GHz | 17GHz | 17GHz |
| Scenario case |  | Aggressor NTN UL to Victim TN UL | Aggressor TN UL to Victim NTN UL | Aggressor NTN UL to Victim TN DL | Aggressor TN DL to Victim NTN UL | Aggressor TN DL to Victim NTN DL | Aggressor NTN DL to Victim TN DL | Aggressor NTN DL to Victim TN UL | Aggressor TN UL to Victim NTN DL) |
| Requirement to be defined |  | NTN UE ACLR | SAN ACS | NTN UE ACLR | SAN ACS | NTN UE ACS | SAN ACLR | SAN ACLR | NTN UE ACS |
| Required ACIR[dB] | GEO | 11.25 | 37.5 | 0 | 11 | 38.2 | 0 | 0 | 12 |
|  | LEO-1200 | 11 | 33.5 | 0 | 20.5 | 38.5 | 0 | 0 | 12.5 |
|  | LEO-600 | 9.2 | 33 | 0 | 23 | >40 | 0 | 0 | 12.1 |

From above Table 2-2, for case 2, required ACIR @ 5% throughput loss for GEO, LEO-1200 and LEO-600 is 37.5, 33.5 and 33, respectively, comparing with BS ACS (24dB), the required ACIR(for SAN ACS) is larger. And for case 5, required ACIR @ 5% throughput loss for GEO, LEO-1200 and LEO-600 is 38.2, 38.5 and >40, respectively, comparing with TN UE ACS (25dB), the required ACIR (for NTN UE ACS) is larger.

- Conclusion

This contribution summarized required ACIR based on our co-existence simulation result for above 10GHz bands. The following observation is provided as follows:

Observation 1: Required ACIR for Ka-band NTN co-existence is as below:

| Case |  | Case1 | Case2 | Case3 | Case4 | Case5 | Case6 | Case7 | Case8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency |  | 27GHz | 27GHz | 27GHz | 27GHz | 17GHz | 17GHz | 17GHz | 17GHz |
| Scenario case |  | Aggressor NTN UL to Victim TN UL | Aggressor TN UL to Victim NTN UL | Aggressor NTN UL to Victim TN DL | Aggressor TN DL to Victim NTN UL | Aggressor TN DL to Victim NTN DL | Aggressor NTN DL to Victim TN DL | Aggressor NTN DL to Victim TN UL | Aggressor TN UL to Victim NTN DL) |
| Requirement to be defined |  | NTN UE ACLR | SAN ACS | NTN UE ACLR | SAN ACS | NTN UE ACS | SAN ACLR | SAN ACLR | NTN UE ACS |
| Required ACIR[dB] | GEO | 11.25 | 37.5 | 0 | 11 | 38.2 | 0 | 0 | 12 |
|  | LEO-1200 | 11 | 33.5 | 0 | 20.5 | 38.5 | 0 | 0 | 12.5 |
|  | LEO-600 | 9.2 | 33 | 0 | 23 | >40 | 0 | 0 | 12.1 |

- Reference

- R4-2317008, Simulation assumption for NTN co-existence study, Samsung, RAN4#108bis

- R4-2318298, Co-existence study result for above 10GHz bands, CATT, RAN4#109