3GPP TSG-RAN WG4 # 109	R4-2318552

Chicago, USA, November 13 – November 17, 2023

Agenda item:	8.7.4.3

Source:	MediaTek Inc.

Title:	Discussion on PMI requirements of FR2 Multi-Rx DL Demod

Document for:	Discussion

# Introduction

In RAN4#108-bis, the Way Forward document was approved [1]. We provide discussion of PMI requirements with proposals for the Rel-18 work in this contribution.

# Discussion

In this chapter we discuss remaining open items of PMI requirements of FR2 Multi-Rx DL Demod. We have provided simulation results of PMI reporting with proposed parameters to this meeting in [2].

Issue 3-1-4: UE Processing for PMI Reporting

- Agreement:

  - Use the same agreement on UE processing as PDSCH demod for sDCI for PMI reporting.

In our PMI reporting simulations [2], we do not see significant differences in γ values between separate and joint processing receiver assumptions. Therefore, we consider that both receiver assumptions would be feasible to define requirements with proposed parameters. Moreover, we are still ok to follow previous agreement to align receiver assumptions in PDSCH and PMI reporting requirements.

Proposal #1: We are open to consider joint processing receiver assumption for PMI reporting requirements if UE capability for joint processing is introduced.

Issue 3-1-6: Simulation parameters for PMI reporting requirement for sDCI SDM scheme

- Antenna configuration per TRxP

  - XP 2x4 (N1,N2) = (2,1), (4 antenna at UE across 2 panels)

In general aspects of this work item, we are still discussing the final formulation of correlation model. We propose to finalize that discussion first and define antenna configuration accordingly.

Proposal #2: We propose to finalize correlation model discussion first and define antenna configuration accordingly.

Issue 3-1-7: Performance Metric

- Agreement:

  - Set test metric as γ=t_ue/t_rnd , where t_ue is [70, 90] % of the maximum throughput obtained at SNR_ue using the precoders configured according to the UE reports, and t_rnd is the throughput measured at SNR_ue with random precoding.

In our PMI reporting simulations [2], we do not see significant differences in γ values between 70% and 90% test metric. Therefore, we propose to use 90% test metric as it is more commonly used in PMI requirements.

Proposal #3: Set test metric as γ=tue/trnd, where tue is 90% of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trnd is the throughput measured at SNRue with random precoding.

Issue 3-1-9: MCS

- Agreement:

- MCS: [11,13], MCS Table 1

In our PMI reporting simulations [2], we do not see significant differences in γ values between MCS11 and MCS13. Therefore, we propose to use MCS13 as it is more commonly used in PMI requirements.

Proposal #4: We propose to use MCS13 in PMI reporting requirements.

# Conclusion

In this paper we provided the view on the PMI requirements of FR2 multipanel RX downlink demodulation requirements. The following proposals are made:

Proposal #1: We are open to consider joint processing receiver assumption for PMI reporting requirements if UE capability for joint processing is introduced.

Proposal #2: We propose to finalize correlation model discussion first and define antenna configuration accordingly.

Proposal #3: Set test metric as γ=tue/trnd, where tue is 90% of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trnd is the throughput measured at SNRue with random precoding.

Proposal #4: We propose to use MCS13 in PMI reporting requirements.

# Reference

- R4-2316963, “WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception”, Qualcomm

- R4-2318553, “Simulation results of PMI requirements of FR2 Multi-Rx DL Demod”, MediaTek inc.