3GPP TSG-RAN WG4 Meeting #109                                      	                        R4-2318672

# Chicago, USA, November 13 – November 17, 2023

Agenda item:	8.3.3.1.2

Source:	Apple Inc.

Title:	On the PDSCH Demodulation Requirements for 8Rx UEs in FR1

Release:	Rel-18

Document for:	Discussion

# Introduction

Overall, this WI has been focusing on:

- Enable 8Rx for CPE/FWA/vehicle/industrial devices

  - Specify UE demodulation performance and CSI requirements with up to 8 layers to support 8Rx

    - Investigate and, if necessary, specify the requirements with up to 8 DL MIMO layers

  - Specify the SDR requirements with 8 MIMO layers

In addition, as per the revised WID RP-231490 in RAN#100, we wait for RAN4 RF to specify UE RF requirements for both single carrier and CA/DC for 8Rx. About specific band combos and configurations, these are FFS.

  - Specify the UE RF requirements to support 8Rx for both single carrier and CA/DC

    - Example band combos and configurations need to be defined

The main aim of this paper is to summarize the status and still open discussions related to PDSCH demodulation requirements for 8Rx UEs.

# Discussion

During the last few sessions, the following agreements have taken place regarding PDSCH requirements in TDD and FDD on MCS, Rank and Propagation Conditions.

| Rank | MCS | Propagation Conditions |
| --- | --- | --- |
| 2 | MCS 19 (64QAM Table) | TDLC300-100 ULA Medium B |
| 4 | MCS 17 (64QAM Table) | TDLA30-10 Low |
| 8 | MCS 17 (64QAM Table) |  |

RAN4 defines PDSCH FDD requirements with same parameters including MCS values, MIMO layers, antenna configuration, and propagation conditions as TDD requirements.

Agreement: CBW for PDSCH CA requirements

- Follow the existing CA requirements defined for 2Rx and 4Rx, i.e.

  - FDD: 15kHz SCS: 5/10/15/20/25/30/35/40/45/50 MHz

  - TDD: 30kHz SCS: 5/10/15/20/25/30/40/50/60/80/90/100 MHz

Agreement: Duplex model

- Consider the following duplex model for 8Rx CA

  - FDD 15kHz + TDD 30kHz

  - FDD 15kHz + FDD 15kHz

  - TDD 30kHz + TDD 30kHz

Agreement: Rank

- Rank 8 for 8Rx+8Rx, Rank 2 for 2Rx+8Rx and 4Rx+8Rx

  - Rank 8 + Rank 8 for 8Rx+8Rx for UE supporting Rank 8 for 8Rx

  - Rank 2 + Rank 2 for 4Rx+8Rx

  - Rank 2 + Rank 2 for 2Rx+8Rx

Based on all these previous agreements, our simulation results are summarized in [2].

# References

- R4-2316914, “WF for 8Rx and 4Tx performance requirements”, Huawei, HiSilicon, RAN4#108bis

- R4-2318675, “Summary of Simulation Results for 8Rx Demodulation Requirements”, Apple, RAN4#109