3GPP TSG-RAN WG4 Meeting #109		R4-2318703

Chicago, USA, November 13 – 17, 2023

Agenda item:	10

Source: 	Apple

Title: 	Views on Release 18 feature list

Document for:	Discussion

- Introduction

# In this document, we share our initial views on Rel-18 NR UE features.

- Introduction

This contribution includes the RAN4 UE feature list for Rel-18 NR.

- NR_ENDC_RF_FR1_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_ENDC_RF_FR1_enh2 | 27-1 | Lower MSD for inter-band CA/EN-DC/DC combinations | Indicates that UE is capable of supporting lower MSD than specified. The following components will be specified: - MSD type: H2, H3, H4, H5, HR2, HR3, HR4, HR5, cross-band, IMD2, IMD3, IMD4, IMD5, Number of bits needed: 4 bits? - MSD threshold: (number of bits needed: 3 bits) Aggressor bands and victim bands (not sure if needed to be included in the feature list) | N/A | Yes | N/A | UE cannot support lower MSD capability | Per BC | No | FR1 only | N/A |  | Optional with capability signalling |

- NR_channel_raster_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28. NR_channel_raster_enh | 28-1 | Enhanced channel raster |  | N/A | Yes | N/A | UE cannot channel rasters with a step less than 100kHz | [Per Band] | No | FR1 only | N/A |  | [Optional with capability signalling] |

- NR_RF_FR2_req_Ph3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_RF_FR2_req_Ph3 | 29-1 | Beam correspondence in IA for PC3 UEs | R18 onward UE shall support beam correspondence in initial access and satisfy the corresponding spherical coverage requirement for IA as specified in 38.101-2 | 8-2 | No |  | UE performance in IA in FR2 cannot be guaranteed | Per UE | N/A | FR2 only |  |  | Mandatory without capability signaling |

- NR_FR2_multiRX_DL

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_FR2_multiRX_DL | 30-1 | Simultaneous reception of NR PDCCH/PDSCH and L1 measurement of reference signal or simultaneous L1 measurement of two RSes when the PDCCH/PDSCH and the L1 RS or two L1 RSes overlap in time and have different QCL Type-D | - Support of simultaneous reception of PDCCH/PDSCH and L1 measurement of reference signal overlapping in time and with different QCL Type-D. - Support of simultaneous measurement of two L1 reference signals overlapping in time and with different QCL Type-D. | 16-2c, [2-29a: groupBeamReporting] | Yes | N/A | The UE does not support simultaneous reception/measurement | [Per FSPC] | N/A | FR2 only |  |  | Optional with capability signaling |
| 30. NR_FR2_multiRX_DL | 30-2 | Fast beam sweeping | Support of fast beam sweeping (or smaller beam sweeping factor N) for SSB-based or CSI-RS based L1 measurement. Candidate values for beam sweeping factor: {2,4,6} for FR2-1 |  | Yes | N/A | The UE does not support fast beam sweeping | [Per FSPC] | N/A | FR2 only |  | It’s agreed in RAN4. [R4-2310047] | Optional with capability signaling |
| 30. NR_FR2_multiRX_DL | [30-3] | [Joint Processing of received signal across all RX for single-DCI] | [Support of joint processing for single DCI transmission schemes with different QCL Type D.] | 16-2c | No | N/A | UE doesn't support joint processing and cannot meet the demod requirements defined with joint processing | [Per FSPC] | TDD only | FR2 only |  | Pending PDSCH demod requirements | Optional without capability signaling |
| 30. NR_FR2_multiRX_DL | [30-4] | [Joint Processing of received signal across all RX for multi-DCI] | [Support of joint processing for multi-DCI fully overlapping transmission with different QCL Type-D.] | 16-2c | No | N/A | UE doesn't support joint processing and cannot meet the demod requirements defined with joint processing | [Per FSPC] | TDD only | FR2 only |  | Pending PDSCH demod requirements | Optional without capability signaling |

- NR_RRM_enh3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31. NR_RRM_enh3 | 31-1 | L3 measurement report for unknown SCell activation | Support of reporting valid L3 measurement results for the target being-activated SCell after receiving the SCell activation command |  | Yes | N/A | UE does not support reporting valid L3 measurement results after receiving the SCell activation command | Per UE | No | No | N/A | UE is required to meet the shortened SCell activation delay requirement in TS38.133 [section 8.x.y] if the feature is supported. | Optional with capability signaling |
| 31. NR_RRM_enh3 | 31-2 | Beam sweeping factor reduction for FR2 unknown SCell activation | Support of reducing beam sweeping factor for cell detection if UE has full set (N=8) of beam sweeping during AGC settling part during FR2-1 unknown SCell activation procedure Support of reducing beam sweeping factor for SSB based L1-RSRP measurement if UE has full set (N=8) of beam sweeping during AGC settling part during FR2-1 unknown SCell activation procedure |  | Yes | N/A | UE does not support beam sweeping factor reduction for cell detection during FR2-1 unknown SCell activation. UE does not support beam sweeping factor reduction for SSB based L1-RSRP measurement during FR2-1 unknown SCell activation. | Per Band | N/A | FR2 only | N/A | UE is required to meet the shortened SCell activation delay requirement in TS38.133 [section 8.x.y] if the feature is supported. Candidate values for beam sweeping reduction for cell detection during FR2-1 unknown SCell activation are 1,2,4, or 6. [Agreed in WF R4-2310081] Candidate values for beam sweeping reduction for SSB based L1-RSRP measurement during FR2-1 unknown SCell activation are 0,1,2,3,4,5,6, or 7. [Agreed in WF R4-2310081] | Optional with capability signaling |
| 31. NR_RRM_enh3 | 31-3 | Other measurement enhancement for unknown SCell activation | Support of using SSB periodicity instead of SMTC periodicity for the measurement interval during unknown SCell activation when the SMTC is only configured in measurement object for enhanced unknown SCell activation requirement. Support of performing L1-RSRP measurement in non-DRX mode even DRX is configured during unknown SCell activation |  | Yes | N/A | UE does not use SSB periodicity instead of SMTC periodicity for the measurement interval during unknown SCell activation when the SMTC is only configured in MO for enhanced unknown Scell activation requirement. UE does not support performing L1-RSRP measurement in non-DRX mode even DRX is configured during unknown SCell activation | Per UE | No | No | N/A | UE is required to meet the shortened SCell activation delay requirement in TS38.133 [section 8.x.y] if the feature is supported. | Optional with capability signaling |

- NR_MG_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_MG_enh2 | 32-1 | Pre-configured MG and concurrent MG | Simultaneous activation/deactivation of two Pre-MGs in the same FR | 1) 19-2, 2) 19-3-1 or 19-3-2 | Yes | N/A | UE does not support simultaneous activation/deactivation of two Pre-MGs in the same FR | [Per UE] | No | [Yes] | N/A | Agreed in RAN4#106 R4-2303197 | Optional with capability signaling |
| 32. NR_MG_enh2 | 32-2 | NCSG and concurrent MG | Concurrent gaps with NCSG in an FR | 1) 19-1a or 19-1b 2) 19-2 | Yes | N/A | UE cannot be configured with NCSG and concurrent gap in the same FR | [Per-UE] | No | [Yes] | N/A | Agreed in RAN4#108 R4-2314323 | Optional with capability signaling |
| 32. NR_MG_enh2 | 32-3 | Inter-RAT measurements without gaps | Inter-RAT LTE measurements without gap when LTE CRS to be measured are fully contained within UE’s active BWP |  | Yes | N/A | UE does not support inter-RAT LTE measurements without gap when LTE CRS to be measured are fully contained within UE’s active BWP | Per-UE | No | Only FR1 | N/A | Already sent LS to RAN2: R4- 2310158 | Optional with capability signaling |

- NonCol_intraB_ENDC_NR_CA

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NonCol_intraB_ENDC_NR_CA | 33-1 | Support of intra-band non-collocated NR CA operation | Indicates the UE supports TDD-TDD intra-band non-collocated NR-CA operation with MTTD/MRTD requirements according to Table 7.5.4.1/Table 7.6.4-2 in 38.133 [5] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in 38.101-1 [2]. And the UE also supports TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in 38.133 and UE RF requirements for intra-band NR-CA except for 7.10A in 38.101-1 [2]. If the capability is not reported, the UE supports TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in 38.133 and UE RF requirements for intra-band NR-CA except for 7.10A in 38.101-1 [2]. |  | Yes | n/a | Intra-band non-collocated NR CA operation is not supported. | Per BC | TDD only | FR1 only |  |  | Optional with capability signaling |

- NR_HST_FR2_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34. NR_HST_FR2_enh | 34-1 | Simultaneous multi-panel operation for train roof-mounted FR2 high power devices | Support of simultaneous multi-panel reception for Rel-18 FR2 PC6 UE | 22-1, 16-2c | Yes | N/A | UE does not support simultaneous multi-panel reception | [Per band or Per FSPC] | N/A | FR2 only | N/A | Agreed in R4-2314297 | Optional with capability signaling |
|  | 34-2 | Intra-band CA and Inter-frequency measurement enhancement | FR2 HST CA and inter-frequency measurement enhancement | 22-1 | Yes | N/A | UE does not support FR2 HST CA and inter-frequency measurement enhancement | [Per-UE] | N/A | FR2 only | N/A | Agreed in R4-2314297 | Optional with capability signaling |
|  | 34-3 | Cross-RRH TCI State Switch | Enhanced MAC CE indication | 22-2 | Yes | N/A | Upon TCI switch, UE does not know if there is a large timing difference between DL signals from two non-collocated RRHs, or the timing difference between DL signals from two RRHs or from the same RRH is smaller than a quarter of CP | [Per-UE] | N/A | FR2 only | N/A | Already sent to RAN2: R4-2314299 | Optional with capability signaling |

- NR_ATG

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 35. NR_ATG | 35-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_demod_enh3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # 36. NR_demod_enh3 | 36-1 | MU-MIMO Interference Mitigation advanced receiver | 1) R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO transmissions for total 2 layers with 2 RX antennas. 2) R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO transmissions for 2,3, 4 total layers with 4 RX antennas. | 3-4 | Yes | N/A | UE not capable of advanced receiver to suppress inter-user inference in MU-MIMO | Per FSPC | N/A | FR1 only |  | Agreed in WF R4-2316915 to introduce optional UE capability for R-ML receiver for MU-MIMO | Optional with capability signaling |
| # 36. NR_demod_enh3 | [36-2] | [Blind detection for co-UE modulation order] | [Blind detection of modulation order for co-scheduled layers which has the same DMRS sequence as the target UE and in each PRB allocated to the target UE only single modulation order is allocated for the co-scheduled layers.] | 36-1 | Yes | N/A | UE not capable of blind detection for modulation order | Per FSPC | N/A | FR1 only |  | If PDSCH demod requirements with blind detection of modulation order are introduced. | Optional with capability signaling |
| # 36. NR_demod_enh3 | 36-3 | DMRS Configuration for MU-MIMO advanced receiver | 1) DMRS configuration supported with R-ML for MU-MIMO 2) Max number of DMRS ports for blind detection on co-scheduled layers with same DMRS sequence as co-scheduled UE. | 36-1 | Yes | N/A | UE indication of number of ports it can detect | Per FSPC | N/A | FR1 only |  |  | Optional with capability signaling |

- NR_pos_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 37. NR_pos_enh2 | 37-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_MC_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 38. NR_MC_enh | 38-1 | UL Tx switching across 3 bands for single-TAG | - UE to indicate support of dynamic UL Tx switching across 3 bands for inter-band UL CA, SUL or inter-band EN-DC - UE to declare the switching period for UL Tx switching across 3 bands for in inter-band EN-DC, inter-band UL CA or SUL band combinations. Switching period value to be from the set (35uSec, 140uSec, 210uSec) |  | yes | no | UE does not support Tx switching across 3 bands for inter-band EN-DC, inter-band UL CA and SUL band combinations, single-TAG | per band pair per BC | No | FR1 only | N/A | Agreed in RAN#98-e RP-223557 | Optional with capability signalling |
|  | 38-2 | UL Tx switching across 4 bands for single-TAG | - UE to indicate support of dynamic UL Tx switching across 4 bands for inter-band UL CA, SUL or inter-band EN-DC - UE to declare the switching period for UL Tx switching across 4 bands for in inter-band EN-DC, inter-band UL CA or SUL band combinations. Switching period value to be from the set (35uSec, 140uSec, 210uSec) |  | yes | no | UE does not support Tx switching across 4 bands for inter-band EN-DC, inter-band UL CA and SUL band combinations, single-TAG | per band pair per BC | No | FR1 only | N/A | Agreed in RAN#98-e RP-223557 | Optional with capability signalling |
|  | 38-3 | UL Tx switching across 2 bands for dual-TAG | - UE to indicate support of dynamic UL Tx switching across 2 bands for inter-band UL CA, SUL or inter-band EN-DC, dual-TAG mode - UE to declare the switching period for UL Tx switching across 2 bands for in inter-band EN-DC, inter-band UL CA or SUL band combinations, dual-TAG mode. Switching period value to be from the set (35uSec, 140uSec, 210uSec) |  | yes | no | UE does not support Tx switching across 2 bands for inter-band EN-DC, inter-band UL CA and SUL band combinations, dual-TAG | per band pair per BC | No | FR1 only | N/A | Agreed in RAN#101 RP-232418 | Optional with capability signalling |
|  | 38-4 | UL Tx switching across 3 bands for dual-TAG | - UE to indicate support of dynamic UL Tx switching across 3 bands for inter-band UL CA, SUL or inter-band EN-DC, dual-TAG mode - UE to declare the switching period for UL Tx switching across 3 bands for in inter-band EN-DC, inter-band UL CA or SUL band combinations, dual-TAG mode. Switching period value to be from the set (35uSec, 140uSec, 210uSec) | 38-1 and 38-2 | yes | no | UE does not support Tx switching across 3 bands for inter-band EN-DC, inter-band UL CA and SUL band combinations, dual-TAG | per band pair per BC | No | FR1 only | N/A | NOTE: Signalling structure is up to RAN2. RAN4 will specify for UL CA and EN-DC for which band combinations DL interruptions are Agreed in RAN#101 RP-232418 | Optional with capability signalling |
|  | 38-5 | UL Tx switching across 4 bands for dual-TAG | - UE to indicate support of dynamic UL Tx switching across 4 bands for inter-band UL CA, SUL or inter-band EN-DC, dual-TAG mode - UE to declare the switching period for UL Tx switching across 4 bands for in inter-band EN-DC, inter-band UL CA or SUL band combinations, dual-TAG mode. Switching period value to be from the set (35uSec, 140uSec, 210uSec) | 38-1 and 38-2 | yes | no | UE does not support Tx switching across 4 bands for inter-band EN-DC, inter-band UL CA and SUL band combinations, dual-TAG | per band pair per BC | No | FR1 only | N/A | NOTE: Signalling structure is up to RAN2. RAN4 will specify for UL CA and EN-DC for which band combinations DL interruptions are Agreed in RAN#101 RP-232418 | Optional with capability signalling |
|  | 38-6 | DL interruption for Tx switching across 3 bands | Capability to indicate that for the band where DL interruption is needed, the RRM interruption requirements defined in RAN4 shall be applied for duplex mode combinations except the combinations - SUL+TDD - TDD+TDD CA with the same UL-DL pattern - TDD+TDD EN-DC with the same UL-DL pattern | 38-1, 38-2, 38-3 | yes | no | UE not reporting this capability means DL interruption is not required | per band pair per BC | No | FR1 only | N/A |  | Optional with capability signalling |
|  | 38-7 | DL interruption for Tx switching across 4 bands | Capability to indicate that for the band where DL interruption is needed, the RRM interruption requirements defined in RAN4 shall be applied for duplex mode combinations except the combinations - SUL+TDD - TDD+TDD CA with the same UL-DL pattern - TDD+TDD EN-DC with the same UL-DL pattern | 38-1, 38-2, 38-3 | yes | no | UE not reporting this capability means DL interruption is not required | per band pair per BC | No | FR1 only | N/A |  | Optional with capability signalling |
|  | 38-8 | Optional UE TX switching capability | - For the band with the number of Tx chain unchanged due to switching, in addition to the baseline UE assumption agreed in RAN4 #104e, introduce optional UE capability to allow UL transmission on the band with the number of Tx chain unchanged (i.e., one Tx chain is maintained on the band) during UL switching. The swithing period is the switching period between band A and band B | 38-1 | yes | no | UE not reporting this capability means UE does not support optional Tx switching capability | per band pair per BC | No | FR1 only | N/A | RAN2 to define how the UE could signal to the network its support of this this optional feature. Agreed in RAN#98-e RP-223557 | Optional with capability signalling |

Additional UE optional capabilities have been proposed by proposed by few companies:

- UL Tx switching across 4 bands: In RAN4 Meeting#107, one company proposed a solution for solving the 4-band UL Tx switching ambiguity issue and improving the corresponding switching period: This proposal relies on introducing two new UE capabilities: The first one is the BandOrdering1T1Tto1T1T through which the UE indicates how the 4 bands (A, B, C, D) are mapped for (1,1,0,0) to (0,0,1,1) switching. The second capability is the preferredBandPairsTxChain which  allocates the UE’s preferred Tx switching band pairs to specific Tx chain. For example, for UL Tx 4-band switching across (A,C,B,D), if the UE declares the two capabilities described above with (A,C) the preferred band pair for TX chain#1 and (B,D) the preferred band band pair of TX chain#2, the following TX switching scenario will be realized for simultaneous dualUL switching: (A—>C) and (B—>D). If UE reports max(Tswitch_A-C, Tswitch_B-D)=SW1, and max(Tswitch_A-D, Tswitch_B-C)=SW2 with SW2>SW1, there will be an improvement of SW2-SW1 on the switching time.

- UL Tx switching acoss 3 and 4 bands with at least one unaffected band : In RAN4 Meeting #108, another company proposed to extend the concept to switching cases where one band is not involved in the switching process (the unaffected band cases) with the goal to reduced the switching period. Supporting that capacity means that the switching period could be  min{max(Tswitch_A-C, Tswitch_B-C), Tswitch_A-B)}for 3-band switching and min{max(Tswitch_A-C, Tswitch_B-C), Tswitch_A-B)} for 4-band switching. There was an issue with this proposal that was not addressed during the offline discussion: If UE declares Tswitch_A-C and Tswitch_B-D = 140uSec, and Tswitch_A-D and Tswitch_B-C = 35uSec, the resulting 4-band switching period would be 35uSec based on the formula above. We would prefer the UE to just declare Tswitch_A-C and Tswitch_C-D as the switching period for the 3-band and 4-band switching respectively.

- NR_Mob_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 39. NR_Mob_enh2 | 39-1 | L1-RSRP measurement on LTM candidate cell | SSB based intra-frequency L1 measurement with RTD between the SSBs of serving cell and neighbour cell on the same carrier is larger than CP length of the corresponding SCS | 45-1 | [Yes] | N/A | [FFS] | [Per-band/BC] | [No] | [No] | N/A | Already sent to RAN1/2 R4-2303308 | Optional with capability signaling |
|  | 39-2 | PDCCH order based RACH to LTM candidate cell | Interruption on DL during PDCCH order RACH transmission to LTM candidate cell | 45-5 | [Yes] | N/A | [FFS] | [Per-FS] | [No] | [No] | N/A | Agreed in RAN4#108bis R4-2317330 | Optional with capability signaling |
|  | 39-3 | Early ASN.1 decoding and validity/compliance check [of LTM candidates] | Early ASN.1 decoding and validity/compliance check [of LTM candidates] | 45-3 | [No] | N/A | 10ms is allowed for ASN.1 decoding and validity/compliance check of target cell configuration | [Per-UE/FR] | [No] | [No] | N/A | Agreed in RAN4#108bis R4-2317330 | Optional without capability signaling |
|  | [39-4] | Number of neighbor cells configured for L1-RSRP measurement | Max number of neighbor cells configured for intra-frequency L1-RSRP measurement Max number of neighbor cells configured for inter-frequency L1-RSRP measurement | 45-1 or 45-1a | Yes | No | NW does not know the max number of cells UE can measure | [Per FS] | No | [No] | N/A |  | Mandatory with capability signaling if UE supports 45-1, or 45-1a |
|  | 39-5 | Measurement validation during connection setup/resume | Improvement on SCell/SCG setup delay based on existing measurement |  | [Yes] | N/A | UE does not support measurement validation during connection setup/resume | [Per-UE] | [No] | [Yes] | N/A |  | Optional with capability signaling |
|  | [39-6] | Enhanced measurement during RRC connection setup/resume | Improvement on SCell/SCG setup delay based on enhanced measurement during RRC connection setup/resume |  | [Yes] | N/A | UE does not support enhanced measurement during RRC connection setup/resume | [Per-UE] | No | [Yes] | N/A |  | Optional with capability signaling |

- NR_NTN_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_NTN_enh | 40-1 | UE architectures for beam steering for band above 10GHz | Support of UE architectures for beam steering in above 10 GHz bands, either fully electronically-steered beam UEs (Type 1 UE) or fully mechanically-steered beam UEs (Type 2 UE) | [Support of above 10GHz band] | Yes | N/A | UE does not support NTN communication on above 10GHz bands. | Per band | FDD only | [FR2 only] | N/A | UE to indicate which architecture is used to support NTN above 10GHz: (1)Type 1: fully electronically-steered beam UE, or (2)Type 2: fully mechanically-steered beam UE. [Agreed in R4-2314447] | Optional with capability signaling |
| 40. NR_NTN_enh | 40-2 | [Beam steering speed for fully mechanically-steered beam UEs (Type 2 UE)] | Support of indicating the beam steering adjustment speed during mobility based measurement in inter-satellite Handover. | 40-x (Type 2 UE) | Yes | N/A | Type 2 UE cannot complete the inter-satellite handover within a time delay requirement in TS38.133 [section 6.x.y] | Per band | FDD only | FR2 only | N/A | UE to indicate which the beam steering adjustment speed during mobility based measurement in inter-satellite Handover, i.e., X degrees/sec. The candidate values for X can be {22, etc} | Optional with capability signaling |
| 40. NR_NTN_enh | [40-3] | [NTN communication with SCS of 120kHz for band above 10GHz] | [Support of NTN communication with SCS of 120kHz for band above 10GHz.] | [Support of above 10GHz band] | [Yes] | [N/A] | [UE does not support NTN communication with SCS of 120kHz for band above 10GHz.] | [Per band] | FDD only | [FR2 only] | N/A | [UE is required to meet the Tx timing requirement for SCS=120kHz in TS38.133 [section 7.x.y].] | [Optional with capability signaling] |
| 40. NR_NTN_enh | 40-4 | DMRS bundling for NTN coverage enhancement | The range of [maxDurationDMRS-Bundling-r17] for NTN bands for which RAN4 has introduced requirements is restricted as follows: - Up to [4] slots if ntn-ScenarioSupport-r17 is present and indicated as NGSO or only the IE field nonTerrestrialNetwork-r17 is present - Up to [16] slots if ntn-ScenarioSupport-r17 is present and indicated as GSO | [maxDurationDMRS-Bundling-r17], [ntn-ScenarioSupport-r17] | Yes | N/A | UE cannot support this feature | Per band |  |  | N/A |  | Optional with capability signalling |

RAN1 has assumed “pre-compensation to keep phase rotation due to timing drift within the phase difference limit can be performed at UE side” in order for the UE to meet the maximum allowable phase difference requirements for DMRS bundling.  Because the side conditions of the phase continuity requirement for DMRS bundling do no allow the UE to perform pre-compensation during the configured DMRS bundling window, the UE has to perform this operation at the beginning of the bundle.  Thus, the range of the maximum duration of DMRS bundling needs to be restricted for NTN bands as follows:

- Up to [4] slots if ntn-ScenarioSupport-r17 is present and indicated as NGSO or only the IE field nonTerrestrialNetwork-r17 is present

- Up to [16] slots if ntn-ScenarioSupport-r17 is present and indicated as GSO

It is proposed to reflect these restrictions in the Rel-18 UE feature list, with the understanding that RAN2 can accommodate the restriction in the capability value within the TS38.306 specification.  Alternatively, if this approach is deemed too difficult, RAN4 can consider introducing new NTN-specific capabilities for DMRS bundling with these restrictions applied.

- NR_cov_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41. # NR_cov_enh2 | 41-1 | Power boosting for QPSK | Indicates whether UE supports power boosting for QPSK at nominal power of advertised power class | N/A | Yes | N/A | UE cannot support power boosting for QPSK | Per band | TDD only | FR1 only | N/A | A configuration signal from BS to UE is needed to activate this boosting option | Optional with capability signalling |
|  | 41-2 | Power boosting for QPSK at reduced power | Indicates whether UE supports power boosting for QPSK at lower power level than advertised power class i.e. when ΔPPowerClass = 3 dB. | N/A | Yes | N/A | UE cannot support power boosting for QPSK for ΔPPowerClass = 3 dB | Per band | TDD only | FR1 only | N/A | A configuration signal from BS to UE is needed to activate this boosting option (same signal as for X-1) | Optional with capability signalling |

- Netw_Energy_NR

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 42. # Netw_Energy_NR | 42-1 | FR1 Inter-band SSB-less SCell operation | Support of SSB-less SCell operation for inter-band CA for FR1 and co-located cells |  | Yes | N/A | UE does not support SSB-less SCell operation for inter-band CA for FR1 and co-located cells, and network cannot configure FR1 inter-band SSB-less SCell to the UE for CA. | Per FSPC (per CC per band per band combination) | No | FR1 only | N/A | UE is required to meet the RRM requirement of FR1 inter-band SSB-less SCell operations in TS38.133. | Optional with capability signaling |

- NR_DualTxRx_MUSIM

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. # NR_DualTxRx_MUSIM | 43-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 44. XX | 44-1 |  |  |  |  |  |  |  |  |  |  |  |  |
| 44. XX |  |  |  |  |  |  |  |  |  |  |  |  |  |

- NR_SL_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 45. # NR_SL_enh2 | 45-1 |  |  |  |  |  |  |  |  |  |  |  |  |

- R18_3Tx_NR_CA_ENDC-core

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| XX. # NR_SL_enh2 | XX-1 | TxD capability in an inter-band combination | Per band per combination indication on the support of TxD capability | N/A | Yes | N/A | TxD is not supported in an inter-band combination | Per band per BC | No | FR1 only | N/A |  | Optional with capability signalling |