# 3GPP TSG-RAN WG4 Meeting #109	R4-2318724

# Chicago, US, 13– 17 Nov, 2023

Source: 	Huawei, HiSilicon

Title:	TP for TR 38.880: DC_41A_n78A with 3Tx

Agenda Item:	7.29.2

Document for:	Approval

# Introduction

This contribution provides a text proposal on DC band combination of LTE bands B41 (1Tx) with NR band n78 (2Tx) for TR 38.880.

# Text Proposal

----- Start of TP -----

## 6.5	DC_41_n78

### 6.5.1	Operating bands for EN-DC

Table 6.5.1-1:  EN-DC band combination of band 41+n78

| EN-DC Band combination | Bands |
| --- | --- |
| DC_41A_n78A | 41, n78 |

### 6.5.2	Maximum output power for inter-band EN-DC

Table 6.5.2-1: UE Power Class for 3Tx inter-band EN-DC

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_41A_n78A5,6 | 26 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: (Void in this TR) NOTE 3: (Void in this TR) NOTE 4: (Void in this TR) NOTE 5: The UE supports PC3 in E-UTRA band, and supports PC2 with UL MIMO in NR band NOTE 6: The UE supports PC3 in E-UTRA band, and supports PC3 with UL MIMO in NR band |  |  |  |  |

### 6.5.3	MSD scenario studies

Table 6.5.3-1 summarizes frequency ranges where harmonics mixing occur for DC_41_n78

Table 6.5.3-1: MSD scenarios for DC_41_n78 with total power class 2

|  | Aggressor Tx band | Victim Rx band | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | 2nd harmonic of n78 | 3rd harmonic of 41 Rx | No, but can reuse from DC_41_n77 |
| Cross band leakage | n78 UL | 41 DL | No, but can reuse from DC_41_n77 |

### 6.5.4	REFSENS requirements

The MSD due to harmonic mixing and the cross band isolation can be taken from a similar band combination such DC_41A_n77A for PC2 with 2Tx, as given below.

Table 6.5.4-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | 41 | 10 | 15 | 12 (RBstart=0) | 5 | 19.4 | NOTE 4 | UL2/DL3 |
| n78 | 41 | 20 | 15 | 50 (RBstart=0) | 20 | 19.4 | NOTE 4 | UL2/DL3 |
| NOTE 4: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |

Table 6.5.4-2: Reference sensitivity exceptions (MSD) due to cross band isolation and uplink/downlink configurations for PC2 EN-DC in NR FR1

| UL band | DL band | UL Fc (MHz) | UL BW (MHz) | SCS of UL band (kHz) | UL RB Allocation LCRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n78 | 411 | 3350 | 100 | 30 | 270 (RBstart=0) | 2687.5 | 5 | 6.5 | >ACLR2 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. |  |  |  |  |  |  |  |  |  |

----- End of TP -----