3GPP TSG-RAN WG4 Meeting #109              				                 R4-2318808

# Chicago, US, November 13 – 17, 2023

Source:	LG Electronics

Title:	UE RF requirements of SL CA

Agenda item:	8.30.2.3

Document for:	Discussion

Introduction

In this paper, we provide our views on the remaining issues of UE RF requirements and SL CA MPR/A-MPR simulation results for intra-band SL CA in Band n47 based on the WF[1].

Discussion

MPR for SLCA PSSCH/PSCCH

For SL CA PSSCH/PSCCH MPR, the existing RB allocations which are defined in the NR contiguous CA MPR can be reused.

- Inner/Outer RB allocation in NR C-CA with contiguous RB allocations

- Inner/Outer1/Outer2 RB allocation in NR C-CA with non-contiguous RB allocations

From the simulation results [3],  it was observed that MPR between contiguous RB allocation and non-contiguous RB allocation is very different, like MOR of NR intra-band contiguous CA. For example, Outer2 MPR of non-contiguous RB allocation is 2.5dB ~ 4.5 higher than Outer MPR of contiguous RB allocation. Therefor, both contiguous RB allocation and non-contiguous RB allocation should be considered.

Based on WF[1] and the simulation results [3],  we propose as follow.

Proposal 1: Specify SL C-CA PSSCH/PSCCH MPR separately for both contiguous RB allocation and non-contiguous RB allocation.

Proposal 2: For SL C-CA PSSCH/PSCCH MPR for contiguous RB allocation, consider Table 2-1.

Table 2-1: PSSCH/PSCCH MPR for SL Contiguous CA with Contiguous RB allocations

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 5.0 |
|  | 16QAM | ≤ 3.0 | ≤ 5.0 |
|  | 64QAM | ≤ 4.5 | ≤ 5.0 |
|  | 256QAM | ≤ 6.5 | ≤ 7.0 |

Proposal 3: For SL C-CA PSSCH/PSCCH MPR for non-contiguous RB allocation, consider Table 2-2.

Table 2-2 PSSCH/PSCCH MPR for SL Contiguous CA with Non-contiguous RB allocations

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 5.0 | ≤ 9.5 |
|  | 16QAM | ≤ 3.0 | ≤ 5.0 | ≤ 9.5 |
|  | 64QAM | ≤ 4.5 | ≤ 5.0 | ≤ 9.5 |
|  | 256QAM | ≤ 7.0 | ≤ 7.0 | ≤ 9.5 |

MPR for SLCA PSFCH

Based on WF[1] and the simulation results [3],  we propose as follow.

Proposal 4: For SL C-CA PSFCH MPR, consider Table 2-3.

Table 2-3 PSFCH MPR for SL Contiguous CA

| MPR for ratio (R) in bandwidth class B(dB) |  |  |
| --- | --- | --- |
| R ≤ 0. 33 | 0.33 < R ≤ 0. 55 | 0.55 < R ≤ 1.0 |
| ≤3.5 | ≤9.0 | ≤13.0 |
| Here, R = NGap/(NRB1+NRB2+ NGBchannel_CC1+ NGBchannel_CC2) |  |  |

Here, R is ratio of NGap/(NRB1+NRB2+ NGBchannel_CC1+ NGBchannel_CC2).

- R = NGap/(NRB1+NRB2+ NGBchannel_CC1+ NGBchannel_CC2)

  - NGap is the gap RB amount from CC1 RBstart  to CC2 RBend for SL contiguous CA when a single PSFCH or contiguous and non-contiguous allocation simultaneous PSFCHs is transmitted in each CC.

  - NGBchannel_CC1 and NGBchannel_CC2 is the number of RB which corresponds to the minimum guardbands for CC1 and CC2, respectively, which have been calculated using the following equation:

NGBchannel =  GBchannel / (SCS x 12)

GBchannel = (BWChannel - NRB x SCS x 12) / 2 - SCS/2,

where NRB are from Table 5.3.2-1 in TS38.101-1, and BWChannel is the channel bandwidth for each CC.

Figure 2-1 shows NGap, NRB1, NRB2, NGBchannel_CC1, and NGBchannel_CC2.

Figure 2-1: Parameters related to Ratio of R

MPR for SLCA S-SSB

Based on WF[1] and the simulation results [3],  we propose as follow.

Proposal 5: Consider Inner/Outer1/Outer2 RB allocation of NR C-CA with non-contiguous RB allocation for SL C-CA S-SSB MPR.

Proposal 6: For SL intra-band C-CA S-SSB MPR, consider Table 2-4.

Table 2-4 S-SSB MPR for SL Contiguous CA

| MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- |
| Inner | Outer1 | Outer2 |
| ≤3.5 | ≤9.0 | ≤13.0 |

A-MPR NS_value for SLCA

Based on WF [1], NS_52 was agreed to be considered for SL CA A-MPR. Like NR CA, ‘SLCA_NS_52’ can be proposed for SLCA, instead of ‘NS_52’.

Proposal 7: Use ‘SLCA_NS_52’ for SL intra-band C-CA A-MPR as Table 2-5.

Table 2-5: Mapping of network signaling label

| Sidelink CA operating band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| SL_n47 | SLCA_NS_01 |  | SLCA_NS_52 |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 |  |  |  |  |  |  |  |  |

A-MPR for SLCA PSSCH/PSCCH

Based on WF[1] and the simulation results [3],  we propose as follow.

Proposal 8: Specify SL C-CA PSSCH/PSCCH A-MPR separately for both contiguous RB allocation and non-contiguous RB allocation.

Proposal 9: For SL-CA NS_52 PSSCH/PSCCH A-MPR for contiguous RB allocation, consider Table 2-6.

Table 2-6 SL CA NS_52 PSSCH/PSCCH A-MPR for SL Contiguous CA with Contiguous RB allocations

| Modulation |  | A-MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ 7.0 | ≤ 8.5 |
|  | 16QAM | ≤ 7.0 | ≤ 8.5 |
|  | 64QAM | ≤ 7.0 | ≤ 8.5 |
|  | 256QAM | ≤ 7.0 | ≤ 8.5 |

Proposal 10: For SL-CA NS_52 PSSCH/PSCCH A-MPR for non-contiguous RB allocation, consider Table 2-7.

Table 2-7 SL CA NS_52 PSSCH/PSCCH A-MPR for SL Contiguous CA with Non-contiguous RB allocations

| Modulation |  | A-MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 8.0 | ≤ 13.5 |
|  | 16QAM | ≤ 3.0 | ≤ 8.0 | ≤ 13.5 |
|  | 64QAM | ≤ 4.5 | ≤ 8.0 | ≤ 13.5 |
|  | 256QAM | ≤ 7.0 | ≤ 8.0 | ≤ 13.5 |

A-MPR for SLCA PSFCH

Based on WF[1] and the simulation results [3],  we propose as follow.

Proposal 11: For SL-CA NS_52 PSFCH A-MPR, consider Table 2-8.

Table 2-8 SL CA NS_52 PSFCH A-MPR for SL Contiguous CA

| Modulation |  | A-MPR for ratio (R) in bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | R ≤0. 1 | 0.1 < R ≤ 0. 55 | 0.55 < R ≤ 1.0 |
| CP-OFDM | QPSK | ≤4.0 | ≤17.0 | ≤19.0 |
| Here, R = NGap/(NRB1+NRB2+ NGBchannel_CC1+ NGBchannel_CC2) |  |  |  |  |

A-MPR for SLCA S-SSB

Based on WF[1] and the simulation results [3],  we propose as follow.

Proposal 12: Consider Inner/Outer1/Outer2 RB allocation of NR C-CA with non-contiguous RB allocation for SL C-CA S-SSB A-MPR.

Proposal 13: For SL-CA NS_52 S-SSB A-MPR, consider Table 2-9.

Table 2-9 SL CA NS_52 S-SSB A-MPR for SL Contiguous CA

| A-MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- |
| Inner | Outer1 | Outer2 |
| ≤ 9.0 | ≤ 13.0 | ≤ 16.5 |

Conclusion

In this contribution, we proposed the MPR/A-MPR requirements for SL intra-band contiguous CA based on WF[1] and simulation results [3].

- MPR for SLCA PSSCH/PSCCH

Proposal 1: Specify SL C-CA PSSCH/PSCCH MPR separately for both contiguous RB allocation and non-contiguous RB allocation.

Proposal 2: For SL C-CA PSSCH/PSCCH MPR for contiguous RB allocation, consider Table 2-1.

Table 2-1: PSSCH/PSCCH MPR for SL Contiguous CA with Contiguous RB allocations

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 5.0 |
|  | 16QAM | ≤ 3.0 | ≤ 5.0 |
|  | 64QAM | ≤ 4.5 | ≤ 5.0 |
|  | 256QAM | ≤ 6.5 | ≤ 7.0 |

Proposal 3: For SL C-CA PSSCH/PSCCH MPR for non-contiguous RB allocation, consider Table 2-2.

Table 2-2 PSSCH/PSCCH MPR for SL Contiguous CA with Non-contiguous RB allocations

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 5.0 | ≤ 9.5 |
|  | 16QAM | ≤ 3.0 | ≤ 5.0 | ≤ 9.5 |
|  | 64QAM | ≤ 4.5 | ≤ 5.0 | ≤ 9.5 |
|  | 256QAM | ≤ 7.0 | ≤ 7.0 | ≤ 9.5 |

- MPR for SLCA PSFCH

Proposal 4: For SL C-CA PSFCH MPR, consider Table 2-3.

Table 2-3 PSFCH MPR for SL Contiguous CA

| MPR for ratio (R) in bandwidth class B(dB) |  |  |
| --- | --- | --- |
| R ≤ 0. 33 | 0.33 < R ≤ 0. 55 | 0.55 < R ≤ 1.0 |
| ≤3.5 | ≤9.0 | ≤13.0 |
| Here, R = NGap/(NRB1+NRB2+ NGBchannel_CC1+ NGBchannel_CC2) |  |  |

Here, R is ratio of NGap/(NRB1+NRB2+ NGBchannel_CC1+ NGBchannel_CC2).

- R = NGap/(NRB1+NRB2+ NGBchannel_CC1+ NGBchannel_CC2)

  - NGap is the gap RB amount from CC1 RBstart  to CC2 RBend for SL contiguous CA when a single PSFCH or contiguous and non-contiguous allocation simultaneous PSFCHs is transmitted in each CC.

  - NGBchannel_CC1 and NGBchannel_CC2 is the number of RB which corresponds to the minimum guardbands for CC1 and CC2, respectively, which have been calculated using the following equation:

NGBchannel =  GBchannel / (SCS x 12)

GBchannel = (BWChannel - NRB x SCS x 12) / 2 - SCS/2,

where NRB are from Table 5.3.2-1 in TS38.101-1, and BWChannel is the channel bandwidth for each CC.

- MPR for SLCA S-SSB

Proposal 5: Consider Inner/Outer1/Outer2 RB allocation of NR C-CA with non-contiguous RB allocation for SL C-CA S-SSB MPR.

Proposal 6: For SL intra-band C-CA S-SSB MPR, consider Table 2-4.

Table 2-4 S-SSB MPR for SL Contiguous CA

| MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- |
| Inner | Outer1 | Outer2 |
| ≤3.5 | ≤9.0 | ≤13.0 |

- A-MPR for SLCA

Proposal 7: Use ‘SLCA_NS_52’ for SL intra-band C-CA A-MPR as Table2-5.

Table 2-5: Mapping of network signaling label

| Sidelink CA operating band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| SL_n47 | SLCA_NS_01 |  | SLCA_NS_52 |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 |  |  |  |  |  |  |  |  |

- A-MPR for SLCA PSSCH/PSCCH

Proposal 8: Specify SL C-CA PSSCH/PSCCH A-MPR separately for both contiguous RB allocation and non-contiguous RB allocation.

Proposal 9: For SL-CA NS_52 PSSCH/PSCCH A-MPR for contiguous RB allocation, consider Table 2-6.

Table 2-6 SL CA NS_52 PSSCH/PSCCH A-MPR for SL Contiguous CA with Contiguous RB allocations

| Modulation |  | A-MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ 7.0 | ≤ 8.5 |
|  | 16QAM | ≤ 7.0 | ≤ 8.5 |
|  | 64QAM | ≤ 7.0 | ≤ 8.5 |
|  | 256QAM | ≤ 7.0 | ≤ 8.5 |

Proposal 10: For SL-CA NS_52 PSSCH/PSCCH A-MPR for non-contiguous RB allocation, consider Table 2-7.

Table 2-7 SL CA NS_52 PSSCH/PSCCH A-MPR for SL Contiguous CA with Non-contiguous RB allocations

| Modulation |  | A-MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 8.0 | ≤ 13.5 |
|  | 16QAM | ≤ 3.0 | ≤ 8.0 | ≤ 13.5 |
|  | 64QAM | ≤ 4.5 | ≤ 8.0 | ≤ 13.5 |
|  | 256QAM | ≤ 7.0 | ≤ 8.0 | ≤ 13.5 |

- A-MPR for SLCA PSFCH

Proposal 11: For SL-CA NS_52 PSFCH A-MPR, consider Table 2-8.

Table 2-8 SL CA NS_52 PSFCH A-MPR for SL Contiguous CA

| Modulation |  | A-MPR for ratio (R) in bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | R ≤0. 1 | 0.1 < R ≤ 0. 55 | 0.55 < R ≤ 1.0 |
| CP-OFDM | QPSK | ≤4.0 | ≤17.0 | ≤19.0 |
| Here, R = NGap/(NRB1+NRB2+ NGBchannel_CC1+ NGBchannel_CC2) |  |  |  |  |

- A-MPR for SLCA S-SSB

Proposal 12: Consider Inner/Outer1/Outer2 RB allocation of NR C-CA with non-contiguous RB allocation for SL C-CA S-SSB A-MPR.

Proposal 13: For SL-CA NS_52 S-SSB A-MPR, consider Table 2-9.

Table 2-9 SL CA NS_52 S-SSB A-MPR for SL Contiguous CA

| A-MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- |
| Inner | Outer1 | Outer2 |
| ≤ 9.0 | ≤ 13.0 | ≤ 16.5 |

- References

[1] R4-2317731, “WF on NR_SL_enh2_UERF_part3”, Huawei

[2] R4-2315963, “TR 38.786 v1.1.0”

[3] R4-2315532, “UE RF requirements of SL CA”, LG Electronics