# 3GPP TSG-RAN WG4 Meeting # 109	R4-2318921

# Chicago, US, November 13 – 17, 2023

Agenda Item:	8.13.2

Source: 	CMCC

Title: 	TP for TR 38.876 to add non-synchronized calibration part for ATG TR

Document for:	Approval

- Introduction

In last meeting, to align isolation distance derived from non-synchronized co-existence scenario. RAN4 calibrate non-synchronized scenario with CL assuming 5km isolation between ATG BS and nearest TN BS. Detailed calibration results are listed as in [2].

This TP is used to add this non-synchronized co-existence calibration results into final TR.

- Reference

- R4-2315904, TR 38.876 ATG 0.6.0, RAN4 #108bis meeting.

- R4-2315192, Collection of non-synchronized scenario calibration data, CMCC, RAN4 #108bis meeting

- TP to TR 38.876

## <<Start of Change for TR 38.876>>

Annex	A:	Calibration results of synchronized operation

A.1	Calibration assumptions

The calibration assumptions is the same as listed in clause 6.2 and 6.3.

A.2	Calibration results at 2GHz and 4GHz

Four kinds of  performance metrics are used for synchronized operation calibration purpose which encompass Coupling Loss (CL), SINR without adajcent channel interference, SINR with adjacent channel interference from synchronized intra-system and SINR CDF with adjacent channel interference from synchronized inter-system. Following show detailed explanation for each performance metric in calibration excel file.

- CL: coupling loss between transmitter and receiver

- SINR CDF without adjacent channel interference: Here the SINR=S/(N+IICI) where IICI is the co-channel interference

- SINR CDF with adjacent channel interference from synchronization intra-system: Here the SINR=S/(N+IICI + IACI).

  - For TN, IACI is from synchronized TN network using adjacent channel

  - For ATG, due to we only consider one ATG BS and UE, IACI and IICI are both 0

- SINR CDF with adjacent channel interference from synchronization inter-system: Here the SINR=S/(N+IICI + IACI).

  - For TN, IACI is from synchronized ATG network using adjacent channel

  - For ATG, IACI is from synchronized TN network using adjacent channel and IICI is 0

The below linked sheet captures the data of calibration co-existence simulation study for all synchronized scenarios. More details on the assumptions can be found in the Cover page of the sheet.

Annex B:	Calibration results of non-synchronized operation

B.1	Calibration assumptions

The calibration assumptions is the same as listed in clause 6.2 and 6.3.

B.2	Calibration results at 2GHz and 4GHz

For non-synchronized operation, the performance metric to be calibrated would be the Cross coupling loss between ATG BS and TN BS with 0 azimuth angle (TN and ATG BS on the same horizontal line). besides, 5 km isolation distance is assumed between ATG BS and nearest TN BS.

The below linked sheet captures the data of calibration co-existence simulation study for all non-synchronized scenarios.

## <<End Change for TR 38.858>>