3GPP TSG-RAN WG4 Meeting # 109	R4-2318979

Chicago, US, November 13 – 17, 2023

Source: 	vivo

Title: 	Discussion on remaining issue on FR2 UL 256QAM

Agenda Item:	8.6.2

Document for:	Approval

- Introduction

In the last meeting, the MPR table for single CC was finished and the only remaining issue is the MPR for intra-band CA under 256QAM. In this contribution, we provide our views on this issue.

- Discussion

In the last meeting, we agreed that for 28GHz, the MPR for 256QAM is 3 dB larger than 64QAM and for 39GHz, 1 dB additional MPR is allowed. In [1], one of proposals tried to apply same rules to CA MPR. To prove whether this proposal is feasible, we try to analyze whether the rationale to define CA MPR depend on the modulation order.

In previous release, the MPR for intra-band CA is defined based on following formula [2]:

MPRcontiguous=MPRsingleCCMaxCCBW+∆MPRnCC+∆MPRBW

The ∆MPRnCC is increased MPR due the PAPR, as shown in Table I

Table I: ∆MPRnCC  as a function of Waveform and Modulation Order in [2]

| Wave-type | Mod | ∆MPRnCC, dB |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| DFT-s-OFDM | BPSK | 0 | 1 | 1.5 | 1.5 | 2 | 2.5 | 2.5 | 3 |
|  | QPSK | 0 | 1 | 1.5 | 1.5 | 2 | 2.5 | 2.5 | 3 |
|  | 16QAM | 0 | 1 | 1.5 | 1.5 | 2 | 2.5 | 2.5 | 3 |
|  | 64QAM | 0 | 1 | 1.5 | 1.5 | 2 | 2.5 | 2.5 | 3 |
| CP-OFDM | QPSK | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 16QAM | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 64QAM | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

The ∆MPRBW reflect the increased MPR due to the droop effect for larger aggregated BW and following formula is used:

∆MPRBW= .1875*AggBWMaxccBW

We can observe that both ∆MPRnCC and ∆MPRBW are not related to the modulation type, so for 256QAM, similar delta value between different aggregated BW can be used.

Observation: Both ∆MPRnCC and ∆MPRBW for intra-band contiguous CA are not dependent on modulation type.

Based on the observation above, we agree that same rules can be applied to intra-band contiguous CA case.

Proposal: For intra-band CA, same delta value compared to 64QAM can be used, i.e., 3 dB for 28GHz and 4 dB for 39GHz.

A draft CR is provided in [3].

- Conclusion

In this contribution, we provide our views on CA MPR for 256QAM.

Observation: Both ∆MPRnCC and ∆MPRBW for intra-band contiguous CA are not dependent on modulation type.

Proposal: For intra-band CA, same delta value compared to 64QAM can be used, i.e., 3 dB for 28GHz and 4 dB for 39GHz.

- References

- R4-2315054, MPR for FR2 UL 256QAM, RAN4#108bis

- R4-1811487, Update FR2 CA MPR, Qualcomm Incorporated, RAN4#88

- R4-2318980, draft CR for introducing CA MPR for FR2-1 UL 256 QAM	, vivo, RAN4#109