3GPP TSG-RAN WG4 Meeting #109	R4-2319013

Chicago, US, November 13 – 17, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM impact due to new BS signaling for intra-band non-collocated CA

Agenda item:	8.11.2

Document for:	Discussion

Introduction

At RAN4#108bis meeting, it was agreed to take the new BS signaling into RRM specification implementation as cited below [1]. In this paper, we are discussing what RRM requirements need to be adapted based on the new BS signaling.

Issue 1-1: New RRC signalling for NR-CA

- Agreement:

  - RAN4 shall take the newly introduced BS signaling into RRM specification implementation for UE supporting [intraBandNRCA-NonCollocated-r18].

  - Discuss the wording for the RRM CR based on RAN2 design on the BS signaling.

Issue 1-2: New RRC signalling for EN-DC

- Agreement:

  - For EN-DC, wait for the conclusion from RF session.

Discussion

At RAN4#106bis, the draft BigCR on RRM requirements for intra-band non-collocated scenarios was endorsed in [2]. The UE type has been considered as a static UE capability and RRM requirements are defined based on the UE capability indication as exampled below on MRTD.

### 7.6.4	Minimum Requirements for NR Carrier Aggregation

For intra-band contiguous CA, only co-located deployment is applied. For intra-band non-contiguous NR carrier aggregation, the UE not capable of [intraBandNRCA-NonCollocated-r18] shall be capable of handling at least a relative receive timing difference between slot timing of different carriers to be aggregated at the UE receiver as shown in Table 7.6.4-1 below.

For FR1 intra-band non-contiguous NR carrier aggregation, the UE shall be capable of handling at least a relative receive timing difference as shown in 7.6.4-2, between slot timing of different FR1 carriers to be aggregated at the UE receiver provided that UE indicates that it is capable of [intraBandNRCA-NonCollocated-r18].

Observation #1: In RRM, the UE type has been considered as UE capability and RRM requirements are defined based on the UE capability indication.

However, from RAN#101 conclusion, a Type 2 UE may be configured to switch between Type 1 and Type 2 with a new BS signalling. This means, for a Type 2 UE capable of [intraBandNRCA-NonCollocated-r18], the UE may operate either Type 1 or Type 2 based on the new BS signaling.

Observation #2: From RAN#101 conclusion, a Type 2 UE indicating the capability of [intraBandNRCA-NonCollocated-r18] may operate either Type 1 or Type 2 based on the new BS signaling.

As a Type 2 UE may operate Type 1 architecture based on the new BS signaling, the RRM requirement shall not consider UE capability [intraBandNRCA-NonCollocated-r18] alone, instead it is dependent on the UE Type in operation based on the new BS signaling. For instance, if a Type 2 UE is configured to apply the Type 1 capability requirement via the BS signaling, the RRM requirements for Type 1 UE shall apply. Therefore, the MRTD/MTTD requirements need to be adapted based on the UE type in operation as indicated by the BS signaling.

Proposal 1: The MRTD/MTTD requirements need to be adapted based on if Type 1 or Type 2 capability requirement is to be applied as indicated by the new BS signaling.

Similarly, the interruption requirements have been defined for intra-band and inter-band scenarios separately. They also need to be adapted based on if Type 1 or Type 2 capability requirement is to be applied as indicated by the new BS signaling. Such adaptation is allowed if the UE indicates the capability of [intraBandNRCA-NonCollocated-r18].

Proposal 2: Interruption requirements need to be adapted based on if Type 1 or Type 2 capability requirement is to be applied as indicated by the new BS signaling.

Based on the proposals, we provide the draftCR illustrating the impact on MRTD/MTTD due to the introduction of new BS signaling for intra-band non-collocated NR-CA [3].

- Conclusion

This contribution discusses the potential impact to RRM requirements for non-collocated FR1 intra-band scenarios due to the new BS signaling agreed in RAN#101 and further agreements in RAN2#123bis. The observations and proposals are summarized as below:

Observation #1: In RRM, the UE type has been considered as UE capability and RRM requirements are defined based on the UE capability indication.

Observation #2: From RAN#101 conclusion, a Type 2 UE indicating the capability of [intraBandNRCA-NonCollocated-r18] may operate either Type 1 or Type 2 based on the new BS signaling.

Proposal 1: The MRTD/MTTD requirements need to be adapted based on if Type 1 or Type 2 capability requirement is to be applied as indicated by the new BS signaling.

Proposal 2: Interruption requirements need to be adapted based on if Type 1 or Type 2 capability requirement is to be applied as indicated by the new BS signaling.

References

- RP-232692, LS on signaling support for intra-band non-collocated NR-CA,EN-DC, KDDI

- R4-2306399, Draft Big CR on RRM requirements for support of intra-band non-collocated EN-DC/NR-CA deployment, Huawei

- R4-2319014, draftCR on MRTD/MTTD due to UE type switching, Nokia, Nokia Shanghai Bell