3GPP TSG RAN WG4 Meeting #109	R4-2319046

Chicago, US, November 13 – November 17, 2023

Agenda item:			8.7.2.6

Source:	vivo

Title:	Remaining issues on RTD requirements for FR2 multi-Rx

Document for:	Discussion

- Introduction

In the RAN4#108 meeting, there were continued discussions on the RRM enhancement for RLM and BFD/CBD for FR2 multi-Rx chain DL reception. The agreements and open issues are captured in the WF [1].

Issue 3-1-1: Whether L1-RSRP GBBR requirements should be extended to RTD >CP if the UE supports RTD>CP capability - Way forward: Companies are requested to bring further analysis Issue 3-1-1: How to capture RTD applicability conditions in multi-RX WI - Way forward: Further discuss in next meeting. - Option 1: This condition is mentioned for each relevant multi-RX RRM requirement such as scheduling/measurement restriction, dual TCI state switching, L1-RSRP for GBBR. - Option 2: In clause 3.6 “Applicability of requirements in this specification version,” it is stated that RTD < CP as a condition for the relevant requirements to apply

In this contribution, we further provide our views on remaining open issues for RTD requirements for FR2 multi-Rx chain DL reception.

- Discussion

Issue 3-1-1: Whether L1-RSRP GBBR requirements should be extended to RTD >CP if the UE supports RTD>CP capability

In the RAN4#108 meeting, following agreements on RTD for multi-Rx WI were reached.

- Whether to consider RTD larger than CP in multi-RX WI - The requirements defined for multi-Rx are only applicable for receive timing difference (RTD) less than CP in Rel-18 multi-Rx WI. - RAN4 understands it is important to support RTD>CP and recommend to consider in future release.

Thus, the assumption of RTD has already been concluded. There is no need to further discuss. Furthermore, there is no UE capability of supporting RTD>CP for multi-Rx multi-TRP operation being introduced in this release.

The receive timing difference has to be within CP for supporting 4-layer MIMO with joint processing. It would be main use case for Rel-18 WI as the demodulation performance requirements are mainly specified for the two cases.

Proposal 1: L1-RSRP measurement requirements are defined under the assumption of RTD <CP.

Issue 3-1-1: How to capture RTD applicability conditions in multi-RX WI

It is better to have a general applicability description in terms of RTD for multi-Rx. If it is stated in different sections, it will be very difficult for future maintenance.

Proposal 2: A dedicated clause 3.6.X is used to capture applicability of requirements for multi-Rx.

- Summary

In this contribution, we further provided views on remaining open issues for RLM and BFD/CBD requirements for FR2 multi-Rx chain DL reception. Following proposals and observations are present.

Proposal 1: L1-RSRP measurement requirements are defined under the assumption of RTD <CP.

Proposal 2: A dedicated clause 3.6.X is used to capture applicability of requirements for multi-Rx.

- References

- R4-2317427	WF on FR2_multiRx_part2, Ericsson