3GPP TSG-RAN WG4 Meeting # 109														R4-2319567

Chicago, USA, November 13 –  November 17, 2023

Source:				Ericsson

Title:					NTN enhancements – Coexistence simulation results

Agenda item:		8.26.2

Document for:		Discussion

# Introduction

This document discusses the first outcomes of the NTN coexistence simulation results.

# Discussion

## Simulation summary

Based on our simulations’ results, the following Table 1 (NF=2.5dB) and Table 2 (NF=6dB) give a summary of the needed ACIR values (5% throughput loss threshold) for the different scenarios, elevation angles and NTN UE types considered in the simulation assumptions ([2]). Detailed figures are provided in Annex and our simulation results values are shared in [3].

Important note: The yellow cells highlight results which have been extrapolated from simulations with 5-10dB ACIR step, this to save simulation time. This means that those values are very rough estimate, simulations with smaller ACIR step size are needed to get a more reliable value. We were not able to do so before this meeting’s deadline due to the very short time between meetings and the very late approval of simulations assumptions.

Table 1: ACIR values (5% throughput loss) summary for all scenarios and use cases – NF = 2.5dB

| Scenario | Elevation angle: 90 deg. |  |  |  |  |  |  |  | Elevation angle: 25 deg. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | GEO - mobile |  | GEO |  | LEO600 |  | LEO1200 |  | GEO - mobile |  | GEO |  | LEO600 |  | LEO1200 |  |
|  | Avg | 5 perc. | Avg | 5 perc. | Avg | 5 perc. | Avg | 5 perc. | Avg | 5 perc. | Avg | 5 perc. | Avg | 5 perc. | Avg | 5 perc. |
| 1 | <5 | <5 | <5 | <5 | <5 | <5 | <5 | <5 | <5 | 9 | <5 | 10 | <5 | 17 | 3 | 20 |
| 2 |  |  | <5 | <5 | <5 | <5 | <5 | <5 |  |  | <5 | NA | <5 | <5 | <5 | NA |
| 3 |  |  | <5 | <5 | <5 | <5 |  |  |  |  | <5 | <5 | <5 | <5 |  |  |
| 4 |  |  | 15 | 18 | 14 | 21 |  |  |  |  | 22 | NA | ~25 | NA |  |  |
| 5 | <38 | 41 | <38 | 39 | <40 | 43.5 | <38 | 38 | 41 | NA | 44? | NA | >40 | NA | >40 | NA |
| 6 |  |  | <5 | <5 | <5 | <5 | <5 | <5 |  |  | NA | NA | <5 | <5 | <5 | <5 |
| 7 |  |  | <5 | <5 | <5 | <5 |  |  |  |  | <5 | <5 | <5 | <5 |  |  |
| 8 |  |  | <5 | 10 | <5 | 15 |  |  |  |  | 7 | NA | 10 | NA |  |  |

Table 2: ACIR values (5% throughput loss) summary for all scenarios and use cases – NF = 6dB

| Scenario | Elevation angle: 90 deg. |  |  |  | Elevation angle: 25 deg. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LEO600 |  | LEO1200 |  | LEO600 |  | LEO1200 |  |
|  | Avg | 5 perc. | Avg | 5 perc. | Avg | 5 perc. | Avg | 5 perc. |
| 1 |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |
| 5 | 32 | 38 | 34 | 36 | >40 | NA | >40 | NA |
| 6 |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |

We could then make the following observations:

Observation1: Scenarios 2, 3, 6 and 7 don’t cause any coexistence issue for all considered types of satellites and NTN UEs, at 90- or 25-degrees elevation angles.

Observation2: For scenario 1, at 25-degrees elevation angle, 20dB ACIR would be needed to guarantee coexistence.

Observation3: For scenario 4, 20-25 dB ACIR would be needed to guarantee coexistence.

Observation4: Scenario 5 is the problematic scenario for this coexistence study, requiring 40dB or more ACIR value to have NTN and TN coexisting properly.

Observation5: For scenario 8, 15-20 dB ACIR would be needed to guarantee coexistence.

Observation6: For scenario 5  with fixed VSAT and 6 dB NF few dB ACIR less are required to guarantee coexistence.

Observation7: At 25 degrees elevation angle, many 5-percentile results are not relevant, most likely due to the un-sufficient link budget. Further investigation would be needed to better understand if there is any impact on the conclusion.

Observation8: Further results would be needed to confirm this, but mobile VSAT seems to require few dB more ACIR than fixed VSAT, at least in scenario 5.

## Preliminary conclusion

From the observation 1, considering scenarios 6 and 7, the SAN ACLR value could be relaxed. Keeping in mind an efficient use of the spectrum, not relaxing too much SAN ACLR then, we would propose an SAN ACLR value in the range of 20 to 30 dB.

From the observations 1 and 2, considering scenarios 1 and 3, considering a 20dB ACIR might be needed (scenario 1, 25 degrees elevation angle), we would propose an NTN fixed VSAT ACLR value of 22 dB.

From the observation 3, if confirmed, the SAN ACS value would need to be high. It might even not be possible to guarantee coexistence between NTN and TN. This would need further study and more accurate simulation results.

From the observation 4 on scenario 5, it would be extremely difficult to make NTN and TN coexisting with current assumptions. Further study would be needed considering additional mitigation techniques, guard band, …

Nevertheless, considering the frequency range (17GHz) of this scenario 5, it should still be possible to agree on acceptable fixed VSAT/mobile VSAT ACS values, even if these values don’t guarantee the coexistence with the assumptions considered for this study.

Proposal: The following table captures our current proposals based on the explanations given before.

| Device | Requirement | Value |
| --- | --- | --- |
| SAN | ACLR | Between 20 and 30 dB |
|  | ACS | More accurate results are needed |
| Fixed VSAT (2.5dB NF) | ACLR | 22 dB |
|  | ACS | Further discussion would be needed but, considering the DL frequency range, it should be possible to converge on an acceptable value. |

# Conclusion

In this contribution, we analyzed the first outcomes of the simulation results and made the following observations and proposal:

Observation1: Scenarios 2, 3, 6 and 7 don’t cause any coexistence issue for all considered types of satellites and NTN UEs, and at 90- or 25-degrees elevation angles.

Observation2: For scenario 1, at 25-degrees elevation angle, 20 dB ACIR would be needed to guarantee coexistence.

Observation3: For scenario 4, 20-25 dB ACIR would be needed to guarantee coexistence.

Observation4: Scenario 5 is the problematic scenario for this coexistence study, requiring 40dB or more ACIR value to have NTN and TN coexisting properly.

Observation5: For scenario 8, 15-20dB ACIR would be needed to guarantee coexistence.

Observation6: For scenario 5  with fixed VSAT and 6 dB NF few dB ACIR less are required to guarantee coexistence.

Observation7: At 25 degrees elevation angle, many 5-percentile results are not relevant, most likely due to the un-sufficient link budget. Further investigation would be needed to better understand if there is any impact on the conclusion.

Observation8: Further results would be needed to confirm this, but mobile VSAT seems to require few dB more ACIR than fixed VSAT, at least in scenario 5.

Proposal: The following table captures our current proposals based on the explanations given before.

| Device | Requirement | Value |
| --- | --- | --- |
| SAN | ACLR | Between 20 and 30 dB |
|  | ACS | More accurate results are needed |
| Fixed VSAT (2.5dB NF) | ACLR | 22 dB |
|  | ACS | Further discussion would be needed but, considering the DL frequency range, it should be possible to converge on an acceptable value. |

# Reference

- RP-230809, NR NTN (Non-Terrestrial Networks) enhancements, Thales

- R4-2317008, Simulation assumption for NTN co-existence study, Samsung

- R4-2319566, NTN enhancement: coexistence simulations results, Ericsson

# Annex

## Coexistence simulations – detailed results.

### Scenario 1: NTN UL Aggressor - TN UL Victim

#### LEO600

Figure 1: Throughput loss – 90deg									Figure 2: SINR cdf	- 90deg

Figure 3: Throughput loss – 25 deg									Figure 4: SINR cdf	- 25deg

#### LEO1200

Figure 5: Throughput loss – 90deg									Figure 6: SINR cdf	- 90deg

Figure 7: Throughput loss – 25 deg								Figure 8: SINR cdf	- 25deg

#### GEO – fixed VSAT

Figure 9: Throughput loss – 90deg										Figure 10: SINR cdf	- 90deg

Figure 11: Throughput loss – 25 deg								Figure 12: SINR cdf	- 25deg

#### GEO – Mobile VSAT

Figure 13: Throughput loss – 90deg								Figure 14: SINR cdf	- 90deg

Figure 15: Throughput loss – 25 deg								Figure 16: SINR cdf	- 25deg

### Scenario 2: TN UL Aggressor - NTN UL Victim

#### LEO600

Figure 17: Throughput loss – 90deg								Figure 18: SINR cdf	- 90deg

Figure 19: Throughput loss – 25 deg								Figure 20: SINR cdf	- 25deg

#### LEO1200

Figure 21: Throughput loss – 90deg								Figure 22: SINR cdf	- 90deg

Figure 23: Throughput loss – 25 deg								Figure 24: SINR cdf	- 25deg

#### GEO – fixed VSAT

Figure 25: Throughput loss – 90deg								Figure 26: SINR cdf	- 90deg

Figure 27: Throughput loss – 25 deg

### Scenario 3: TN UL Aggressor - NTN UL Victim

#### LEO600

Figure 28: Throughput loss – 90deg								Figure 29: SINR cdf	- 90deg

Figure 30: Throughput loss – 25 deg								Figure 31: SINR cdf	- 25deg

#### LEO1200

Figure 32: Throughput loss – 90deg								Figure 33: SINR cdf	- 90deg

Figure 34: Throughput loss – 25 deg								Figure 35: SINR cdf	- 25deg

#### GEO

Figure 36: Throughput loss – 90deg								Figure 37: SINR cdf	- 90deg

Figure 38: Throughput loss – 25 deg								Figure 39: SINR cdf	- 25deg

### Scenario 4: TN UL Aggressor - NTN UL Victim

#### LEO600

Figure 40: Throughput loss – 90deg								Figure 41: SINR cdf	- 90deg

Figure 42: Throughput loss – 25 deg								Figure 43: SINR cdf	- 25deg

#### LEO1200

Figure 44: Throughput loss – 90deg								Figure 45: SINR cdf	- 90deg

Figure 46: Throughput loss – 25 deg								Figure 47: SINR cdf	- 25deg

#### GEO

Figure 48: Throughput loss – 90deg								Figure 49: SINR cdf	- 90deg

Figure 50: Throughput loss – 25 deg									Figure 51: SINR cdf	- 25deg

### Scenario 5: TN DL Aggressor - NTN DL Victim

#### LEO600

Figure 52: Throughput loss – 90deg								Figure 53: SINR cdf	- 90deg

Figure 54: Throughput loss – 25 deg								Figure 55: SINR cdf	- 25deg

#### LEO600 – 6dB NF

Figure 56: Throughput loss – 90deg								Figure 57: SINR cdf	- 90deg

Figure 58: Throughput loss – 25 deg								Figure 59: SINR cdf	- 25deg

#### LEO1200

Figure 60: Throughput loss – 90deg								Figure 61: SINR cdf	- 90deg

Figure 62: Throughput loss – 25 deg								Figure 63: SINR cdf	- 25deg

#### LEO1200 – 6 dB NF

Figure 60: Throughput loss – 90deg								Figure 61: SINR cdf	- 90deg

Figure 62: Throughput loss – 25 deg								Figure 63: SINR cdf	- 25deg

#### GEO – fixed VSAT

Figure 64: Throughput loss – 90deg								Figure 65: SINR cdf	- 90deg

Figure 66: Throughput loss – 25 deg								Figure 67: SINR cdf	- 25deg

#### GEO – mobile VSAT

Figure 68: Throughput loss – 90deg								Figure 69: SINR cdf	- 90deg

Figure 70: Throughput loss – 25 deg								Figure 71: SINR cdf	- 25deg

### Scenario 6: NTN DL Aggressor - TN DL Victim

#### LEO600

Figure 72: Throughput loss – 90deg								Figure 73: SINR cdf	- 90deg

Figure 74: Throughput loss – 25 deg								Figure 75: SINR cdf	- 25deg

#### LEO1200

Figure 76: Throughput loss – 90deg								Figure 77: SINR cdf	- 90deg

Figure 78: Throughput loss – 25 deg								Figure 79: SINR cdf	- 25deg

#### GEO – fixed VSAT

Figure 80: Throughput loss – 90deg								Figure 81: SINR cdf	- 90deg

Figure 82: Throughput loss – 25 deg								Figure 83: SINR cdf	- 25deg

### Scenario 7: TN UL Aggressor - NTN UL Victim

#### LEO600

Figure 84: Throughput loss – 90deg								Figure 85: SINR cdf	- 90deg

Figure 86: Throughput loss – 25 deg							Figure 87: SINR cdf	- 25deg

#### LEO1200

Figure 88: Throughput loss – 90deg								Figure 89: SINR cdf	- 90deg

Figure 90: Throughput loss – 25 deg								Figure 91: SINR cdf	- 25deg

#### GEO – fixed VSAT

Figure 92: Throughput loss – 90deg								Figure 93: SINR cdf	- 90deg

Figure 94: Throughput loss – 25 deg								Figure 95: SINR cdf	- 25deg

### Scenario 8: TN UL Aggressor - NTN UL Victim

#### LEO600

Figure 96: Throughput loss – 90deg								Figure 97: SINR cdf	- 90deg

Figure 98: Throughput loss – 25 deg								Figure 99: SINR cdf	- 25deg

#### LEO1200

Figure 100: Throughput loss – 90deg								Figure 101: SINR cdf	- 90deg

Figure 102: Throughput loss – 25 deg								Figure 103: SINR cdf	- 25deg

#### GEO

Figure 104: Throughput loss – 90deg								Figure 105: SINR cdf	- 90deg

Figure 106: Throughput loss – 25 deg								Figure 107: SINR cdf	- 25deg