3GPP TSG-RAN WG4 Meeting #109	R4-2319620

Chicago, USA, 11th – 15th November 2023

Agenda item:	5.3

Source: 	Inmarsat, Viasat, Globalstar, Ligado Networks, Hughes/Echostar, Thales, Sateliot

Title: 	Flexible TX-RX Separation for NR NTN FR1 bands

Document for:	Approval

# Introduction

During Release 17, core specifications for NR NTN bands were defined as part of the initial NR_NTN_Solutions Work Item.  Two initial FR1 bands were defined, corresponding to existing MSS bands S-band and L-band, as band n256 and n255 respectively.

In the initial specification, these bands were defined with a Fixe Default TX-RX separation, as part of an effort to minimize the work in Release 17, and due to lack of insight on upcoming NTN live deployments.

Following these initial band specifications, new FR1 NTN bands have been proposed in Release 18 as New Spectrum Work Items, such as MSS L/S band, now being specified as band n254, as well as Extended  L-band (proposed as new band n253).

As part of these new efforts, operators have noted that there is a strong desire to introduce the support for Flexible TX-RX separation for NTN, in order to maximize the flexibility by NTN operators to deploy NTN services with existing spectrum constraints.

# Discussion

In this paper, we wish to propose the introduction of flexible TX-RX separation for all NR NTN operating bands.

There is a strong desire by NTN operators to have more flexibility in the frequency allocation and relationship between DL and UL when deploying NTN services.  This is a feature that is typically already supported by existing non-3GPP satellite systems, and is quite fundamental to efficient NTN operation both on current and future space segment.

During the discussions in RAN4#107, RAN#108 and RAN4#108-bis, in the context of the spectrum work items targeting L/S band and Extended L-band, many companies commented that it would be best to address this requirement in a way that is not specific to only one NTN band (i.e. Band 253 or n254), but rather generally applicable to all NTN bands. This is a valid suggestion, as interest for flexible TX-RX separation had already been manifested during the work on new NR NTN L/S Band.

In the context of terrestrial deployments, there are a number of terrestrial NR bands that already support flexible TX-RX separation, due to operator or industry requirements, as shown by the following snippet from TS 38.101-1 Table 5.4.4-1:

Table 5.4.4-1: UE TX-RX frequency separation

| NR Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n24 | -101.5, -120.5 MHz |
| n91 | 570 MHz – 595 MHz (NOTE 2) |
| n92 | 575 MHz – 680 MHz (μ = 0) 580 MHz – 675 MHz (μ = 1) (NOTE 2) |
| n93 | 517 MHz – 547 MHz (NOTE 2) |
| n94 | 522 MHz – 632 MHz (μ = 0) 527 MHz – 627 MHz (μ = 1) (NOTE 2) |
| NOTE 1: Void NOTE 2: The range of TX-RX frequency separation given paired UL and DL channel bandwidths BWUL and BWDL is given by the respective lower and upper limit FDL_low – FUL_high + 0.5(BWDL + BWUL) and FDL_high – FUL_low – 0.5(BWDL + BWUL). The UL and DL channel bandwidth combinations specified in Table 5.3.5-1 and 5.3.6-1 depend on the subcarrier spacing configuration μ [6]. |  |

Observation 1: Flexible TX-RX separation is already supported by a number of terrestrial NR operating bands.

In particular, due to the nature of usage of the NTN bands via satellite, which include deployment of NTN cells around already existing services and a much stronger need for flexible frequency allocation, there is a strong need and benefit for allowing flexibility in the allocation of UL channels vs DL channels across the frequency range in an unpaired manner.  As previously mentioned, this is a feature that is typically already supported by existing non-3GPP satellite systems, and is quite fundamental to efficient NTN operation both on current and future space segment.

In the future, NTN technologies will likely be deployed in a combination of standalone, in-band and guard band, as soon as new or enhanced space segment and ecosystem maturity allow.  This will place a strong requirement for aligning technologies especially when deployed from the same Satellite Access Node (SAN).

In addition, the same satellites may overfly different service areas, where frequency allocations across different satellite services may be different.  Therefore, maximum flexibility in the allocation of DL vs UL channels is of key importance.

Observation 2: Flexibility in allocating UL channels in respect to DL channels is a typical mode of operation for existing satellite systems and is becoming a strong requirement for NTN, in order to accommodate deployment of NTN cells around existing services and within geographical spectrum allocations, within the NTN bands frequency range.

It can also be considered that for all presently-specified NTN bands, the minimum TX-RX separation that the band can have is relatively large, as can be seen by the table below:

| Band | Frequency Range (MHz) | Total Bandwidth (MHz) | TX-RX Separation Min-Max (MHz) |
| --- | --- | --- | --- |
| 255 | DL 1525 - 1559, UL 1626.5 - 1660.5 | 34 | -67.5 to -135.5 |
| 253 | DL 1518 - 1525, UL 1668 - 1675 | 7 | -143 to -157 |
| 255+253 | DL 1518 - 1559, UL 1626.5 - 1660.5, 1668 - 1675 | 41 | -67.5 to -157 |
| 256 | DL 2170 - 2200, UL 1980 - 2010 | 30 | 160 to 220 |
| 254 | DL 2483.5 - 2500, UL 1610 - 1626.5 | 16.5 | 857 to 890 |

Even in the worst case, the smallest frequency separation seen is -67.5 MHz.

Given such minimum spacing between DL and UL, we consider that flexible TX-RX separation could be supported.

Observation 3: Given the minimum TX-RX separation even in the worst case, it should be possible to support flexible TX-RX separation for all of the NTN bands.

Proposal 1: Introduce Flexible TX-RX separation for all NR NTN in FR1 bands as a mandatory release-independent requirement, starting from Release 17.

# Conclusion

Observation 1: Flexible TX-RX separation is already supported by a number of terrestrial NR operating bands.

Observation 2: Flexibility in allocating UL channels in respect to DL channels is a typical mode of operation for existing satellite systems and is becoming a strong requirement for NTN, in order to accommodate deployment of NTN cells around existing services and within geographical spectrum allocations, within the NTN bands frequency range

Observation 3: Given the minimum TX-RX separation even in the worst case, it should be possible to support flexible TX-RX separation for all of the NTN FR1 bands.

Proposal 1: Introduce Flexible TX-RX separation for all NR NTN FR1 bands as a mandatory release-independent requirement, starting from Release 17.

# Reference

- R4-2317641, “WF on IoT NTN Extended L-band” Moderator (Inmarsat)

- 3GPP TS 38.101-1

- 3GPP TS 38.101-5