3GPP TSG-RAN WG4 Meeting #109	R4-2319961

Chicago, US, November 13 – 17, 2023

Title: 	Discussion on RRM test cases for supporting intra-band non-collocated NR-CA

Source: 	Huawei, HiSilicon

Agenda item:	8.11.3

Document for:	Discussion

Introduction

In RAN4#108bis meeting, RAN4 achieved the following agreements on RRM performance part for FR1 intra-band non-collocated NR-CA.

Issue 2-1: whether to introduce new RRM test case for non-collocated FR1 intra-band NRCA - Agreement: - No new RRM test case is needed for non-collocated FR1 intra-band NR-CA. - FFS whether to modify the exiting interruption test case by considering the new RRC signaling for UE capable of type 2 requirements.

In this contribution, we will provide discussion on the impacts on exiting interruption test case for FR1 intra-band non-collocated NR-CA.

Discussion

In TS38.133, the interruption test cases in FR1 and the corresponding verified requirements are summarized as below:

| Section | Interruption test | Interruption requirements to be verified |
| --- | --- | --- |
| A.6.5.2.1 | Interruptions during measurements on deactivated NR SCC in FR1 | Interruptions during measurements on deactivated SCC |
| A.6.5.2.2 | SA interruptions at NR SRS carrier based switching | Interruptions at NR SRS carrier based switching |
| A.6.5.2.3 | SA interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in NR-CA | Interruptions at NR SRS antenna port switching |
| A.6.5.2.4 | SA interruptions at NR SRS antenna port switching |  |
| A.6.5.7 | DL interruptions at switching between two uplink carriers | DL Interruptions at UE switching between two uplink carriers |
| A.6.5.7A | DL interruptions at switching between two uplink carriers with two transmit antenna connectors |  |
| A.6.5.7B | DL interruptions at switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors |  |
| A.6.5.7C | DL interruptions at switching between two uplink bands with two transmit antenna connectors |  |

For above interruption requirements to be verified, only the interruption requirements during the measurement on the deactivated NR SCC are separately defined for UE not supporting intra-band non-collocated NR-CA (Type 1 capability) and UE supporting intra-band non-collocated NR-CA (Type 2 capability).

In current test for interruptions during measurements on deactivated NR SCC in FR1, the verified interruption duration on NR PCell are defined as:

Table A.6.5.2.1.2-1: Interruption duration if the PCell is not in the same band as the deactivated SCell NR Slot length (ms); Interruption length. 0; 1; 1. 1; 0.5; 1. Table A.6.5.2.1.2-2: Interruption duration if the PCell is in the same band as the deactivated SCell NR Slot length (ms); Interruption length. 0; 1; 2 + SMTC duration. 1; 0.5; 2 + SMTC duration.

It can be observed that the current verified interruption duration for inter-band CA scenario is defined as Table A.6.5.2.1.2-2, which is based on Type 1 capability. However, for Type 2 capability UE in intra-band non-collocated CA scenario, the interruption requirements are the same as inter-band CA scenario, i.e. the verified interruption duration shall be defined as Table A.6.5.2.1.2-1.

Proposal 1: In current test on interruptions during measurements on deactivated NR SCC in FR1, it needs to be clarified that the interruption duration verified for Type 2 capability UE in intra-band CA scenario shall be same as that for inter-band CA scenario.

Conclusions

In this paper we provided our analysis on the impacts on RRM tests for supporting intra-band non-collocated EN-DC/NR-CA deployment. The followings are provided.

Proposal 1: In current test on interruptions during measurements on deactivated NR SCC in FR1, it needs to be clarified that the interruption duration verified for Type 2 capability UE in intra-band CA scenario shall be same as that for inter-band CA scenario.

Reference

- R4-2317286, “WF on RRM part for NonCol_intraB_ENDC_NR_CA”, Huawei, HiSilicon