3GPP TSG-RAN WG4 Meeting #109	R4-2320234

# Chicago, 13th - 17th Nov, 2023

Title: 	Discussion on UE CSI reporting requirements for NR FR2 multi-Rx chain DL reception

Source: 	Huawei, HiSilicon

Agenda item:	8.7.4.3

Document for:	Discussion

Background

During RAN4#108b meeting, WF [1] on UE demodulation and CSI requirements for FR2 multi-Rx DL reception was approved. In this contribution, we share our views about UE CSI reporting requirements for NR FR2 multi-Rx chain DL reception.

Discussion

Performance metric

Based on our simulation results in section 3, we can see that there is much gain at 90% of the maximum throughput comparing to 70% of the maximum throughput. Also considering 90% of the maximum throughput is the same value as legacy requirements, we propose to select 90% of the maximum throughput for FR2 multi-Rx PMI reporting requirements. Considering implementation margin, we prefer to select γ = 1.3 as the test metric.

Select 90% of the maximum throughput for FR2 multi-Rx PMI reporting requirements.

Select γ = 1.3 as the test metric.

MCS

Based on our simulation results in section 3, there is no obvious gain difference for different MCS that means both options are feasible. We slightly prefer to select MCS13 for FR2 multi-Rx PMI reporting requirements.

Select MCS13 for FR2 multi-Rx PMI reporting requirements.

Reference channel

Considering that we have two cases with different PTRS configuration, two reference channels should be defined. We propose to use reference channels as following Table 2.3-1 for FR2 multi-Rx PMI reporting requirements.

Table 2.3-1 Reference channels for FR2 multi-Rx PMI reporting requirements

| Parameter | Unit | 1 PTRS port | 2 PTRS port |
| --- | --- | --- | --- |
| Channel bandwidth | MHz | 100 | 100 |
| Subcarrier spacing | kHz | 120 | 120 |
| Allocated resource blocks | PRBs | 66 | 66 |
| Number of consecutive PDSCH symbols |  | 12 | 12 |
| Allocated slots per 2 frames |  | 63 | 63 |
| MCS table |  | 64QAM | 64QAM |
| MCS index |  | 13 | 13 |
| Modulation |  | 16QAM | 16QAM |
| Target Coding Rate |  | 0.48 | 0.48 |
| Number of MIMO layers |  | 2 | 2 |
| Number of DMRS REs (Note 3) |  | 24 | 24 |
| Overhead for TBS determination |  | 6 | 12 |
| Information Bit Payload per Slot |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A | N/A |
| For Slot i = 80 | Bits | 28680 | 27144 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 28680 | 27144 |
| Transport block CRC per Slot |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A | N/A |
| For Slot i = 80 | Bits | 24 | 24 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | CBs | N/A | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | CBs | N/A | N/A |
| For Slot i = 80 | CBs | 4 | 4 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | CBs | 4 | 4 |
| Binary Channel Bits Per Slot |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A | N/A |
| For Slot i = 80 | Bits | 57648 | 55008 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 60720 | 58080 |
| Max. Throughput averaged over 2 frames | Mbps | 90.342 | 90.342 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data |  |  |  |

Use reference channels as Table 2.3-1 for FR2 multi-Rx PMI reporting requirements.

Simulation

Here we provide the initial simulation results for UE CSI reporting for NR FR2 multi-Rx chain DL reception for alignment, as shown in Table 3-2 and Figure 3-1, based on simulation assumption as shown in Table 3-1.

Table 3-1 Simulation assumption for UE CSI reporting requirements for NR FR2 multi-Rx chain DL reception

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | TRxP #1 (Note 1) | TRxP #2 (Note 1) |
| Transmit TRxP of SSB |  |  | TRxP #1 |  |
| PDCCH configuration | TCI state |  | TCI State #1 |  |
|  | CORESETPoolIndex |  | 0 |  |
| Duplex mode |  |  | TDD |  |
| Bandwidth |  | MHz | 100 |  |
| Subcarrier spacing |  | kHz | 120 |  |
| TDD DL-UL configurations |  |  | FR2.120-1 as specified in Annex A of TS 38.101-4 |  |
| Active DL BWP index |  |  | 1 |  |
| Propagation channel |  |  | TDLA30-35 |  |
| Antenna configuration per TRxP |  |  | XP 2x4 (N1,N2) = (2,1), (4 antenna at UE across 2 panels) |  |
| Beamforming Model |  |  | As specified in Annex B.4.1 of TS 38.101-4 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 1.75 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |

Table 3-2 Simulation results for UE CSI reporting requirements for NR FR2 multi-Rx chain DL reception

| Case Number | MCS | UE processing | γ |  |
| --- | --- | --- | --- | --- |
|  |  |  | 70% | 90% |
| 1 | 13 | independent | 1.31 | 1.68 |
| 2 | 13 | joint | 1.32 | 1.70 |
| 3 | 11 | independent | 1.31 | 1.69 |
| 4 | 11 | joint | 1.32 | 1.69 |

Figure 3-1 Simulation results for UE CSI reporting requirements for NR FR2 multi-Rx chain DL reception

Proposals

In this contribution, we discuss on UE CSI reporting requirements for NR FR2 multi-Rx chain DL reception. Our observations and proposals are:

- Select 90% of the maximum throughput for FR2 multi-Rx PMI reporting requirements.

Select γ = 1.3 as the test metric.

Select MCS13 for FR2 multi-Rx PMI reporting requirements.

Use reference channels as Table 2.3-1 for FR2 multi-Rx PMI reporting requirements.

Table 2.3-1 Reference channels for FR2 multi-Rx PMI reporting requirements

| Parameter | Unit | 1 PTRS port | 2 PTRS port |
| --- | --- | --- | --- |
| Channel bandwidth | MHz | 100 | 100 |
| Subcarrier spacing | kHz | 120 | 120 |
| Allocated resource blocks | PRBs | 66 | 66 |
| Number of consecutive PDSCH symbols |  | 12 | 12 |
| Allocated slots per 2 frames |  | 63 | 63 |
| MCS table |  | 64QAM | 64QAM |
| MCS index |  | 13 | 13 |
| Modulation |  | 16QAM | 16QAM |
| Target Coding Rate |  | 0.48 | 0.48 |
| Number of MIMO layers |  | 2 | 2 |
| Number of DMRS REs (Note 3) |  | 24 | 24 |
| Overhead for TBS determination |  | 6 | 12 |
| Information Bit Payload per Slot |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A | N/A |
| For Slot i = 80 | Bits | 28680 | 27144 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 28680 | 27144 |
| Transport block CRC per Slot |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A | N/A |
| For Slot i = 80 | Bits | 24 | 24 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | CBs | N/A | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | CBs | N/A | N/A |
| For Slot i = 80 | CBs | 4 | 4 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | CBs | 4 | 4 |
| Binary Channel Bits Per Slot |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A | N/A |
| For Slot i = 80 | Bits | 57648 | 55008 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 60720 | 58080 |
| Max. Throughput averaged over 2 frames | Mbps | 90.342 | 90.342 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data |  |  |  |

Reference

R4-2316963, WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception, RAN4#108b, Qualcomm