3GPP TSG-RAN WG4 Meeting # 109															R4-2320826

Chicago, US, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.113 | CR | 0069 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [NR_newRAT-Perf, TEI18] CR to TS 38.113: framework for the EMC-specific manufacturer's declarations, Rel-18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf, TEI18 |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In order to align all BS EMC specifications, this CR mirrors modifications from TS 37.113 and TS 37.114, introducing new section for the definition of EMC-specific manufacturer declarations to TS 38.113. Technical content was Endorsed in R4-2313995 during RAN4#108 meeting (Toulouse). Even though this is legacy issue, it is proposed to introduce it in Rel-18 version of specification (no specific need to introduce such modification to earlier releases). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 4.5: adding explicit referneces to the decalrations in RF testing spec. - 4.6: new section for EMC-specific manufacturer declarations, aligning. - 6.2: adding more detailed cross-references to the declarations in RF test specs. - 9.4, 9.5: new cross-references added for EMC-specific declarations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Framework for introduction of EMC-specific declaration would be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 4.5, 4.6, 6.2,9.4, 9.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.113 CR0092, TS37.114 CR0109, TS 37.113 CR0130 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

# 1	Scope

The present document covers the assessment of NR Base Station (BS) and ancillary equipment in respect of Electromagnetic Compatibility (EMC).

The present document specifies the applicable requirements, procedures, test conditions, performance assessment and performance criteria for NR base stations and associated ancillary equipment in the following categories:

-	BS equipped with antenna connectors or TAB connectors which are possible to be terminated during EMC testing, meeting the BS type 1-C and BS type 1-H RF requirements of TS 38.104 [2], with conformance demonstrated by compliance to TS 38.141-1 [3].

-	BS not equipped with antenna connectors nor TAB connectors, i.e. with antenna elements radiating during the EMC testing, meeting the BS type 1-O and BS type 2-O RF requirements of TS 38.104 [2], with conformance demonstrated by compliance to TS 38.141-2 [4].

Technical requirements related to the antenna and TAB connectors are not included in the present document. These are found in the relevant NR BS product standards TS 38.104 [2], 3GPP TS 38.141-1[3], and 3GPP TS 38.141-2 [4].

The environment classification used in the present document refers to the residential, commercial and light industrial environment classification used in IEC 61000-6-1 [7] and IEC 61000-6-3 [8].

The EMC requirements have been selected to ensure an adequate level of compatibility for apparatus at residential, commercial and light industrial environments. The levels, however, do not cover extreme cases which may occur in any location but with low probability of occurrence

------------------------------ Next modified section ------------------------------

## 4.5	BS test configurations

The present clause defines the BS test configurations that shall be used for demonstrating conformance. A single NR carrier shall be used for testing of single-carrier capable BS.

Single carrier configuration (SC) tests shall be performed using signal with narrowest supported BS channel bandwidth with the smallest supported subcarrier spacing declared per operating band in TS 38.141-1 [3] clause 4.6 (see declaration D.14), and TS 38.141-2 [4] clause 4.6 (see declaration D.7).

For other NR base stations, the test configurations in table 4.5-1 and table 4.5-2 shall be used. The NR test configurations (NRTCx) are defined in TS 38.141-1 [3], clause 4.7 for BS type 1-C and BS type 1-H and in TS 38.141-2 [4], clause 4.8 for BS type 1-O and BS type 2-O.

Table 4.5-1: Test configurations for BS type 1-C and BS type 1-H

| BS test case | BS capable of multi-carrier and/or CA in a single band |  |  | BS capable of multi-band operation |  |
| --- | --- | --- | --- | --- | --- |
|  | Contiguous spectrum capable BS | C and NC capable BS with identical parameters | C and NC capable BS with different parameters | BS capable of multi-band operation with common connector | BS capable of multi-band operation with separate connector |
| Emission tests | NRTC1 | NRTC3 | NRTC1, NRTC3 | NRTC1/3 (Note 1), NRTC5 | NRTC1/3 (Note 1), NRTC5 (Note 2) |
| Immunity tests | NRTC1 | NRTC3 | NRTC1, NRTC3 | NRTC1/3 (Note 1), NRTC5 | NRTC1/3 (Note 1), NRTC5 (Note 3) |
| Note 1: NRTC1 and/or NRTC3 shall be applied in each supported operating band. Note 2: For single-band operation test, other antenna connector(s) is (are) terminated. Note 3: NRTC5 is only applicable for multi-band receiver. |  |  |  |  |  |

------------------------------ Next modified section ------------------------------

## 4.6	Manufacturer declarations

The following EMC-specific manufacturer’s declarations listed in table 4.6-1 are required to be provided by the manufacturer for NR BS requirements testing.

NOTE:	The below listed manufacturer’s declarations are supplementary to declarations covered in TS 38.141-1 [3] and/or TS 38.141-2 [4].

Table 4.6-1 EMC-specific manufacturer declarations for NR BS test requirements

| Declaration identifier | Declaration | Description |
| --- | --- | --- |
| DEMC.1 | Ports intended to be used with cables > 3 m | Declaration of any port(s) intended to be used with cables longer than 3 m. |

------------------------------ Next modified section ------------------------------

## 6.2	Performance criteria for transient phenomena for BS

At the conclusion of the total test (comprising the series of individual exposures to transient phenomena) the EUT shall operate as intended with no user noticeable loss of control functions or stored data. At the conclusion of each exposure the EUT shall operate with no user noticeable loss of the communication link. The channel bandwidth, sub-carrier spacing and bearer information data rate should be chosen based on manufacturers declarations (see declaration D.14 in TS 38.141-1 [3] and/or D.7 in TS 38/141-2 [4]) and defined according to table 6.1-1 and table 6.1-2.

------------------------------ Next modified section ------------------------------

## 9.4	Fast transients common mode

The test shall be performed on AC mains power input ports.

This test shall be performed on signal ports, telecommunication ports, control ports and DC power input/output ports if the cables may be longer than 3 m.

Where this test is not carried out on a port or any other ports because the manufacturer declares in EMCD.1 (see table 4.6-1) that it is not intended to be used with cables longer than 3 m, a list of ports which were not tested for this reason shall be included in the test report.

This test shall be performed on a representative configuration of the equipment, the associated ancillary equipment, or representative configuration of the combination of radio and ancillary equipment.

------------------------------ Next modified section ------------------------------

## 9.5	RF common mode (0.15 MHz - 80 MHz)

The test shall be performed on AC mains power input/output ports.

This test shall be performed on signal ports, telecommunication ports, control and DC power input/output ports, which may have cables longer than 3 m.

Where this test is not carried out on a port or any other ports because the manufacturer declares in EMCD.1 (see table 4.6-1) that it is not intended to be used with cables longer than stated above, a list of ports which were not tested shall be included in the test report.

This test shall be performed on a representative configuration of the equipment, the associated ancillary equipment, or representative configuration of the combination of radio and ancillary equipment.

NOTE:	This test can also be performed using the clamp injection method, where appropriate, see IEC 61000-4-6 [21].

------------------------------ End of modified section ------------------------------