3GPP TSG-RAN WG4 Meeting #109      		                                  R4-2320952

Chicago, USA, 13th-17th of November, 2023

# Source:	THALES

# Title:	Draft TP for TR 37.911 - Study on self-evaluation towards the IMT-2020 submission of the 3GPP Satellite Radio Interface Technology

# Type:	pCR

# Document for:	approval

# Agenda Item:	8.26.1.3

Release:	Rel-18

# Introduction

As per the approved SID [RP-231945] on IMT-2020 satellite radio interface evaluation, it is expected that 3GPP will be actively contributing to the satellite component of IMT-2020 radio interface(s) development and make the submission to ITU-R WP 4B at the appropriate time. Therefore, it is necessary for 3GPP to complete the description and compliance templates and conduct a self-evaluation to complete the submission and evaluation process for IMT-2020.

For more information, please also check RP-231444 for SI work plan for Study on self-evaluation towards the IMT-2020.

Detailed objectives of this study item include:

- Complete all required submission templates as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc] - Provide self-evaluation results against technical performance requirements for eMBB-s as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc, RAN1, RAN2], including - Peak data rate - Peak spectral efficiency - User experienced data rate - 5th percentile user spectral efficiency - Average spectral efficiency - Area traffic capacity - Latency, including user plane latency and control plane latency - Energy efficiency, including both network and device - Mobility - Mobility interruption time - Provide self-evaluation results against technical performance requirements for mMTC-s as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc, RAN1, RAN2], including - Connection density - Provide self-evaluation results against technical performance requirements for HRC-s as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc, RAN1, RAN2], including - Reliability - Provide self-evaluation results for other requirements (including bandwidth) as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc, RAN1, RAN2, RAN4] This study shall start with evaluating features that are supported by Rel-17 NTN (NR NTN + IoT NTN), as relevant for the above aspects.

For more information, one may also refer to the ITU R report: Rep. ITU-R M.2514-0

### 7.2.13 Bandwidth Bandwidth is the maximum aggregated system bandwidth. Scalable bandwidth is the ability of the candidate RIT/SRIT to operate with different bandwidth. The bandwidth capability of the RIT/SRIT is defined for the purpose of IMT-2020 evaluation. The RIT/SRIT should support a scalable bandwidth up to 30 MHz. The support of maximum bandwidth is verified by inspection of the proposal. The scalability requirement is verified by demonstrating that the candidate RITs/SRITs can support different bandwidth values. These values shall include the minimum and maximum supported bandwidth of the candidate RIT/SRIT.

As a result of this work, several CRs have been also submitted and agreed to support 30 MHz channel bandwidth as part of a new work item.

The next section provides TP for on-going TR 37.911.

# TP for on-going TR 37.911

<<Start of Change>>

# 7	Self-evaluation of generic requirements

## 7.1	Service aspects

## 7.2	Bandwidth

The transmission bandwidth configuration NRB for each SAN channel bandwidth and subcarrier spacing is specified in Table 7.2-1 for NTN-FR1 as explained in TS 38.108 and TS 38.101-5.

Table 7.2-1: Transmission bandwidth configuration NRB for NTN-FR1

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 160 |
| 30 | 11 | 24 | 38 | 51 | 78 |
| 60 | N/A | 11 | 18 | 24 | 38 |

The minimum guard band for each SAN channel bandwidth and SCS is specified in Table 7.2-2 for NTN-FR1.

Table 7.2-2: Minimum guard band (kHz) (NTN-FR1)

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz |
| --- | --- | --- | --- | --- | --- |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | 592.5 |
| 30 | 505 | 665 | 645 | 805 | 945 |
| 60 | N/A | 1010 | 990 | 1330 | 1290 |

The number of RBs configured in any SAN channel bandwidth shall ensure that the minimum guard band specified in this clause is met.

## 7.3	Spectrum

Table 5.1-1: Definition of frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1 | 410 MHz – 7125 MHz |

Corresponding to the current definition of frequency ranges from TS 38.108, NTN-FR1 frequency range is defined based on 410 MHz – 7125 MHz frequency interval. The Satellite Access Node and related UEs are designed to operate in the operating bands defined in Table 7.3-1.

Table 7.3-1: Satellite operating bands in FR1

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980 MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from n256. |  |  |  |

<<End of Change>>

END