3GPP TSG-RAN WG4 Meeting # 109												R4-2321064

Chicago, USA, November 13 – November 17, 2023

Agenda item:			8.14.7

Source:	Nokia, Nokia Shanghai Bell

Title:	WF on [109][322] NR_FR1_lessthan_5MHz_BW_demod

Document for:	Approval

# Topic#1: UE Demod

## Sub-topic#1-1: General

Issue 1-1-1: Will UE exist that supports ONLY 5MHz CBW and less?

Agreement (online session):

- For the purpose of demodulation test coverage in Rel-18, assume there are no UEs that ONLY support maximum channel bandwidth of 5 MHz.

Issue 1-1-2: Channel BW for UE Demodulation performance requirements

Agreement:

- Introduce UE demodulation requirements only for 3MHz CBW

  - FFS, a need for PDCCH requirements also for 5MHz CBW, 20PRB

Issue 1-1-3: Number of RX antenna in UE Demodulation requirements

Agreement (online session):

- Cover both 2Rx and 4Rx for requirements definition.

Issue 1-1-3: HST propagation conditions

Way forward:

Consider The following parameters for performance evaluation in HST conditions with less than 5MHz CBW:

- [HST-417]: Single-tap propagation conditions (B3.1 model), Ds = 300 m, Dmin = 2 m, f_d = [417] Hz

- [HST-DPS-417]: DPS propagation conditions (B3.3 model), Ds = 700 m, Dmin = 150 m, f_d = [417] Hz

Issue 1-1-4: Applicability rules

Way forward:

Further discussion is needed on how to define applicability rules:

- Option 1: Create the UE demodulation requirement applicability table for UE supporting NR_FR1_lessthan_5MHz_BW.

- Other options are not precluded.

## Sub-topic#1-2: PDSCH

Issue 1-2-1: Introduction of PDSCH requirements

Way forward:

Introduction pf PDSCH requirements for less then 5MHz requires further discussion:

- Option 1: Do not introduce new PDSCH requirements for 3MHz CBW

- Option 2: Introduce new requirements for PDSCH for 3MHz CBW

  - Option 2-a: in non-HST conditions

  - Option 2-b: with HST channel

  - Option 2-c: Option 2-a and Option 2-b.

Issue 1-2-2: Parameters for PDSCH performance evaluation:

Way forward:

Interested companies can consider the following parameter for PDSCH performance evaluation with less than 5MHz CBW:

- in non-HST conditions.

| Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Rank | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| R.PDSCH.1-1.1 FDD | 3 / 15 | QPSK, 0.30 | TDLB100-400 | Rank 1 | 2x2, ULA Low | 70 | TBD |
|  |  |  |  |  | 2x4, ULA Low | 70 | TBD |
| R.PDSCH.1-2.1 FDD | 3 / 15 | 16QAM, 0.48 | TDLC300-100 | Rank 1 | 2x2, ULA Low | 70 | TBD |
|  |  |  |  |  | 2x4, ULA Low | 70 | TBD |

- In with HST case:

  - for Single-tap propagation conditions:

| Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| R.PDSCH.1-8.2 FDD | 3 / 15 | 64QAM, 0.43 | [HST-417] | 1x2 | 70 | TBD |
| R.PDSCH.1-8.2 FDD | 3 / 15 | 64QAM, 0.43 | [HST-417] | 1x4 | 70 | TBD |

  - for HST-DPS propagation conditions:

| Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| R.PDSCH.1-8.4 FDD | 3 / 15 | 64QAM, 0.43 | [HST-DPS-417] | 1 | 2x2 | 70 | TBD |
| R.PDSCH.1-8.4 FDD | 3 / 15 | 64QAM, 0.43 | [HST-DPS-417] | 1 | 2x4 | 70 | TBD |

- Interested companies are encouraged to bring simulation results.

Issue 1-2-3: SDR requirements

Way forward:

Introduction of SDR requirements for less than 5MHz CBW requires further discussion:

- Option 1: Do not introduce new SDR requirements for 3MHz CBW

- Option 2: Apply SDR tests for 3MHz CBW. Update TS 38.101-4 Tables 5.5A-1 and 5.5A-4 to support 3MHz CBW

Issue 1-2-4: PDCCH AL for PDSCH requirements (test setup)

Way forward:

FFS, whether it is necessary to revisit PDCCH AL configuration to fit to 3MHz CBW for PDSCH demodulation requirements with 3MHz CBW. Possible PDCCH configuration is to set AL2 without puncturing.

## Sub-topic#1-3: PDCCH

Issue 1-3-1: Requirements for non-punctured PDCCH

Agreement:

- Not to introduce new demodulation performance requirements for non-punctured PDCCH with CBW below 5MHz in normal conditions.

Issue 1-3-2: Requirements for punctured PDCCH

Way forward:

Further discussion of requirements for punctured PDCCH is needed:

- Option 1: Define punctured PDCCH demodulation requirements with 15PRBs for UE supporting NR_FR1_lessthan_5MHz_BW considering the following parameters:

  - 15PRBs, 3 symbols, non-interleaved, AL4, DCI 1_0 (35 bits for 15 PRBs); Use CCEs #4, #5, #6, and #7 to transmit PDCCH with DCI 1_0.

- Option 2: Introduce requirements, if testability issue is resolved.

- Option 3: Do not introduce new requirements for punctured PDCCH with focus on CORESET#0 puncturing.

Issue 1-3-3: PDCCH requirements in HST conditions

Way forward:

The Issues requires further discussion:

- Option 1: Introduce PDCCH requirements at 3MHz CBW in HST conditions.

- Option 2: Not to introduce HST scenario for PDCCH requirements.

## Sub-topic#1-4: PBCH

Issue 1-4-1: PBCH requirement in non-HST conditions

Agreement:

Use the following parameters for PBCH requirement in normal conditions. Interested companies are encouraged to bring simulation results.

| Duplex | Bandwidth (MHz) / Subcarrier spacing (kHz) | SSB/PBCH index | Propagation condition | Antenna configuration and correlation matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| FDD | 3 / 15 | Unknown | TDLC300-100 | 1 x 2 Low, | 1 | TBD |
|  |  |  |  | 1x4 Low | 1 | TBD |

Issue 1-4-2: PBCH requirement in HST conditions

Way forward:

Further discuss whether to introduce PBCH requirements in HST conditions:

- Option 1: Define PBCH requirements in HST conditions considering the following parameters:

| Duplex | Bandwidth (MHz) / Subcarrier spacing (kHz) | SSB/PBCH index | Propagation condition | Antenna configuration and correlation matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| FDD | 3 / 15 | Unknown | [HST-417] | 1Tx/2Rx Low | 1 | TBD |
| FDD | 3 / 15 | Unknown | [HST-417] | 1Tx/4Rx Low | 1 | TBD |

- Option 2: Not to introduce HST scenario for PBCH requirements.

## Sub-topic#1-5: CSI reporting

Issue 1-5-1: Introduction and scope of the requirements

Agreement:

- Do not define new CSI reporting requirements in less than 5MHz.

# Topic#2: BS Demod

## Sub-topic#2-1: General

Issue 1-1-1: Manufacturer declaration for 3MHz CBW

Agreement:

- Introduce new BS manufactory declaration for the support of less than 5MHz CBW.

Issue 1-1-2: Will BS exist that supports only less than 5MHz CBW?

Agreement:

- For the purpose of demodulation test coverage in Rel-18, assume there are no BSs that support only less than 5 MHz.

## Sub-topic#2-2: PUSCH

Issue 2-2-1: A need for PUSCH requirements with less than 5MHz CBW

Way forward:

The Issue requires further discussion:

- Option 1: Introduce limited set of PUSCH requirements for less than 5MHz.

  - FFS for requirements in HST conditions, including PUSCH demodulation performance and/or UL timing adjustment.

- Option 2: Don’t define PUSCH requirements with 3MHz bandwidth.

Issue 2-2-2: Testing of PUSCH with/without precoding enabled (CP-OFDM / DFT-s-OFDM)

Agreement:

- If PUSCH requirement for less than 5MHz CBW is decided to be introduced, then only consider CP-OFDM.

Issue 2-2-3: UCI multiplexed on PUSCH

Agreement:

- RAN4 shall NOT define requirements for UCI multiplexed on PUSCH for the less than 5MHz work item.

Issue 2-2-4: PUSCH simulation parameters for normal conditions

Way forward:

- Consider the following parameters for the evaluation of PUSCH performance:

  - Number of PRBS: 12, 15

  - MCS: 2, 16, 20

  - 1T2R, 1 layer

  - mapping type A

  - DMRS: (1+0), (1+1)

  - Channel conditions:

    - For MCS 2 TDLB 100-400

    - For MCS 20 TDLA 30-10,

    - For MCS 16 TDLC 300-100

- Interested companies are encouraged to provide simulations.

- Further downselection of parameters is not precluded.

Issue 2-2-5: PUSCH requirements in HST conditions

Way forward:

Introduction of PUSCH requirements in HST conditions require further discussion:

- Consider the following parameters for the evaluation of PUSCH performance in HST condition:

  - 3DMRS if HST

  - The existing High Speed Train condition (Clause G.3 in TS 38.104) at 500km/h can be reused as a starting point

    - Ds=700m, Dmin=100m

    - Maximum Doppler shift f_d = 815Hz

- Companies are encouraged to bring simulations results for PUSCH demodulation performance

- Interested companies can also evaluate UL timing adjustment requirement in Moving propagation conditions (Clause G.4 from TS 38.104), Scenario Z.

## Sub-topic#2-3: PUCCH

Issue 2-3-1: Introduction of PUCCH requirements

Agreement:

- Introduce new PUCCH format 2 for UCI BLER requirements for 3MHz.

  - Other formats are not precluded.

- Use the following parameters for the requirement:

  - PUCCH requirement with frequency hopping for 1Tx2Rx antenna configuration.

  - Use TDLC 300-100 channel conditions

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Test metric | BLER |

## Sub-topic#2-3: RACH

Issue 2-4-1: Applicability rule or note for long RACH sequences

Way forward:

The Issues requires further discussion:

- Option 1: Add applicability rule or note

  - Option 1-a: For BS supporting less than 5MHz carrier bandwidth only test requirements relating to short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS shall apply.

  - Option 1-b: Unless otherwise stated, for the subcarrier spacing to be tested, the test requirements shall apply only for anyone channel bandwidth not less than 20MHz declared to be supported (see D.14 in table 4.6-1).

- Option 2: No need to add applicability rule or note.