3GPP TSG-RAN WG4 Meeting #109	R4-2321140

Chicago, USA Nov 13th ‒ Nov 17th, 2023

Title: 	WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception

Agenda Item:	5.7.4

Source: 	Qualcomm

Document for:	Approval

Moderator’s note: For all the open Issues listed in the WF below, other options are not precluded unless explicitly specified.

# General Aspects for FR2 Multi-Rx Demod

Issue 1-1-1: Receiver assumption for mDCI fully and non-overlapping case.

Agreement:

- Consider separate processing for mDCI in Rel-18.

- RAN4 may consider joint processing in future releases.

Issue 1-1-3: Receiver assumption for sDCI SDM case.

Agreement:

- Consider separate processing for sDCI in Rel-18.

- RAN4 may consider joint processing in future releases.

Issue 1-1-4: MCS/layer/cross-talk for sDCI SDM case.

- Agreement:

  - 1+1, MCS17, ρ=-12dB

  - Do not define requirements with 2+2

Issue 1-1-5: MCS/cross-talk for mDCI fully overlapping 1+1 case.

- Agreement:

  - 1+1, MCS17, ρ=-12dB

Issue 1-1-6: MCS/cross-talk for mDCI fully overlapping 2+2 case.

- Agreement:

  - Do not define requirements with 2+2

Issue 1-1-7: MCS/layer/cross-talk for mDCI non overlapping case.

- Agreement:

  - 2+2, MCS13, ρ=-12dB

Issue 1-1-8: TxEVM.

  - FFS for further study.

Issue 1-1-9: Assumption on correlation model.

- Agreement:

  - Use agreed correlation model from RAN4#107.

Issue 1-1-10: Whether to adopt NR FR2 OTA enhancements when defining demodulation requirements.

- Agreement:

  - Defer this discussion to FR2 OTA.

Issue 1-1-11: UE feature for joint processing (if introduced).

- Agreement:

  - Not applicable since requirement with joint processing will not be introduced.

Issue 1-1-12: TR update.

- Agreement:

  - RAN4 will expand the scope of TR 38.751 to include demod evaluation.

    - Content and work split to be discussed over email (post meeting RAN4#109)

# PDSCH Demodulation Requirements

Issue 2-1-1: PT-RS EPRE Ratio

- Agreement:

- Define EPRE-Ratio state ‘0’ for FR2 multi-panel RX simulation assumptions.

Issue 2-1-2: Number of DMRS CDM groups without data

- Agreement:

- Confirm that the number of CDM groups without data is 2 for DMRS configuration for mDCI.

Issue 2-1-3: Time/frequency offsets for sDCI SDM

- Agreement:

  - 600 Hz, -0.0625us

Issue 2-1-4: Time/frequency offsets for mDCI fully overlapping.

- Agreement:

  - 600 Hz, -0.0625us

Issue 2-1-5: Time/frequency offsets for mDCI non overlapping.

- Agreement:

  - 0.25us, 0Hz

Issue 2-1-6: General PDSCH/PDCCH configuration for mDCI

- Agreement:

    - Configure PDCCH from each TRP non-overlapping in time for mDCI transmission mode.

    - PDCCH from TRP1 is transmitted on symbol 0 and PDCCH from TRP2 is transmitted on symbol 1 of the slot.

    - PDSCH transmission starts from symbol 2 (No of PDSCH symbols is 12)

Issue 2-1-7: General PDSCH/PDCCH configuration for sDCI SDM

- Agreement:

  - For sDCI SDM transmission scheme, PDCCH is transmitted on symbol 0 and PDSCH is transmitted from symbol 1.

# PMI Reporting Requirements

Issue 3-1-1: Receiver assumption for PMI Reporting

- Agreement:

  - Consider separate processing for PMI reporting with sDCI SDM.

Issue 3-1-2: MCS

- Agreement:

  - MCS13 with 1+1.

Issue 3-1-3: Time/frequency offsets for PMI reporting

- Agreement:

  - Set FO=0Hz and TO=0us for PMI reporting test

Issue 3-1-4: Performance Metric

- Agreement:

  - X= [90%]

Issue 3-1-5: Throughput ratio (γ) value

- Agreement:

  - Consider γ = [1.3] in the next meeting, other values are not precluded. Consider some margin (to be discussed).

Issue 3-1-6: Reference Channel

- <way forward>

  - Option 1: Use reference channels as presented in Table 2.3-1 for FR2 multi-Rx PMI reporting requirements.

  - Option 2: Reuse the reference channel from TS38.101-4, section 6.3.2.1.7 with adaptation to 4 ports as starting point. Configure the codebook for 1 panel.