3GPP TSG-RAN WG4 Meeting # 109											R4-2321725

Chicago, US, November 13 – 17, 2023

Source: 	vivo

Title: 	TP to 38.751 on further evaluation of gain difference between V-pol and H-pol

Agenda Item:	8.7.1.2

Document for:	Approval

- Introduction

In [1], a new method for calculating the received signal when gain imbalance between V-pol and H-pol is agreed. The DL polarization part in TR 38.751 need to be updated accordingly.

- Text Proposal

### 6.3.6	DL polarization combination

There are four different DL polarization combinations, i.e., (TRP1q, TRP2q), (TRP1f, TRP2q), (TRP1q, TRP2f) or (TRP1f, TRP2f). It is known that the UE performance can differ if the same polarization or orthogonal polarization is adopted by the two probes in testing. With the same polarization, a higher correlation between the two DL data streams can be expected, especially when the AoA offset is small, representing the worst-case performance.

In deciding which combination to use for verification, there are two main considerations. First, the requirements should be derived based on the worst-case polarization match at the UE, and should apply for any combination of DL polarizations from each TRP. Second, there is a need to minimize the testing overhead while ensuring good testing coverage.

In the two-AoA simulations, the worst-case polarization match between the 2 TRPs is assumed. In other words, the transmitted signal from one TRP is considered interference without any polarization isolation at the UE when decoding the transmitted signal from the other TRP.

It was also pointed out that the radiation pattern of V-pol and H-pol of an antenna module is not exactly same, and when metal blockage exist, the difference may be amplified due to the reflection as shown in the Figure below. In practical UE design, to avoid severe blockage, the metal frame near the antenna module can be removed and replaced by other materials, but the V/H element may still suffer from different loss, which still lead to difference between V-pol and H-pol.

Figure 6.3.6-1 The radiation pattern of a pair of V/H element in 1x4 antenna module

EIS50%(AoA1,θ,AoA2,θ)(AoA1,φ,AoA2,φ)

If the gain difference exists between V-pol and H-pol, for each receiver branch, the projection from  and  should be calculated, and the following procedure to calculate received signal can be used:

Step-1: Extract the antenna gain in θ direction and φ direction in module reference system when only V-elements are activated, i.e., GV,M (M, M) and GV,M (M, M)

Step-2: Extract the antenna gain in θ direction and φ direction in module reference system when only H-elements are activated, i.e., GH,M (M, M) and GH,M (M, M)

Figure 6.3.6-2 illustration of antenna gain in θ direction and φ direction

Step-3: For the DL signal from each TRP in the TE coordinate system, transform the TRP location into the module reference system, e.g., for antenna module#1, AoA1(1, 1)AoA1(1M#1, 1M#1), AoA2(2, 2)  AoA1(2M#1, 2M#1) and for antenna module#2, AoA1(1, 1) AoA1(1M#2, 1M#2), AoA2(2, 2)  AoA1(2M#2, 2M#2)

Figure 6.3.6-3 illustration of coordinate system transformation

Step-4: The received signal for each antenna module (after MRC of rank1 DL) can be calculated when DL signal polarization is (θ,θ), e.g., the received signal from AoA1 by antenna module#1 is:

s1,θM#1= GV,θMM, M+GH,θMM, M, where M=1M#1, M=1M#1

The received signal from AoA1 by antenna module#2 is:

s1,θM#2= GV,φMM, M+GH,φMM, M, where M=1M#2, M=1M#2

Step-5: The received signal for each antenna module (after MRC of rank1 DL) can be calculated when DL signal polarization is (φ, φ), e.g., the received signal from AoA1 by antenna module#1 is:

s1,φM#1= GV,φMM, M+GH,φMM, M, where  M=1M#1, M=1M#1

The received signal from AoA1 by antenna module#2 is:

s1,φM#2= GV,φMM, M+GH,φMM, M, where  M=1M#2, M=1M#2

The whole formula is shown in Table below:

Table 6.3.6-1 The formula for received signal

| DL | Module #1 | Module #2 |
| --- | --- | --- |
| Signal level(*) | sθM#1= GV,θMM, M+GH,θMM, M | sθM#2=GV,φMM, M+GH,φMM, M |
| TRP1 location transformed to module reference system | M=1M#1, M=1M#1 | M=1M#2, M=1M#2 |
| TRP 2 location transformed to module reference system | M=2M#1, M=2M#1 | M=2M#2, M=2M#2 |

| DL | Module #1 | Module #2 |
| --- | --- | --- |
| Signal level(*) | sφM#1= GV,φMM, M+GH,φMM, M | sφM#2= GV,φMM, M+GH,φMM, M |
| TRP1 location transformed to module reference system | M=1M#1, M=1M#1 | M=1M#2, M=1M#2 |
| TRP 2 location transformed to module reference system | M=2M#1, M=2M#1 | M=2M#2, M=2M#2 |

(*For simplicity, the DL signal in formula is normalized to 1)

The method above is a simplified one compared to the realistic MRC implementation at UE receiver, in figure 6.3.6-4, a simulation is performed to compare UE performance projections using the method above against a more realistic LMMSE type demodulation for two orientations of the UE. The UE model is a ‘4-component type’ described in Table 6.3.6-1. The MMSE solver assumed RSRP-based beam selection and a noise covariance matrix that depends on both, the uncorrelated thermal noise of the receiver and the correlated noise from the interferer. Channel estimation is genie type based on knowledge 4 component antenna gains and AWGN assumption.

Figure 6.3.6-4: UE performance difference between LMMSE vs RAN4 simplified SINR estimation

Some observations are listed below:

1.	For UEs with modules on opposite faces, there is not much difference between the simplified RAN4 SINR method and the LMMSE-based method. The similarity is expected since this type of UE does not have significant overlap of coverage areas, so SINR tends to be noise limited.

2.	For UEs with modules on adjacent faces, there can be significant overlap in coverage. Some AoA pairs (specifically those with lower AoA offsets) are interference limited. For these scenarios, MMSE can help significantly because of polarization mismatch in the general case between the 2 rank1 DLs.

3.	The characteristic of the MMSE based receiver to help in interference limited cases becomes clear for a UE with modules on the same face. This type of UE has a lot of spatial overlap and therefore strong interference for low AoA offsets. The MMSE based received performs better than the RAN4 SINR calculation estimate for narrow AoA offsets where interference limited cases become more dominant.

To further investigate how the gain imbalance will impact on the UE performance under the method above, Figure 6.3.6-5 show the overall probability performance gap between  DL and  DL, and three implementations are considered: Implementation#1 is 2 panels in the same side, Implementation#1 is 2 panels in the adjacent side, Implementation#3 is 2 panels in the opposite side. The X dB additional gain difference is added to V pattern and H pattern separately, 1.e., +X/2 dB to V pattern and -X/2 dB to H pattern. Both the results of OR combing and arithmetic mean are provided.

Figure 6.3.6-5 Performance gap between  DL and  DL

The results show that when gain difference increase, the performance gap is not always enlarged since the gain difference will affect the received signal in both V branch and H branch and how the performance gap changes depending on panels location, AoA offset, etc. To reduce the performance gap between different polarization of DL polarization, one possible way is to construct final results by averaging the performance between  DL and  DL. Figure 6.3.6-6 shows that how the gain difference impact on the results after average.

Figure 6.3.6-6 Overall probability under different gain imbalance after average

Compared to the performance gap between different DL polarizations, the performance change under different additional gain difference is smaller after average. We can also find that there is no conclusive relationship between gain imbalance and UE performance.

In the end, the UE RF requirement is defined as the average (arithmetic mean) of the metric values for two DL polarization test conditions: the first condition is when the DL polarizations at both TRPs are ‘’ and the second condition is when they are ‘’.

N%= N1%(AoA1θ, AoA2θ)+N2%(AoA1φ, AoA2φ)2

Where ‘N%(.,.)’ is the metric value for the requirement for the DL pols specified in the subscripts.

- References

- R4-2314668	WF on FR2_multiRx_UERF,	vivo, RAN4#108

- R4-2315812, 	Discussion on remaining issue on FR2 multi-Rx, vivo, RAN4#108bis