3GPP TSG-RAN WG4 Meeting # 109						R4-2321857

Chicago, USA, November 13 – November 17, 2023

Title:	TP to TR 38.718-02-01 Addition to CA_n7-n102 and DC_n7-n102

Source:	Nokia, BT

Agenda item:	7.10.2

Document for:	Approval

# 1	Introduction

This TP was partially accepted through [1] and this is an addition to TR 38.718-02-01 to add band n7 and contiguous intra-band carrier aggregation in uplink band n102.

This TP is dependent on fallback also submitted for same meeting in R4-2320035

************************************* Start of TP*****************************************

## 5.29		CA_n7-n102

### 5.29.1	Common for 1 band UL and 2 bands UL CA

#### 5.29.1.1 Operating bands for CA

Table 5.29.1.1-1: CA band combination of band n7+n102

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| n102 | 5925 MHz | – | 6425 MHz | 5925 MHz | – | 6425 MHz | TDD |

#### 5.29.1.2	Channel bandwidths per operating band for CA

Table 5.29.1.2-1: Supported bandwidths per CA band combination of band n7+n102

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n7A-n102A | CA_n7A-n102A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n7A-n102(2A) | CA_n7A-n102A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n7A-n102B | CA_n7A-n102A CA_n7A-n102B | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n7A-n102C | CA_n7A-n102A CA_n7A-n102C | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n7A-n102D | CA_n7A-n102A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n7A-n102E | CA_n7A-n102A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |

#### 5.29.1.3	Co-existence studies

Table 5.29.1.3-1/2 summarizes frequency ranges where UL harmonics and/or harmonic mixing occur for CA_n7-n102.

Table 5.29.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5000 | 5140 | 7500 | 7710 | 10000 | 10280 | 12500 | 12850 |
| n102 | 5925 | 6425 | 5925 | 6425 | 11850 | 12850 | 17775 | 19275 | 23700 | 25700 | 29625 | 32125 |

Based on above table, there is no uplink harmonic issues.

Table 5.29.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5240 | 5380 | 7860 | 8070 | 10480 | 10760 | 13100 | 13450 |
| n102 | 5925 | 6425 | 5925 | 6425 | 11850 | 12850 | 17775 | 19275 | 23700 | 25700 | 29625 | 32125 |

Based on above table, there is no harmonic mixing issues.

#### 5.29.1.4	∆TIB and ∆RIB values

For CA_n7-n102, the TIB,c is based on CA_n7-n79 having n79 as the highest frequency range among similar CA’s. For CA_n7-n102, the RIB values are also based on CA_n7-n79

Table 5.29.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n7-n102 | 0.5 | 0.8 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.29.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n7-n102 | - | 0.5 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

#### 5.29.1.5	REFSENs requirements

#### There are no specific REFSENS requirements for 1 band UL

#### 5.29.1.6	OOB blocking exception requirements

There is no OOB exception for the CA combination.

### 5.29.2		Specific for 2 bands UL CA

#### 5.29.2.1	Maximum output power for inter-band CA

Table 5.29.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n7A-n102A | 23 | +2/-3 |
| CA_n7A-n102B | 23 | +2/-3 |
| CA_n7A-n102C | 23 | +2/-3 |

#### 5.29.2.2		UE co-existence

Table 5.29.2.2-1 gives IMD interference analysis for CA_ n7-n102 with uplink CA.

Table 5.29.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 2500 | 2570 | 5925 | 6425 |
| DL Frequency [MHz] | 2620 | 2690 | 5925 | 6425 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3355 | 3925 | 8425 | 8995 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1425 | 785 | 9280 | 10350 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10925 | 11565 | 14350 | 15420 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1075 | 1785 | 15205 | 16775 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 7850 | 6710 | 16850 | 17990 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 13425 | 14135 | 20275 | 21845 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 23200 | 21130 | 4355 | 3575 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 14275 | 12635 | 4140 | 5350 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 26200 | 28270 | 15925 | 16705 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 22775 | 24415 | 19350 | 20560 |

Based on the table 5.29.2.2-1, there are no IMD interference issues of this band combination

Table 5.29.2.2-3 gives an IMD interference analysis for CA_ n7-n102 with uplink CA for triple beat using the method of [2].

A triple beat study having n102C transmit in two 20MHz blocks with a 160MHz separation indicates IMD3.

Table 5.29.2.2-3: Triple beat IMD analysis of n102C

| CC location | fU1L | fU2L | fU3L | fU1H |  | Channel BW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 5925 | 5965 | 6085 | 6425 |  | 20 |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min channel separation |
| Frequency | 2500 | 2570 | 6385 | 6265 |  | 20 |
| 1st order TB | IfU3L -fU1L- fSCCL¦ | IfU2L -fU1L + fSCCL¦ | IfU2L -fU1L- fSCCH¦ | IfU3L -fU1L + fSCCH¦ |  | Max channel separation |
| Ranges | 2340 | 2540 | 2530 | 2730 |  | 160 |
| 1st order TB | IfU2L+fU1L-fSCCH¦ | IfU1H+fU2H-fSCCL¦ | IfU2L +fU1L+fSCCL¦ | IfU1H +fU2H+fSCCH¦ |  |  |
| Ranges | 9320 | 10310 | 14390 | 15380 |  |  |

The uplink of n7A and n102C has 1st order triple beat falling inside n7 DL.

Since this is including a share spectrum access band there is no additional protected bands as compared to n7.

#### 5.29.2.3		REFSENS requirements

As stated for figure 1 in [3] the MSD value is not eligible if the two UL inter-bands aren’t part of the same or adjacent band group.

********************    Removed sections unaffected         *******************************

## 7.3	DC_n7-n102

### 7.3.1	Configurations for DC_n7-n102

Table 7.3.1-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC Configuration |
| --- | --- |
| DC_n7A-n102A | DC_n7A-n102A |
| DC_n7A-n102B | DC_n7A-n102B |
| DC_n7A-n102C | DC_n7A-n102C |
| DC_n7A-n102D |  |
| DC_n7A-n102E |  |
| DC_n7A-n102(2A) | DC_n7A-n102A |

### 7.3.2	Maximum output power for NR-DC

Table 7.3.2-1: UE Power Class for uplink inter-band DC

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n7A-n102A | 23 | +2/-3 |
| DC_n7A-n102B | 23 | +2/-3 |
| DC_n7A-n102C | 23 | +2/-3 |

************************************* End of TP*****************************************

# 2	References

[1] R4-2303580 TP to TR 38.718-02-01 Addition of CA_n7-n102 and DC_n7-n102, RAN4#106

[2] R4-2304941 Discussion of triple beat rules for MSD analysis, RAN4#106-e

[3] R4-2220556 WF on triple beat rules and MSD for inter-band with 2UL with intra-band ULCA, RAN4#105