3GPP TSG RAN WG4 Meeting #109	R4-231xxxx

Chicago, USA, November 13th – 17th, 2023

Title:                LS on signalling for 2Tx-TxD and 4Tx-TxD

Response to:

Release:		Rel-18

Work Item:		NR_ENDC_RF_FR1_enh2-Core

Source:		RAN4

To:		RAN2

Cc:

Contact person:		Yuanyuan Zhang, tina55 (dot) zhang (at) samsung (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:		None

# 1	Overall description

In RAN4#108bis meeting, RAN4 sent a LS (R4-2317617) asking RAN2 to introduce a new per band per band combination (FS) capability for UE to indicate the support of TxDiversity (TxD) for 4Tx in the band configured. Meanwhile RAN4 sent a separate LS (R4-2317762) asking RAN2 to introduce a new FS UE capability to indicate the support of TxDiversity for 2Tx in the band configured.

After further discussion during RAN4#109, RAN4 would like to have following further clarification with RAN2.

1) New Rel-18 FS [txDiversity2Tx-r18] is applicable for both single band (non-CA) case and CA case.

2) New Rel-18 FS [txDiversity4Tx-r18] is applicable for both single band (non-CA) case and CA case.

3) In terms of the interaction between [txDiversity2Tx-r18] and [txDiversity4Tx-r18], RAN4 concludes that the presence of [txDiversity4Tx-r18] does not require the support of [txDiversity2Tx-r18].

4) RAN4 discuss the interaction between txDiversity-r16 and [txDiversity2Tx-r18], and the following understanding is reached for single band (Non-CA) case and RAN4 assume there is no NBC issue with the assumption that the advertised benefit for TxD reporting is to facilitate the conformance test.

- In case only txDiversity-r16 is indicated for a single band, it means UE can support TxD capability for this single band;

- In case only [txDiversity2Tx-r18] is indicated for a single band, it means UE can support TxD capability for this single band;

- It is not necessary for UE to indicate both txDiversity-r16 and [txDiversity2Tx-r18] for a single band, but if both are indicated, it means UE can support TxD capability for this single band

(below table is just here as a reference and would be removed in the final version)

|  | Rel-17 and backwards NW | Rel-18 and onwards NW |
| --- | --- | --- |
| Rel-17 and backwards UE | Legacy case (Network can only understand legacy txDiversity-r16) | Network can understand legacy txDiversity-r16 |
| Rel-18 and onwards UE | In case only txDiversity-r16 is indicated, it determines the TxD capability for the single band case; In case only [txDiversity2Tx-r18] is indicated by UE, NW cannot understand it; In case both txDiversity-r16 and [txDiversity2Tx-r18] are indicated for single band case, NW can only understand txDiversity-r16 and it determines the TxD capability for the single band | In case only txDiversity-r16 is indicated, it determines the TxD capability for the single band case; In case only [txDiversity2Tx-r18] is indicated by UE, it determines the TxD capability for the single band case; In case both txDiversity-r16 and [txDiversity2Tx-r18] are indicated for single band case, NW can understand both. |

And the following understanding for CA case is also reached for Rel-18 and onwards:

- In terms of 2Tx TxD capability, it is agreed that CA does not inherit the corresponding single band TxD capability, which means for example if [txDiversity2Tx-r18] is absent for band A within CA_A-B, it means A within A-B does not support 2Tx TxD even TxD is supported for single band A.

A clarification for Rel-17 and backwards is that there is no requirements specified in RAN4 for inter-band ULCA+ TxD case which requires 3Tx Chain transmitting simultaneously.

5) RAN4 discuss the interaction between txDiversity-r16 and [txDiversity4Tx-r18], the logic is same as the interaction between txDiversity-r16 and [txDiversity4Tx-r18].

# 2	Actions

To RAN2:

ACTION: 	RAN4 respectfully ask RAN2 to take above clarification into account.

# 3	Dates of next TSG-RAN WG4 meetings

TSG-RAN WG4 Meeting #110	26 Feb – 01 Mar, 2024		        Athens, Greece

TSG-RAN WG4 Meeting #110bis	15 – 19 Apr, 2024		             China