3GPP TSG-RAN WG4 Meeting #110                                                                                            R4-2400042

Athens, GR, February 26 – March 01, 2024

Source: 	CATT

Title: 	Discussion on SAN RF conformance testing requirements for above 10GHz bands

Agenda item:	8.18.4

Document for:	Approval

- Introduction

This contribution provides our views on SAN RF conformance testing requirements for above 10GHz bands.

- Discussion

Spatial characteristics of requirements

According to Table 6.1.1.1-1 of TR 38.821, the 3dB beam-width of Ka-band SAN antenna for GEO, LEO-1200 and LEO-600 are 0.1765, 1.7647 and 1.7647 degree, respectively.  The 0.1765 degree 3dB beam-width of Ka-band SAN antenna for GEO is narrower, for such narrower 3dB beam-width, is it possible to test the 5 conformance directions? It is dependent on the turn table angle accuracy of OTA measurement systems.

As shown in following tables 2-1 and 2-2, for some requirements (radiated transmit power, OTA modulation quality, OTA reference sensitivity level), whether 5 conformance directions is applicable for SAN type 2-O with narrower beam-width need further discussion.

Observation 1:  Whether 5 conformance directions is applicable for SAN type 2-O with narrower beam-width need further discussion.

Table 2-1: Overview of radiated Tx requirements

| Tx requirement |  | Classification | Coverage range |  | Number of |
| --- | --- | --- | --- | --- | --- |
|  |  |  | FR1-NTN | FR2-NTN | conformance directions |
| Radiated transmit power |  | Directional | OTA peak directions set |  | [5] |
| OTA Satellite Access Node output power |  | TRP | N/A |  | See TS 38.141-2 [18], annex I |
| OTA output power dynamics |  | Directional | OTA peak directions set |  | 1 |
| OTA modulation quality |  | Directional | OTA coverage range |  | [5] |
| OTA frequency error |  | Directional | OTA coverage range |  | 1 |
| OTA occupied bandwidth |  | Directional | OTA coverage range |  | 1 |
| OTA ACLR |  | TRP | N/A |  | See TS 38.141-2 [18], annex I |
| OTA out-of-band emissions |  | TRP | N/A |  | See TS 38.141-2 [18], annex I |
| OTA transmitter spurious emission | General requirement | TRP | N/A |  | See TS 38.141-2 [18], annex I |
| NOTE: Directional requirement does not imply one compliance direction only. The directional requirement applies to a single direction at a time. |  |  |  |  |  |

Table 2-2: Overview of radiated Rx requirements

| Rx requirement |  | Classification | Applicability levels |  | Coverage range |  | Number of |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FR1-NTN | FR2-NTN | FR1-NTN | FR2-NTN | conformance directions |
| OTA sensitivity |  | Directional | Minimum EIS | N/A | OSDD | N/A | 5 |
| OTA reference sensitivity level |  | Directional | OTA REFSENS | OTA REFSENS | OTA REFSENS RoAoA | OTA REFSENS RoAoA | [5] |
| OTA Dynamic range |  | Directional | OTA REFSENS | N/A | OTA REFSENS RoAoA | N/A | 1 |
| OTA adjacent channel selectivity |  | Directional | minSENS | OTA REFSENS | minSENS RoAoA | OTA REFSENS RoAoA | 1 |
| OTA out-of-band blocking | General requirement | Directional | minSENS | [OTA REFSENS] | minSENS RoAoA | [OTA REFSENS RoAoA] | 1 |
| OTA in-channel selectivity |  | Directional | minSENS | OTA REFSENS | minSENS RoAoA | OTA REFSENS RoAoA | 1 |
| NOTE: Directional requirement does not imply one compliance direction only. The directional requirement applies to a single direction at a time. |  |  |  |  |  |  |  |

Measurement uncertainties

Table 2-1: NTN operating band for Ka-band NTN [1]

| NTN operating band | UL Earth-to-Space | DL Space-to-Earth |
| --- | --- | --- |
| n5121 | 27.5 - 30.0 GHz | 17.3 - 20.2 GHz |
| n5112 | 28.35 - 30.0 GHz | 17.3 - 20.2 GHz |
| n5103 | 27.5 - 28.35 GHz | 17.3 - 20.2 GHz |

According to the agreed NTN operating band for Ka-band NTN as shown in above Table 2-1, the DL frequency ranges for the three bands are within 17.3-20.2 GHz, and the UL frequency ranges of the three bands are within 27.5-30GHz. In TS 38.141-1, there are measurement uncertainties for 24.25 – 29.5 GHz for FR2 OTA receiver tests. Since most of the range of 27.5-30GHz overlaps with the range of 24.25 – 29.5 GHz, we think that MU for 24.25 – 29.5 GHz for FR2 TN BS OTA receiver tests can be reused for MU for 27.5-30GHz for SAN type 2-O OTA receiver tests. For MU for 17.3-20.2 GHz for FR2 SAN type 2-O OTA transmitter tests, TE vendors’ inputs are needed.

Proposal 1: MU for 24.25 – 29.5 GHz for FR2 TN BS OTA receiver tests can be considered to be reused for MU for 27.5-30GHz for SAN type 2-O OTA receiver tests.

Proposal 2: For MU for 17.3-20.2 GHz for FR2 SAN type 2-O OTA transmitter tests, TE vendors’ inputs are needed.

Manufacturer declarations

Since it is agreed that there is no OTA sensitivity requirement for SAN type 2-O [2], so manufacturer declarations related to OTA sensitivity (OSDD-related) is not needed. Therefore, manufacturer declarations for SAN type 1-O except OSDD-related manufacturer declaration can be reused for SAN type 2-O. In addition, EISREFSENS_50M need to be added.

Proposal 3: Manufacturer declarations for SAN type 1-O except OSDD-related manufacturer declaration can be reused for SAN type 2-O. In addition, EISREFSENS_50M need to be added.

Test Signal

Since the SCS and CBW for FR2-1 TN BS were agreed to be reused for Ka-band SAN[3], the test signal for BS type 2-O with BS channel bandwidth replaced by SAN channel bandwidth can be reused for SAN type 2-O. So test signal for SAN type 2-O can be defined as shown in Table 2-2.

Table 2-2: Signal to be used to build NR TCs for SAN type 2-O

| Operating band characteristics |  | FDL_high – FDL_low ≤ 3250 MHz |
| --- | --- | --- |
| TC signal | BWchannel | 100 MHz (Note 1, Note 2) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing declared per operating band (D.7) |
| NOTE 1: SAN vendor can decide to test with 50 MHz SAN channel bandwidth and smallest supported SCS declared per operating band (D.7) instead of 100 MHz SAN channel bandwidth in certain regions, where spectrum allocation and regulation require testing with 50 MHz. NOTE 2: If this SAN channel bandwidth is not supported, the narrowest supported SAN channel bandwidth declared per operating band (D.7) shall be used. |  |  |

Proposal 4: Agree the following test signal in Table 2-2 for SAN type 2-O,

Table 2-2: Signal to be used to build NR TCs for SAN type 2-O

| Operating band characteristics |  | FDL_high – FDL_low ≤ 3250 MHz |
| --- | --- | --- |
| TC signal | BWchannel | 100 MHz (Note 1, Note 2) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing declared per operating band (D.7) |
| NOTE 1: SAN vendor can decide to test with 50 MHz SAN channel bandwidth and smallest supported SCS declared per operating band (D.7) instead of 100 MHz SAN channel bandwidth in certain regions, where spectrum allocation and regulation require testing with 50 MHz. NOTE 2: If this SAN channel bandwidth is not supported, the narrowest supported SAN channel bandwidth declared per operating band (D.7) shall be used. |  |  |

Test Model

From perspective of test model, since RAN1 parameters of Ka-band NTN and FR2-1 TN are basically the same except 64QAM is FFS for NTN[3,2], so we think the NR-FR2-TM1.1, NR-FR2-TM2 and NR-FR2-TM3.1 for FR2 in TS 38.141-2 are applicable for Ka-band NTN. As the transmit ON/OFF power, TAE and receiver spurious emissions are not applicable for Ka-band NTN, so NR-FR2-TM1.1 is not applicable for transmit ON/OFF power, TAE and Receiver spurious emissions requirements.

Table 2-3: Test model for Ka-band NTN

| Test models for FR2 in TS38.141-2 | Applicability for NTN SAN | Test model for FR2-NTN |
| --- | --- | --- |
| NR-FR2-TM1.1 | Applicable, remove the wording of applying to transmit ON/OFF power, TAE and Receiver spurious emissions compared with TN test specification; | NR-SAN-FR2-TM1.1 |
| NR-FR2-TM2 | Applicable. | NR-SAN-FR2-TM2 |
| NR-FR2-TM3.1 | Applicable | NR-SAN-FR2-TM3.1 |

Proposal 5: Agree the test model in following table for Ka-band NTN

Table 2-3: Test model for Ka-band NTN

| Test models for FR2 in TS38.141-2 | Applicability for NTN SAN | Test model for FR2-NTN |
| --- | --- | --- |
| NR-FR2-TM1.1 | Applicable, to remove the wording of applying to transmit ON/OFF power, TAE and Receiver spurious emissions compared with TN test specification; | NR-SAN-FR2-TM1.1 |
| NR-FR2-TM2 | Applicable | NR-SAN-FR2-TM2 |
| NR-FR2-TM3.1 | Applicable | NR-SAN-FR2-TM3.1 |

Test Configuration

It is agreed that multi-band and non-contiguous operation are not supported in Rel-18[4], only single carrier and multi-carrier are supported for SAN type 2-O. So test configuration NRTC1 is sufficient for SAN type 2-O, no other TC is needed. Existing test configuration NRTC1 in TS 38.181 is applicable for SAN type 2-O.

Proposal 6: Existing test configuration NRTC1 in TS 38.181 is applicable for SAN type 2-O.

RF Channels

Since only single carrier and multi-carrier are supported for SAN type 2-O[4], RF channels for single carrier and multi-carrier definition in TS 38.181 are sufficient for SAN type 2-O, other RF channels are not needed. RF channels for single carrier and multi- carrier operation for transmitter requirements for SAN type 1-O can be reused for SAN type 2-O. So RF channels for SAN type 2-O can be defined as shown in Table 2-4.

Table 2-4: RF channels for SAN type 2-O

| Requirements |  | single carrier | Multi-carrier |
| --- | --- | --- | --- |
| 9.2 | Radiated transmit power | B,M,T | BRFBW, MRFBW, TRFBW |
| 9.3 | OTA SAN output power | B,M,T | BRFBW, MRFBW, TRFBW |
| 9.4 | OTA total power dynamic range | M | - |
| 9.6.2 | OTA Frequency Error | Same as EVM | Same as EVM |
| 9.6.3 | OTA Modulation quality | B, T | BRFBW, TRFBW |
| 9.7.2 | OTA Occupied BW | M | - |
| 9.7.3 | OTA ACLR | B, T | BRFBW, TRFBW |
| 9.7.4 | OTA Out-of-band emissions | B,M,T | BRFBW, MRFBW, TRFBW |
| 9.7.5 | OTA Transmitter spurious emissions | B for spurious frequencies below the band, T for frequencies above the band | BRFBW for spurious frequencies below the band, TRFBW for frequencies above the band |

Proposal 7: RF channels for single carrier and multi-carrier operation for transmitter requirements for SAN type 1-O can be reused for SAN type 2-O.

- Conclusion

This contribution provides our general consideration on SAN RF conformance testing requirement for above 10GHz bands. The following observations and proposals are provided as follows:

Observation 1:  Whether 5 conformance directions is applicable for SAN type 2-O for some narrower beam-width need further discussion.

Proposal 1: MU for 24.25 – 29.5 GHz for FR2 TN BS OTA receiver tests can be considered to be reused for MU for 27.5-30GHz for SAN type 2-O OTA receiver tests.

Proposal 2: For MU for 17.3-20.2 GHz for FR2 SAN type 2-O OTA transmitter tests, TE vendors’ inputs are needed.

Proposal 3: Manufacturer declarations for SAN type 1-O except OSDD-related manufacturer declaration can be reused for SAN type 2-O. In addition, EISREFSENS_50M need to be added.

Proposal 4: Agree the following test signal in Table 2-2 for SAN type 2-O,

Table 2-2: Signal to be used to build NR TCs for SAN type 2-O

| Operating band characteristics |  | FDL_high – FDL_low ≤ 3250 MHz |
| --- | --- | --- |
| TC signal | BWchannel | 100 MHz (Note 1, Note 2) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing declared per operating band (D.7) |
| NOTE 1: SAN vendor can decide to test with 50 MHz SAN channel bandwidth and smallest supported SCS declared per operating band (D.7) instead of 100 MHz SAN channel bandwidth in certain regions, where spectrum allocation and regulation require testing with 50 MHz. NOTE 2: If this SAN channel bandwidth is not supported, the narrowest supported SAN channel bandwidth declared per operating band (D.7) shall be used. |  |  |

Proposal 5: Agree the test model in following table for Ka-band NTN

Table 2-3: Test model for Ka-band NTN

| Test models for FR2 in TS38.141-2 | Applicability for NTN SAN | Test model for FR2-NTN |
| --- | --- | --- |
| NR-FR2-TM1.1 | Applicable, to remove the wording of applying to transmit ON/OFF power, TAE and Receiver spurious emissions compared with TN test specification; | NR-SAN-FR2-TM1.1 |
| NR-FR2-TM2 | Applicable. | NR-SAN-FR2-TM2 |
| NR-FR2-TM3.1 | Applicable | NR-SAN-FR2-TM3.1 |

Proposal 6: Existing test configuration NRTC1 in TS 38.181 is applicable for SAN type 2-O.

Proposal 7: RF channels for single carrier and multi-carrier operation for transmitter requirements for SAN type 1-O can be reused for SAN type 2-O.

- Reference

- R4-2302998, WF for system parameters of above 10 GHz NTN band, Moderator (THALES), RAN4#106

- R4-2302877, WF for SAN RF requirements on above 10 GHz, Moderator (THALES), RAN4#106

- R4-2220239, WF for above 10GHz band definition and system parameters, Moderator (THALES), RAN4#105

- R4-2305927, WF on SAN RF requirements – NTN Enhancements, Ericsson, RAN4#106bis-e