3GPP TSG-RAN WG4 Meeting #110	R4-2400082

# Athens, GR, Feburary 26 – March 1

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 3956 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_pos_enh2-Core) CR on correction of measurement period requirements for RedCap UE with FH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - PRS-based positioning measurement requirements with FH in RRC_IDLE/RRC_INACTIVE/RRC_CONNECTED states in TS 38.133 should be corrcted. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the measurement period requiremnts with FH in RRC_IDLE/ RRC_INACTIVE/RRC_CONNECTED states and update the corresponding signallings. - Correct some clause citation errors and wording errors in positioning clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - PRS-based positioning measurements with FH in RRC_IDLE / RRC_INACTIVE / RRC_CONNECTED states are not completed. - Some citations and wordings are not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 4.6.2.6, 4.6.3.5, 4.6.4.5, 5.6A.4.6, 5.6A.5.6, 5.6A.6.6, 9.9A.2.6, 9.9A.3.6, 9.9A.4.8, 9.9A.5.5, 9.9A.5.6, 9.9A.5.7, and 9.9A.5.8. New sections: 4.6.3.6, 4.6.4.6. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

4.6.2.6	Measurement Period Requirements with RX FH

After receiving both NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message from the LMF via LPP [34], requesting RedCap UE to measure DL RSTD measurement with FH, the RedCap UE shall be able to measure multiple (up to the RedCap UE capability specified in Clause 4.6.2.3) DL RSTD measurements with FH, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined in Clause 4.6.2.5 with using the following definition for Nsample and Lavailable_PRS,i:

Nsample is the number of PRS RSTD measurement samples, where

-	Nsample= 2 if the RedCap UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

-	Nsample= 4 otherwise.

Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured, and is calculated by:

Lavailable_PRS,i=Nhop*Lper_hop

Lper_hop is the time duration of available PRS per hop, and Nhop is the number of hops which is derived with the following equation:

Nhop=min6, BWPRS-NRB_overlapNRB_hop-NRB_overlap

Where BWPRS is the configured BW for PRS resources, NRB_hop is the number of RBs per hop and NRB_overlap is the number of overlapping RBs between adjacent hops.

<Unchanged contents are omitted>

4.6.3.5	Measurement Period Requirements without FH

<<<Unchanged contents are omitted>>>

If the UE’s RRC state changes from the RRC_IDLE to RRC_CONNECTED during the PRS-RSRP measurement period, then the UE shall continue the PRS-RSRP measurement in the RRC_CONNECTED state. The PRS-RSRP measurement period can be longer.

4.6.3.6	Measurement Period Requirements with FH

When the physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], requesting RedCap UE to measure DL PRS-RSRP measurement with FH, the RedCap UE shall be able to measure multiple (up to the RedCap UE capability specified in Clause 4.6.3.3) PRS-RSRP measurements with FH, defined in TS 38.215 [4], from configured PRS resources for configured TRPs on configured positioning frequency layers, within the measurement period  TPRS-RSRP,total defined in Clause 4.6.3.5 with using the following definition for Nsample and Lavailable_PRS,i:

Nsample is the number of PRS RSTD measurement samples, where

-	Nsample= 2 if the RedCap UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

-	Nsample= 4 otherwise.

Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured, and is calculated by:

Lavailable_PRS,i=Nhop*Lper_hop

Lper_hop is the time duration of available PRS per hop, and Nhop is the number of hops which is derived with the following equation:

Nhop=min6, BWPRS-NRB_overlapNRB_hop-NRB_overlap

Where BWPRS is the configured BW for PRS resources, NRB_hop is the number of RBs per hop and NRB_overlap is the number of overlapping RBs between adjacent hops.

<Unchanged contents are omitted>

4.6.4.5	Measurement Period Requirements without FH

When the physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message in RRC_IDLE state, measurement period requirements for PRS-RSRP defined in 4.6.3.5 is re-used for PRS-RSRPP measurement.

4.6.4.6	Measurement Period Requirements with FH

When the physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message in RRC_IDLE state, requesting RedCap UE to measure DL PRS-RSRP measurement with FH, measurement period requirements for PRS-RSRP defined in 4.6.3.6 is re-used for PRS-RSRPP measurement with FH.

<End of Change 1>

<Start of Change 2>

5.6A.4.6	Measurement period requirement with FH

After receiving both NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation  message from the LMF via LPP [34] requesting the RedCap UE to measure and report DL RSTD measurements defined in TS 38.215 [4] with FH via nr-DL-PRS-RxHopping-Request, the RedCap UE shall be able to measure multiple (up to the UE capability specified in Clause 5.6A.4.3) DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined in Clause 5.6A.4.5 with using the following definition for Nsample and Lavailable_PRS,i:

Nsample is the number of PRS RSTD measurement samples, where

-	Nsample= 2 if the RedCap UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

-	Nsample= 4 otherwise.

Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured, and is calculated by:

Lavailable_PRS,i=Nhop*Lper_hop

Lper_hop is the time duration of available PRS per hop, and Nhop is the number of hops which is derived with the following equation:

Nhop=min6, BWPRS-NRB_overlapNRB_hop-NRB_overlap

Where BWPRS is the configured BW for PRS resources, NRB_hop is the number of RBs per hop and NRB_overlap is the number of overlapping RBs between adjacent hops.

TRSTD_FH,Total=i=1LTRSTD_FH,i+ L-1×maxTeffect,i

i

L

Teffect,i

TRSTD_FH,i

TRSTD_FH,i= Kcarrier_PRS× NRx,TEG,i×NRxBeam,i×NPRS,islotN'×Lavailable_PRS,iN×Nsample-1×Teffect,i+Tlast,i

<Unchanged contents are omitted>

5.6A.5.6	Measurement period requirement with FH

After receiving both NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation  message from the LMF via LPP [34] requesting the RedCap UE to measure and report DL RSRP measurements defined in TS 38.215 [4] with FH via nr-DL-PRS-RxHopping-Request, the RedCap UE shall be able to measure multiple (up to the UE capability specified in Clause 5.6A.4.3) DL RSRP measurements, defined in TS 38.215 [4], during the measurement period TRSRP,Total defined in Cluase 5.6A.5.5 with using the following definition for Nsample and Lavailable_PRS,i:

Nsample is the number of PRS RSTD measurement samples, where

-	Nsample= 2 if the RedCap UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

-	Nsample= 4 otherwise.

Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured, and is calculated by:

Lavailable_PRS,i=Nhop*Lper_hop

Lper_hop is the time duration of available PRS per hop, and Nhop is the number of hops which is derived with the following equation:

Nhop=min6, BWPRS-NRB_overlapNRB_hop-NRB_overlap

Where BWPRS is the configured BW for PRS resources, NRB_hop is the number of RBs per hop and NRB_overlap is the number of overlapping RBs between adjacent hops.

TPRS-RSRP_FH,Total=i=1LTPRS-RSRP_FH,i+ L-1×maxTeffect,i

i

L

Teffect,i

TRSRP_FH,i

TRSRP_FH,i= Kcarrier_PRS_RedCap× NRx,TEG,i×NRxBeam,i×NPRS,islotN'×Lavailable_PRS,iN×Nsample-1×Teffect,i+Tlast,i

<Unchanged contents are omitted>

5.6A.6.6	Measurement Period Requirements with FH

When physical layer receives last of NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [34], requesting the RedCap UE to measure and report UE Rx-Tx time difference measurements with FH, the RedCap UE shall be able to measure multiple (up to the RedCap UE capability specified in clause 5.6A.6.3) UE Rx-Tx time difference measurements as defined in TS 38.215 [4] in configured positioning frequency layers within the measurement period TUERxTx,Total ms defined in 5.6.4.5 with the following definition for Nsample and Lavailable_PRS,i:

Nsample is the number of PRS RSTD measurement samples, where

-	Nsample= 2 if the RedCap UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

-	Nsample= 4 otherwise.

Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured, and is calculated by:

Lavailable_PRS,i=Nhop*Lper_hop

Lper_hop is the time duration of available PRS per hop, and Nhop is the number of hops which is derived with the following equation:

Nhop=min6, BWPRS-NRB_overlapNRB_hop-NRB_overlap

Where BWPRS is the configured BW for PRS resources, NRB_hop is the number of RBs per hop and NRB_overlap is the number of overlapping RBs between adjacent hops.

<End of Change 2>

<Start of Change 3>

#### 9.9A.2.6	Measurements Period Requirements with FH

##### 9.9A.2.6.1	Measurements Period Requirements with FH with MG

When physical layer receives last of NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message from LMF via LPP [34], requesting RedCap UE to measure DL RSTD measurement with FH, the RedCap UE shall be able to measure multiple (up to the RedCap UE capability specified in Clause 9.9A.2.3) DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined in Clause 9.9A.2.5.1 with using the following definition for Nsample and Lavailable_PRS,i:

Nsample is the number of PRS RSTD measurement samples, where

-	Nsample= 2 if the RedCap UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

-	Nsample= 4 otherwise.

Measurement sample under FH is defined as a PRS measurement over multiple hops within a single measurement gap occasion.

Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured during a MG, and is calculated by:

Lavailable_PRS,i=Nhop*Lper_hop

Lper_hop is the time duration of available PRS per hop, and Nhop is the number of hops within a single MG occasion which is derived with the following steps:

-	K' is a RF switching time related parameter. K'=1, if the RF switching time is ≤ 7 OFDM symbols. Otherwise, K'=2.

-	If K'=1 and Nhopslot is equal to or larger than 1, the number of hops within a single MG occasion can be derived by the equation:

Nhop=Nhopslot*minN,Nhop,max

where N is the number of PRS repetitions within a single MG occasion, Nhop,max is the maximum number of hops derived from the configured PRS bandwidth, and Nhopslot is the number of hops that UEs can perform in a slot which is given by the equation:

Nhopslot=min14NCombSize+NRFswiching,NPRS_OFDMslotNCombSize+NRFswiching

where NPRS_OFDMslot is the number of OFDM symbols for PRS resources in one slot, NCombSize is the PRS comb size, and NRFswiching is the number of OFDM symbols for RF switching time.

-	If K'=2 or Nhopslot=0, the number of hops within a single MG occasion can be derived by the equation with K'=2:

Nhop=minN-1*MK+1,Nhop,max

where M is the PRS repetition interval (given by dl-PRS-ResourceTimeGap) and K is the number of slots per hop and K=maxK',M.

<Unchanged contents are omitted>

9.9A.3.6	Measurements Period Requirements with FH

9.9A.3.6.1	Measurements Period Requirements with FH with MG

When the physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], requesting RedCap UE to measure PRS-RSRP measurement with FH, the RedCap UE shall be able to measure multiple (up to the RedCapUE capability specified in Clause 9.9A.3.3) PRS-RSRP measurements, defined in TS 38.215 [4], from configured PRS resources for configured TRPs on configured positioning frequency layers, within TPRS-RSRP,total ms as defined Clause 9.9A.3.5.1 with using the following definition for Nsample and Lavailable_PRS,i:

Nsample is the number of PRS RSRP measurement samples, where

-	Nsample= 2 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

-	Nsample= 4 otherwise.

Measurement sample under FH is defined as a PRS measurement over multiple hops within a single measurement gap occasion.

Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured during a MG, and is calculated by:

Lavailable_PRS,i=Nhop*Lper_hop

Lper_hop is the time duration of available PRS per hop, and Nhop is the number of hops within a single MG occasion which is derived with the following steps:

-	K' is a RF switching time related parameter. K'=1, if the RF switching time is ≤ 7 OFDM symbols. Otherwise, K'=2.

-	If K'=1 and Nhopslot is equal to or larger than 1, the number of hops within a single MG occasion can be derived by the equation:

Nhop=Nhopslot*minN,Nhop,max

where N is the number of PRS repetitions within a single MG occasion, Nhop,max is the maximum number of hops derived from the configured PRS bandwidth, and Nhopslot is the number of hops that UEs can perform in a slot which is given by the equation:

Nhopslot=min14NCombSize+NRFswiching,NPRS_OFDMslotNCombSize+NRFswiching

where NPRS_OFDMslot is the number of OFDM symbols for PRS resources in one slot, NCombSize is the PRS comb size, and NRFswiching is the number of OFDM symbols for RF switching time.

-	If K'=2 or Nhopslot=0, the number of hops within a single MG occasion can be derived by the equation with K'=2:

Nhop=minN-1*MK+1,Nhop,max

where M is the PRS repetition interval (given by dl-PRS-ResourceTimeGap) and K is the number of slots per hop and K=maxK',M.

<Unchanged contents are omitted>

9.9A.4.8	Measurements Period Requirements with FH

The requirements in this clause apply when a RedCap UE is request by the LMF to perform measurements with FH, and UE reports measurement based on multiple hops.

The requirements in clause 9.9A.4.5 shall apply with the following modifications.

-	Nsample= 2 if the UE supports the capability of positioning measurements with reduced number of samples as indicated by supportedDL-PRS-ProcessingSamples-RRC-CONNECTED specified in TS 37.355 [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

-	Measurement sample under FH is defined as a PRS measurement over multiple hops within a single measurement gap.

-	Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured during a MG, and is calculated by:

Lavailable_PRS,i=Nhop*Lper_hop

Lper_hop is the time duration of available PRS per hop, and the number of hops within a single MG occasion is derived with the following steps:

-	K' is a RF switching time related parameter. K'=1, if the RF switching time is ≤ 7 OFDM symbols. Otherwise, K'=2.

-	If K'=1 and Nhopslot is equal to or larger than 1, the number of hops within a single MG occasion can be derived by the equation:

Nhop=Nhopslot*minN,Nhop,max

where N is the number of PRS repetitions within a single MG occasion, Nhop,max is the maximum number of hops derived from the configured PRS bandwidth, and Nhopslot is the number of hops that UEs can perform in a slot which is given by the equation:

Nhopslot=min14NCombSize+NRFswiching,NPRS_OFDMslotNCombSize+NRFswiching

where NPRS_OFDMslot is the number of OFDM symbols for PRS resources in one slot, NCombSize is the PRS comb size, and NRFswiching is the number of OFDM symbols for RF switching time.

-	If K'=2 or Nhopslot=0, the number of hops within a single MG occasion can be derived by the equation with K'=2:

Nhop=minN-1*MK+1,Nhop,max

where M is the PRS repetition interval (given by dl-PRS-ResourceTimeGap) and K is the number of slots per hop and K=maxK',M.

<Unchanged contents are omitted>

9.9A.5.5	Measurement Period Requirements without FH with MG

For PRS measurement within MG configured to UE, measurement period requirements for PRS-RSRP defined in 9.9A.3.5.1 is re-used for PRS-RSRPP.

9.9A.5.6	Measurement Period Requirements without FH without MG

For PRS measurement without MG configured to UE, measurement period requirements for PRS-RSRP defined in 9.9A.3.5.2 is re-used for PRS-RSRPP.

The PRS-RSRPP measurement requirements in this section apply for the first path PRS-RSRP measurement.

9.9A.5.7	Measurements Period Requirements without FH with both MG and PPW

If the UE is configured with both MG applicable to positioning measurement and PPW, the measurement period requirements for PRS-RSRP defined in 9.9A.3.5.3 is re-used for PRS-RSRPP.

9.9A.5.8	Measurements Period Requirements with FH

The measurement period requirements for PRS-RSRP defined in 9.9A.3.6 are re-used for PRS-RSRPP.

<End of Change 3>